# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-05

SAFE WORK PROCEDURE

CHAIN OR SLING LIFTING PROCEDURE

### 2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear (steel cap boots) must be worn at all times in work areas. | Close fitting/protective, Hi vis, long sleeve and long pant clothing must be worn. |
| Hard hat required. | Wear protective Gloves. |
| Rings and jewellery must not be worn. |  |

Risk Factors to be considered

- Unauthorised or untrained persons entering the work site.

- Proximity of people to mobile plant.

- Plant being operated in reverse.

- Working at night.

- Vehicles being loaded and unloaded.

- Noisy work locations masking the sound of approaching plant.

- Keep hands clear of lifting points.

- Climbing in and out of the job truck tray (always maintain 3 points of contact, use steps and ladders where provided).

### Pre-operational Safety Checks

- Check the SWL and the lifting lugs of the machine being used.

- Check the Weight of the equipment being lifted.

- Check slings for broken wires, damaged splice, bird-caging, kinks and SWL is clearly marked.

- Check chains for cracks, there is not over 10% wear, stretched links or hooks, spotwelded links, no knots in chain and that the SWL tag is attached and clearly marked.

- Check that the correct chain or sling is being used by checking the SWL tag.

- DO NOT use a chain or sling if the tag is missing.

- Check the lifting lugs on the equipment being lifted and that the SWL is clearly marked.

- Check for overhead wires in the area where you are lifting and placing the equipment.

- Ensure area and path of lifting machine is clear.

SWP- Chain or Sling Lifting Procedure	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-05

### Operational Safety

- If there is any sign of instability lower the load immediately.

- When the load is raised keep all people at a safe distance.

- Do not swing loads over other people’s heads.

- Do not leave the load suspended in the air. Lower it as soon as possible.

- Always lower the load to the ground if leaving the machine.

- If you don’t know the weight of the load DON’T lift it.

- Do not reeve or nip slings around buckets as the sling can be cut by cutting edge or teeth of bucket.

- Check that the lifting hook is attached securely before lifting.

- When traveling keep the load as close to the ground as possible.

- Check the lifting lug on the machine that it is within the limit of the item being lifted.

- Use a tag line to control the movement of the load.

- Make sure tag line is at least 16mm in diameter.

- Make sure you are clear of the load before it is lifted.

- Ensure you have clear contact with the loader/Skid steer Bobcat operator at all times.

- Ensure the site where the equipment is being put is suitable to take the load.

### You should NOT LIFT PEOPLE in the bucket of a front-end loader/Skid steer Bobcat as it has not been designed for this purpose. To lift anyone with the bucket is AGAINST ALL SAFETY OPERATING PROCEDURES.

Site Induction

- Site induction is a critical means of communicating site specific safety rules including lifting of equipment to all workers and visitors to asphalting/spray sealing sites.

- Site inductions should be immediately repeated as site conditions and safety measures change.

- Toolbox meetings provide an alternate and/or supplementary means of ensuring that all workers and visitors on site are kept up to date with changes.

SWP- Chain or Sling Lifting Procedure	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.