# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-06

SAFE WORK PROCEDURE

COLD SAW (METAL)

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

SWP-Coldsaw (Metal)	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

Pre-Operational Safety Checks

PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-06

- Check workspace and walkways to ensure that no slip/trip hazards are present.

- Ensure saw blade is in good condition.

- Locate and check the operation of the ON/OFF starter.

- Check that all safety guards are in working order and fitted.

- Check the operation of the work vice.

- Check coolant delivery system to allow for sufficient flow of coolant.

- Faulty equipment must not be used. Immediately report suspect equipment.

### Operational Safety Checks

- Ensure that the work piece is securely held in the work vice.

- Support overhanging work and signpost if it presents a hazard.

- Never leave the machine running unattended.

- Attention must be paid to unusual noises during the sawing process.

- Never force the saw into the workpiece. Use a slow and even feed rate.

- Before making adjustments or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

- Immediately absorb any coolant spills.

### Housekeeping

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

### Potential Hazards

- Hair/clothing entanglement

- Eye injuries

- Flying swarf/chips

- Sharp edges & burrs, metal splinters.

- Ejection of work pieces.

- Shattering of the saw blade.

### A Cold Saw MUST NOT BE USED FOR:

- Attempting to cut very small items

- Attempting to cut materials other than metal.

SWP-Coldsaw (Metal)	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.