PANELTEC PTY LTD

SWP-07

SAFE WORK PROCEDURE

COMPRESSED AIR

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Wear protective Gloves. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. |  |

Only one person may operate this machine at any one time.

- Before starting the compressor, be sure the manual and all warning signs have been completely read.

- Pipes should be properly labeled that carry compressed air and the direction of air flow correctly labeled with an arrow. Shutoff valves should be properly labeled and identified so air can be shut off quickly in an emergency situation.

- Hoses, fittings, regulators, and valves should be inspected periodically for leaks, damage, and other defects.

- Goggles must be worn over safety glasses when cleaning with compressed air.

- Flexible air hoses should be kept as short as possible to minimize tripping hazards and to reduce whipping action in the event a hose would fail.

- High pressure jacketed lines should be anchored at several points to prevent them from whipping.

- Quick disconnect fittings should be installed on flexible air hoses in high fire hazard areas; the hoses can be disconnected quickly, preventing whipping actions that might not only cause injury and damage but also stoke a fire.

- Use a vacuum system rather than compressed air for cleaning whenever possible. Vacuuming stirs up less dust and other (combustible) particles than an air compressor does.

- DO NOT use compressed air to:

  - Transfer flammable liquids. Static electricity build-up can discharge and ignite the liquid.

  - Empty containers. The container could rupture due to excessive internal pressure.

  - Clean clothes, hair, or skin.

- When using compressed air, direct air away from eyes and skin.

- To reduce noise exposure and prevent exhaust from the equipment or tool, direct the pressure relief valve away from work areas.

SWP- Compressed Air V10 - 31.08.2025	Authorised & Approved by Management	Page 1 of 1

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.