# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-20

SAFE WORK PROCEDURE

WORKSHOP SAFETY

DO NOT use ANY TOOL OR piece of EQUIPMENT unless you have been instructed in its safe use and operation.

IF YOU ARE NOT SURE – ASK

Refer to Tool/Activity Specific Safe Work Procedures

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas as defined by signage or as per SWMS or SWP. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in workshop areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used where signed or when using noisy equipment such as drills. | Wear protective Gloves as per signage or as per SWMS or SWP. |
| Rings and jewellery must not be worn. |  |

| Issue/Activity - Hazard | Control Measures |
| --- | --- |
| Preventing Vehicles or mobile plant from Moving. | - Vehicles are moved to a safe area that is remote from site traffic and plant before work commences. - Keys are removed from the ignition. - The hand brake is on. - Wheels are chocked on both sides of the vehicle using serviceable purpose-built chocks. Or - Vehicle raised on a pillar hoist with all wheels off the ground. Prior to lowering vehicle/plant make sure you inform all persons in the vicinity of the vehicle being lowered and have them, stand well clear. |
| Preventing slips and trips | - All leads are stored out of the way, eg retractable reels. - Power and pneumatic outlets located close to the service area. - Spills, including water spills, are covered with absorbent material and immediately cleaned up. - A non-slip surface coating is applied to potential slippery surfaces, eg steps. - Projecting equipment, such as bench vices, must be painted yellow (ideal safety colour) to alert workspace users to pass cautiously. - Do not allow work areas to become cluttered. |
| Guarding | - Ensure that machine safety guards are always in place and well maintained. |

SWP- Workshop Safety   Ver 10 – 31.08.2025	Authorised & Approved by Management	Page 1 of 4

#### This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-20

| Issue/Activity - Hazard | Control Measures |
| --- | --- |
| Safe Operating Instructions | - Fit safety operating instructions prominently on all relevant machines and equipment. |
| Safety Signage for PPE | - Display appropriate safety posters near machines. |
| Changing Tyres Injury including amputation may result from split rim wheel assembly blow apart | - Vehicle is unloaded or load is reduced to a safe level prior to the commencement of tyre changing or wheel/ brake servicing work. - Chock wheels at the rear axle. - Use blocks or stands to stabilise machinery that is raised rather than relying on a jack. Prior to lowering vehicle or plant make sure you inform all persons in the vicinity of the vehicle of your intentions and have them, stand well clear. - When inflating tyres use a cage if one is available. - Inspect rims before placing tyres on them - Ensure rim pieces are seated properly. - Inflate tyres in 10psi stages and check the rim after each inflation. |
| Lifting and moving of larger tyres, parts and wheels | - Size and type of tyre and wheel is determined before the job is allocated, and so the appropriate number of people and mechanical aids assigned to job. - Mechanical equipment used to move or stack truck tyres and wheels. |
| Vehicle hoists | - Hoist inspections undertaken to manufacturer’s specifications and with reference to Australian Standard AS/NZS 2550.9. - Pre-operation inspections done daily. - Routine inspection and maintenance done at least every 3 months. - Comprehensive annual inspections conducted. - Results of all inspections, servicing and maintenance are recorded in a log book and available to users and maintenance or inspection personnel. - All servicing and maintenance carried out by a suitably qualified and competent person. - Operating and maintenance instruction plates are prominently displayed on hoist. - Safe Working Load prominently displayed on equipment. - The moving parts of the hoist or its load have a minimum 600mm clearance to any fixed structure or other equipment that moves. - Electrical hoists and wiring are tested as per AS/NZS 3000 Australian Wiring Rules. - Hoist controls in good condition, properly positioned and clearly marked for safe operation. - Dropper bar (safety prop) fitted to in-ground hydraulic ram type hoist which prevents hoist lowering to less than 760mm. - All hoist-safety devices operational. - All hoist operators are trained in the correct use and inspection of the vehicle hoist. Prior to lowering vehicle or plant make sure you inform all persons in the vicinity of the vehicle of your intentions and have them stand well clear. |

SWP- Workshop Safety	Authorised & Approved by Management	Page 2 of 4

#### This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-20

| Issue/Activity - Hazard | Control Measures |
| --- | --- |
| Hydraulic jacks and trolley jacks | - Hydraulic jack and trolley jack inspections undertaken using Australian Standard AS/NZS 2615 as a guide, eg - pre-operation inspections done daily, - routine inspection and maintenance done at least every 3 months, - annual inspections conducted. - Results of all inspections, servicing and maintenance are recorded in a log book and available to users and maintenance staff. - All servicing and maintenance carried out by a suitably qualified and competent person. - Operating and maintenance instruction plates available and prominently displayed on jack. - Safe Working Load prominently displayed on equipment. - Vehicle stands used at all times when vehicle raised by hydraulic- trolley jack. Prior to lowering vehicle or plant make sure you inform all persons in the vicinity of the vehicle of your intentions and have them, stand well clear. |
| Vehicle ramps and stands | - Vehicle ramps and stands marked with rated capacity (Safe Working Load). - Vehicle ramp and stands inspections undertaken. - Pre-operation inspections done daily. - Routine inspection and maintenance done at least every 3 months. - Annual inspections conducted. - Results of all inspections, servicing and maintenance are recorded in a log book and available. - Vehicles on ramps secured in such a way as to prevent movement (use of handbrake, wheel chocks, etc). Prior to lowering vehicle or plant make sure you inform all persons in the vicinity of the vehicle of your intentions and have them stand well clear. |
| Lock Out & Tag system | - All hoists, trolley jacks, stands and ramps that are found to be unsafe are immediately removed from service. - Tag Out system in place which ensures equipment not used until made safe. - If you are not sure of the company Lock Out & Tag procedure please ask |
| Forklifts and lifting equipment | - No High Risk Licence – Do Not Operate Forklift for load shifting tasks. Exception – Forklift is being serviced, maintained or being loaded for transporting. - Forklift operated on-site with strict traffic management. |
| Storage systems | - No tyre/parts storage racking above shoulder height. - Minimum stock on-site, based on usage rate. - Limits on pyramid stacking are strictly followed. - Rack sharp and heavy tools and other items at a low level. - Store heavy materials at a low level (ideally between knee and waist height). |

SWP- Workshop Safety    	Authorised & Approved by Management	Page 3 of 4

#### This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-20

| Issue/Activity - Hazard | Control Measures |
| --- | --- |
| Working at heights | - Maintain 3 points of contact at all times when climbing up and down from vehicles or mobile plant. - If having to work above head height use a suitable platform, if unsure speak with your supervisor. |
| Smoking | - No Smoking in the Workshop, any enclosed area or near hazardous or flammable goods. |
| Electrical Safety | - Refer to the Electrical Safety Tools & Equipment Procedure & your Employee Induction Booklet. |
| Hazardous Substances & Dangerous goods Chemicals | - Refer to the Hazardous Substances & Dangerous Goods Procedure & your Employee Induction Booklet. - Cover all wounds when handling substances, preferably using disposal gloves. - Clearly label all containers in use and in storage. - Food and drink preparation/consumption/storage is prohibited in areas where hazardous substances are stored and used. - Wash hands after activities and before leaving particularly when work has involved handling substances and animals. |
| Housekeeping | - At all times keep the workshop in a neat & tidy manner. - Return equipment and substances to designated storage places. - Minimise dust hazards. - Remove rubbish regularly, especially flammable waste — do not allow it to accumulate. |

SWP- Workshop Safety	Authorised & Approved by Management	Page 4 of 4

#### This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.