BENCH GRINDER

DO NOT use this machine unless you have been instructed in its safe use and operation.

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

Pre-Operational Safety Checks

- Check workspaces and walkways to ensure no slip/trip hazards are present.

- Ensure all guards and safety shields are in position before starting the grinder.

- Ensure that the wheels do not touch the work rest and that the gap between wheel and rest is no greater than 1.5mm.

- Check that wheels are running true and are not glazed or loaded.

- Locate and ensure you are familiar with the operation of the ON/OFF starter.

- Faulty equipment must not be used. Immediately report any suspect machinery.

### Operational Safety Checks

- Stand to the side of the wheels when starting up.

- Let the wheels gain maximum speed before starting to grind.

- Do not grind on the side of the wheel.

- Small objects must not be held by hand.

- Never leave the machine running unattended.

- Do not bend down near the machine whilst it is running.

- Never force the workpiece against a wheel.

- Slowly move the workpiece across the face of the wheel in a uniform manner.

- Coolant spilt on the floor should be immediately absorbed.

- Keep hands and fingers well clear of abrasive wheels.

- Use full face of grinding wheel.

- Materials may become hot when grinding.

### Housekeeping

- Lead and unit is to be tested and tagged as required by Australian Standards.

- Monitor grinding wheels and ensure even wear.

- Switch off the grinder.

- Leave the machine in a safe, clean and tidy state.

### Potential Hazards

- Abrasions

- Hot Metal - Burns to skin

- Sparks - Eye injuries

- Noise - Hearing damage

- Sharp edges and burrs

- Wheels ‘run on’ after switching off

- Entanglement

### A Bench Grinder MUST NOT BE USED FOR

- Use other than recommended tasks.

- Workpiece must never be held with gloves, cloth, apron or pliers.

- Grinding of Non-ferrous materials i.e aluminium, brass, copper, timber etc.

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.