| Hand Grout Pump sop |  |  |
| --- | --- | --- |
| Acronyms LOTO - Lockout/Tagout. PPE - Personal Protective Equipment. SOP - Safe Operating Procedure. | Hazards - What can cause harm? - Noise - Electricity - Rotating/moving parts - Dust, fumes, and vapours - Manual Handling - Poor housekeeping | Pre-Operational Inspection - Follow the pre-operational checklist in the manufacturer’s operational manual - Check pump: - Check all components in place as per manufacturers’ instructions - Verify all Guards and Shrouds are secure and function correctly (Check all fasteners and interlocks) - Emergency stop button clearly marked and operational - All switches and handles move freely (avoid accidentally starting machine) - Grout lines are as short as possible to reduce pumping distances. - Work area: - Consideration is given to the disposal of waste materials and washout residue - Source of solid materials (cement, etc.) readily accessible and an adequate supply of water available - Dry, level ground, free of oil or trip hazards - Sufficient lighting - Work area kept clean and tidy and is free from trip, slip, and fall hazards - Crystalline silica: When handling cement-based products - Ensure adequate ventilation Use water or fine mist to suppress dust at the point of generation where possible - Wear suitable breathing protection e.g., face mask. |
| Precautions - All operators must be trained and competent to operate a Hand Grout Pump safely. Operator training should include: - Workplace induction - Safe operating procedures - Manufacturer’s instructions - Nature of hazards - Fitting of PPE - Manual handling - Housekeeping - First Aid. maintenance - Follow LOTO procedures, record service/repairs information in Maintenance Log. Maintenance is conducted as per manufacturers’ instructions and by qualified persons only - When the machine is taken out of service for maintenance: Report ALL broken and/or malfunctioning equipment to a supervisor. Shut off and isolate the power supply - The power supply is clearly labelled/tagged “do not use.” |  |

| Hand Grout Pump sop |  |
| --- | --- |
| Operation - Operate in accordance with manufacturers’ instructions |  |
| - Wear eye and respiratory protection - Respirator with at least P1 filtration – fit-tested to the worker if half or full-face mask is used - Setup Hand Grout Pump as per manufactures instructions - Visually inspect that there are no foreign objects/old materials in either the pump or the mixer - Pre-pump check - With operating levers, valves, or handles in either “NEUTRAL” or “OFF” position and the primary power source turned OFF, fill the pump hopper with clear water - Switch on the machine and pump water out (check for blockages and correct pressures currently) - Mixer operation: - Place the measured quantity of water for your mix in the bowl. - Ensure that the valve with the hose going into the bowl is open and the second valve is closed. This will recirculate the water and help with mixing - Start the machine and add the required amount of cement/product and water - Mix the desired volume of grout following the product guidelines and any pre-planned requirements - Keep the mixer or mixers free of material build-up - When it is mixed completely, open the valve to the grouting job and close the cement recirculating valve - Keep all machine guards in place during use Pumping Operation: - Pump grout to the required location - Continuously monitor both the pumps and the mixers or mixers’ performance, being alert to any changes in conditions - Keep the outside of the machine clean - Ensure the correct pressure ratings are maintained throughout the procedure - Do not let the pump run when it contains no product 7. On Completion: - Dispose of excess production material in the correct manner - Wash out mixer tanks and agitators into the pump hopper and pump the resulting washout material through the grout hoses to a suitable disposal site - Rinse the mixers and charge the pump hopper with water to flush the pump and all grout lines - Drain any water out of the lines and hoses. Clean down the machinery and surrounding areas. | PPE - Do not run the pump without water - Do not run the pump with both valves in closed position, as doing so may burst the hoses - Do not leave the machine while it is running - Do not use the machine for other than its intended use. |

| HAZARD CHECKLIST (POWERED NON-MOBILE PLANT) | Make, Model, ID No.: |  |
| --- | --- | --- |
| Names of Representatives/Workers involved: |  | Date: |

|  | Y | N | NA |  | Y | N | NA |  | Y | N | NA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk assessment completed |  |  |  | Monitoring dismantle/ decommission |  |  |  | Guards are securely fixed |  |  |  |
| Daily Inspection conducted |  |  |  | Warning devices installed |  |  |  | Guards are in good condition |  |  |  |
| Daily logbook completed |  |  |  | Warning devices positioned for best effect |  |  |  | Guards are functional |  |  |  |
| Instruction manual available |  |  |  | Danger areas guarded: |  |  |  | Machinery doesn’t start if guards removed |  |  |  |
| SOP available |  |  |  | Entanglement risks |  |  |  | Controls labelled correctly |  |  |  |
| Maintained within a year |  |  |  | Shearing risks |  |  |  | Controls have easy access |  |  |  |
| The operator has a license |  |  |  | Crushing risks |  |  |  | Controls prevent accidental operation |  |  |  |
| No unauthorised alterations |  |  |  | Heat risks |  |  |  | Controls can be locked to “off.” |  |  |  |
| Intended for use as designed |  |  |  | Cold risks |  |  |  | E-stops are “stop & lock-off” type |  |  |  |
| Safety procedures in place for: |  |  |  | Laceration risks |  |  |  | E-stops marked & coloured red |  |  |  |
| Installation |  |  |  | Radiation risks |  |  |  | E-stops immediately accessible to operators |  |  |  |
| Assembly |  |  |  | Ejected materials |  |  |  | E-stops not affected by an electrical malfunction |  |  |  |
| Construction |  |  |  | Fluid /air under pressure |  |  |  | PPE supplied as required |  |  |  |
| Commissioning |  |  |  | Guarding follows hierarchy: - Fixed. If not practicable, then: - Interlocked. If not practicable, then: - Physical barrier. If not practicable, then: - Presence-Sensing |  |  |  | Protected from electric shock |  |  |  |
| Monitoring assembly etc. |  |  |  |  |  |  |  | Sufficient room for safe operation |  |  |  |
| Use |  |  |  |  |  |  |  | Correct for job |  |  |  |
| Cleaning |  |  |  |  |  |  |  | Correct attachments |  |  |  |
| Maintenance |  |  |  |  |  |  |  | LOTO procedures in place |  |  |  |
| Dismantle |  |  |  | Guards are of solid construction |  |  |  | Where “No” is selected, check with the Supervisor. |  |  |  |
| Decommission |  |  |  | Guards are difficult to bypass |  |  |  |  |  |  |  |