All Paneltec drill sites are to be visited and thoroughly inspected by operational and health, safety and environmental personnel to determine the topography, the suitability of the exit and entry points, surface features and surface evidence of buried services or structures to ensure the accuracy of the initial site information, and to gather detailed information for assembly of the Safety Plan.

The site survey should ensure that all utility plans are available on site, including gas, water, electricity, telecommunications, cable, sewer, storm water drains, and indicate the presence of any third-party pipelines that may be present.

Before You Dig Australia inquiry replies, made for the site are to be reviewed in detail by the site surveyors. The presence of all buried infrastructure is to be confirmed and marked on the ground, then potholed and locations confirmed by the use of Paneltec vacuum trucks.

Where pipelines are proposed through environmentally sensitive areas an Environmental Statement must be submitted to the EPA and approval granted.

If hazardous underground infrastructure is identified and potentially affected, the relevant network owner should be contacted for appropriate precaution measures.

Note: This includes all utility networks, as in the event of a high-pressure water main being damaged, flooding and possibly road collapse due to soil washout.

Evidence of recent redevelopment or ground disturbance should be taken into account, as it could be significant

or could indicate the presence of buried assets.

Note: For example, if the ground levels have changed, existing services could be at a different depth to that anticipated

and this would need to be taken into account when planning the works.

Having gathered information for the Drill, a review should be undertaken to confirm:

 The feasibility of the chosen technique for the project

 That the plant and equipment available is fit for purpose