DO NOT use this machine unless you have been instructed in its safe use and operation.

ANGLE GRINDER (HAND HELD ELECTRIC)

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

Angle grinders are one of the most dangerous hand tools in the manufacturing metal products industry. Kickback, where the disc is thrust away from the object it is grinding, can result in severe cuts to hands, arms, head, torso and legs. Discs can shatter or explode, sending pieces flying across the workshop. Most angle grinder injuries are from metal particles lodging in the operator's eye.

### What are Angle Grinders used for?

Angle grinders can be used for several functions including sanding, grinding and cutting processes, depending on the type of disc used.

Disc Selection: The disc must be suitable for the material (eg steel, masonry), the correct size for the grinder (this includes centre hole size), and capable of being used at the maximum r.p.m. of the grinder. If any doubt exists, the disc should not be attached to the grinder. Cutting discs should never be used for grinding and vice versa.

#### Hand held electric angle grinders over 125mm (5”) in capacity must not be used for the purpose of cutting.

Pre-Operational Safety Checks

- Check work area to ensure no slip/trip hazards are present.

- Ensure you are familiar with the operation of angle grinder.

- Check all safety features of the angle grinder.

- Inspect before use: for damaged parts, discs and cords for cracks or damage, all guards and shields in place. Faulty equipment must not be used. Immediately tag out and report suspect machinery to your supervisor.

- Flange and flange nuts must suit the disc and be in good condition.

- Discs should be stored and handled carefully to avoid damage.

- Check electrical lead for damage and currently tested and tagged.

- Circuit to be RCD protected. Check operation of RCD.

- Never use angle grinder around flammable liquids or gases.

- Do not wear loose clothing or jewelry.

- Always work in proper lighting

### Operational Safety Checks

- Grip grinder firmly with both hands.

- Hold side handle - NOT THE GRINDER - firmly when starting, as twisting motion can be Severe.

- Newly mounted discs should run free for one minute before cutting or grinding -Allow grinder to run- up to full rpm.

- The work item should be rigidly supported.

- Do not over-reach when using grinder, keep footing and balance at all times.

- Only the bottom flat section of wheel and lower edge is to be used.

- Do not apply excessive wheel pressure.

- Never apply pressure to stop a disc spinning.

- Always unplug tool before changing disc, servicing or when not in use.

#### When using cutting discs on electric, handheld angle grinders of 125mm (5”) capacity and below the following must be observed.

- Ensure wheel is of the correct type for job application.

- Speed of machine must not exceed speed rating of wheel.

- Ensure side handle is in place to allow adequate control by operator.

- Ensure back-plates and flanges are correct..

- Flanges must be fitted as illustrated in Operator’s Manual.

### Special Note

#### UNDER NO CIRCUMSTANCES IS THE CUTTING EDGE OF THE DISC TO FACE THE OPERATOR. ENSURE SAFETY GUARD IS ALWAYS BETWEEN THE OPERATOR AND THE WHEEL.

Housekeeping

- Lead to be tested and tagged as required by Australian Standards.

- Monitor grinding wheels and ensure even wear.

- Check for damage to the casing and body of the equipment.

### Potential Hazards

- Abrasions

- Burns to skin

- Eye injuries

- Electric Shock

- Hearing damage

- Most angle grinder injuries are from metal particles lodging in the operator's eye.

- However, the most serious injuries are from kick-back, where the disc is thrust back violently towards the operator.

- Discs can shatter or explode, sending pieces flying in all directions.

- Because angle grinders are designed for grinding and not for cutting, the use of cutting discs with angle grinders exposes operators to even greater risks. Use another cutting tool that is available AN ANGLE GRINDER SHOULD NOT BE USED AS A CUTTING TOOL.

- Wheels worn small through use should be discarded and NEVER used on smaller machines.

### Maintenance

All angle grinders should be regularly checked for electrical safety to ensure:

- No breaks or damage to the machine's outer body.

- All screws tight.

- Brush caps intact and firmly in position?

- Sheathing of flexible cord held securely at the tool.

- No exposed wires.

- Flexible cord in good condition, free from cuts or breaks.

- Plugs and extension sockets free from cracks or damage.

- A safety switch or residual current device (RCD) is always used. All defects repaired by a licensed electrical person.

### DO NOT Use Angle Grinders for:

- Grinding of Non-ferrous materials i.e. aluminium, brass, copper, timber etc.

- Use other than recommended tasks.

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.