# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-13

SAFE WORK PROCEDURE

HAND HAMMERS IN METAL WORK

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be used when using this machine. |

Only one person may operate this machine at any one time.

Introduction

Hand hammers in metal work are generally used to strike pins, punches, rivets, cold chisels and "dollies". Light hammers are used for stamping job numbers on structural steel work or marking drill points on metal.

Heavy hammers, also known as sledgehammers, long - handle hammers or double – handed hammers, have hardened steel heads weighing up to 12 kg. Often, they are used to hammer a pin into a hole while another person holds either the pin or the dolly to take the shock between two hard metal surfaces.

### Did you know?

Hand hammers are among the top five causes of injury in the manufacturing metal products industry.

Most injuries are broken or badly bruised fingers and thumbs. A badly broken finger or thumb can result in time off work and leave lasting disabilities.

Other injuries, especially to eyes, are caused by chips of metal flying off damaged hammer faces or burred punch and chisel heads.

### Working with light hammers

All users of light hammers shall be trained in this safe operating procedure and ensure they:

- Grasp the hammer a little down the handle, so the butt stands out less than 25 mm past the little finger.

- Choose a hammer with a striking face diameter approximately 25 mm wider than the face of the tool being struck.

- Strike a hammer blow squarely, with the striking face parallel to the surface being struck.

- Avoid making glancing blows.

- Use only the striking face for hammering, never the side or flat of the hammer.

- Never use one hammer to strike another hammer. The hardened steel on both can shatter.

SWP-  Hand Hammers in Metal Work V10-31.08.25	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

### Safety devices can include:

PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-13

- Screens to guard bystanders from flying metal fragments.

- A sponge rubber pad or solid rubber hand-guard to be fitted on the shaft of striking tools.

- Gloves for the non-hammer hand on specific heavy striking tool jobs.

- A chisel-holder handle to keep the hand clear of glancing blows.

- Use automatic punches wherever possible.

### Working with heavy hammers

  - Holding the pin or dolly while another person strikes it with a sledgehammer is extremely hazardous, and is strictly forbidden. ALWAYS use long-handled tongs or grips to hold the pin or dolly.

All users of heavy hammers shall be trained in this safe operating procedure and make sure:

- The correct weight hammer is used for the job.

- The hammer operator has the skills and strength for the weight of hammer and the

- Task.

- They avoiding striking hardened metal objects or tools, which may chip or shatter, sending particles flying at high speed.

- NEVER shorten the handle of a heavy hammer; you may strain your back.

### Personal protective equipment

The appropriate protective equipment shall be used, for example:

- Goggles, safety glasses or face shield for operators and others nearby. Metal fragments can injure the eyes of others working nearby if safety glasses do not have adequate side protection.

- Safety boots to protect the feet against dropped metal and tools.

- Personal hearing protectors, earmuffs or earplugs.

### Maintenance

Hazards increase when flaws or faults develop in the hammerhead or the handle. All hand hammers, punch and chisel heads need to be regularly checked to ensure:

- The head is firmly fixed to the handle and the wedge is tight.

- Striking faces of the head are smooth and free from oil.

- Striking faces are free from chips or burrs.

- Handles are free of sharp edges, splinters and surface roughness.

- Wooden handles are varnished or waxed to give a smooth non-slip finish. Avoid painting handles that may obscure defects.

All damaged hammers, punches, chisels and handles need to be discarded. Never use a repaired item.

SWP-  Hand Hammers in Metal Work	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.