PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-18

SAFE WORK PROCEDURE

WACKER PLATE

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Wear protective Gloves. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Rings and jewellery must not be worn. |

Only one person may operate this machine at any one time.

Safe Operation

- Do not smoke while operating or refueling the machine.

- Always operate with all the safety devices in place and in working order.

- Always guide the machine so that hands do not get caught on obstacles.

- Keep both hands firmly in contact with the guide handle while operating machine.

- Stop the engine when leaving the machine or when refueling.

- Do not refuel hot or running engine. Do not spill fuel over hot engine parts.

- Do not refuel near open flame. Do not operate with fuel tank cap loose or missing.

- Always keep hands, feet and clothing away from moving parts.

- Always wear safety boots when operating this equipment.

- Always operate in well ventilated area. Engine emits poisonous fumes.

- Be aware of surface condition and use special care when working on uneven ground or when compacting coarse material.

- Never allow bystanders to stand close to the machine while it is being started or while it is running.

- Transport machine upright only.

- Seek assistance when lifting machine onto back of vehicle or use a lifting device.

SWP- Wacker Plate Ver 10 – 31.08.2025	Authorised & Approved by Management	Page 1 of 1

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.