# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-15

SAFE WORK PROCEDURE

OXY-ACEYLENE

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Welding goggles must be worn at all times. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Oil free leather gloves must be worn when welding. |

Only one person may operate this machine at any one time.

Pre-Operational Safety Checks

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Keep area clean and free of grease, oil and other flammable material.

- Gas hoses must be in good condition and not create a tripping hazard.

- Before lighting up make a visual inspection of all equipment for damage.

- Check that the area is well ventilated and fume extraction unit is on.

- Faulty equipment must not be used. Immediately report suspect equipment.

### Pressure Setting

- Check that the oxygen and acetylene regulator adjusting knobs are loose.

- Check that both blowpipe valves are closed.

- Slowly open the cylinder valves on each cylinder for half a turn only.

- Screw in the regulator adjusting knobs slowly until the delivery pressure gauges register 70kPa.

- Purge and check for constant oxygen gas flow.

Open the oxygen blowpipe for 2 seconds valve and check the delivery gauge. If necessary, re-adjust the oxygen regulator to achieve a 70kPa pressure.

Close the oxygen blowpipe valve.

- Purge and check for constant acetylene gas flow.

Open the acetylene blowpipe valve for 2 seconds and check the delivery gauge. If necessary, re-adjust the acetylene regulator to achieve a 70kPa pressure.

Close the acetylene blowpipe valve.

### Lighting Up

- Open the acetylene blowpipe valve slightly and light the blowpipe with a flint lighter.

- Continue to slowly open the acetylene valve until the flame no longer produces soot.

- Slowly open the oxygen blowpipe valve until a neutral flame is produced.

SWP- Oxy-Acetylene V10- 31.08.2025	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

### Shutting Off Blowpipe

PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-15

- Close the acetylene blowpipe valve first.

- Then close the oxygen blowpipe valve.

### Closing Down

- Close down both cylinder valves.

- Open oxygen blowpipe valve to allow the gas to drain out.

- When oxygen gauges read zero, unscrew regulator-adjusting knob.

- Close oxygen blowpipe valve.

- Turn off acetylene cylinder valve.

- Open acetylene blowpipe valve and release gas.

- When acetylene gauges read zero, release regulator adjusting knob.

- Close acetylene blowpipe valve.

### Housekeeping

- Hang up welding blowpipe and hoses.

- Turn off fume extraction.

- Leave the work area in a safe, clean and tidy state.

### Potential Hazards

- Burns

- Flying sparks

- Fumes

- Flashbacks

  - Radiation damage to eyes

  - Explosion by gas leakage

  - Radiation burns to hands & arms

### Forbidden

- Lighting blowpipe with matches or lighters

- Using oil, grease or other hydrocarbons

SWP- Oxy-Acetylene	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.