# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-17

SAFE WORK PROCEDURE

SPRAY PAINTING

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | A respirator fitted with approved respiratory cartridges must be worn. |

Only one person may operate this machine at any one time.

Pre-Operational Safety Checks

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Locate and ensure you are familiar with the operation of the ON/OFF starter.

- Locate and check air isolation valve.

- Visually check that the primary spray booth filters are in sound condition.

- Check hoses and fittings are in good condition and are properly connected.

- Check air pressure regulator and pressure gauge operates.

- Ensure fume extraction unit is on before beginning to spray.

- Faulty equipment must not be used. Immediately report suspect plant.

### Operational Safety Checks

- Close water drain tap on both air compressor and regulator liquid traps.

- Adjust pressure regulator to suit work requirements.

- Place the workpiece in a position where the spray gun operator always has the spray between themselves and the exhaust fan.

### Housekeeping post spray-operations

- Switch off the compressor.

- Turn off fume extraction.

- Ensure solvents, paints etc. are stored in an approved storage facility.

- Leave the work area in a safe, clean and tidy state.

SWP- Spray Painting Ver 10 31/08/2025	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-17

### Potential Hazards

- Compressed air

- Hazardous substances

- Whipping hoses

- Explosions

- Spontaneous combustion

### Never use Spray Painting Equipment

- Directing air stream at yourself or other people

- Spraying without a compliant respirator

SWP- Spray Painting	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.