# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-08

SAFE WORK PROCEDURE

DRILL PRESS

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

### Pre-Operational Safety Checks

- Check workspace and walkways to ensure no slip-hazards are present.

- Check that the drill chuck guard is in position.

- Ensure the chuck key (if used) has been removed from the drill chuck.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Follow correct clamping procedures to ensure work is secure.

- If the job obstructs the walkway erect a barricade.

- Adjust spindle speed to suit drill or cutter diameter.

- Faulty equipment must not be used. Immediately report suspect equipment.

### Operational Safety Checks

- Never leave the Drill Press while it is running.

- Before making adjustments or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

- Feed downwards at a sufficient rate to keep the drill cutting.

- Feed with care as the drill breaks through the underside of the work.

- Use a safe working posture (beware of hair catching).

- Keep handles dry, clean and free from oil and grease.

- Before connecting the tool to a power source (receptacle, outlet), be sure voltage supplied is the same as that specified on the nameplate of the tool

- Use the drill press in a well-lit area and on a level face, clean and smooth enough to reduce the risk of trips and falls.

- Never place your fingers in a position where they could contact the drill bit or other cutting tool parts.

- Always support work piece so it won't shift or bind on the tool.

- Always position backup material underneath the work piece.

SWP-Drill Press V10 31.08.25	Authorised & Approved by Management	Page 1 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-08

- Whenever possible, position the work piece to contact the left side of the column.

- When using a drill press vise, always fasten to the table.

- Never do any work "free hand". (Hand holding a work piece rather than supporting it on the table)

- Never move the head or table support while the tool is running.

- Before starting the operation, jog the motor switch to make sure the drill bit or other cutting tools do not wobble or cause vibration.

- If a work piece overhangs the table such that it will fall or tip if not held, clamp it to the table or provide auxiliary support.

- Use fixtures for unusual operations to adequately hold, guide and position the work piece.

- Use the spindle speed recommended for the specific operation and work piece material.

- Never climb on the drill press table; it could break or pull the entire drill press down on you.

- Turn the motor switch off and unplug from the power source when not in operation.

- To avoid injury from thrown work or tool contact, do not perform layout, assembly, or setup work on the table while the cutting tool is rotating.

### Housekeeping

- Switch off the machine when leaving the area.

- Leave the machine in a safe, clean and tidy state.

- Place swarf into correct waste receptacle.

### Potential Hazards

- Hair/clothing entanglement - rotating spindle/drill

- Eye injuries

- Flying swarf/chips

- Sharp edges & burrs

### A Drill Press MUST NOT BE USED FOR:

1. Tasks other than drilling.

SWP-Drill Press   V10 31.08.25	Authorised & Approved by Management	Page 2 of 2

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.