Personal Protective Equipment (PPE)

SAFE WORK PROCEDURE – SWP 34

Wear Construction Safety Gloves.

(Eg: Razor shield 5)

| Safety glasses must be worn at all times in work areas. |  |
| --- | --- |
| Sturdy footwear (steel cap boots) must be worn at all times in work areas. | Close fitting/protective, Hi vis, long sleeve long pant clothing must be worn. |
| Hard hat as required. |  |

Safe Work Procedure – 34  (SWP)

POTENTIAL HAZARDS AND ASSOCIATED RISKS

Slip, Trips, Falls

- Slip, trip, fall due to cluttered, uneven or slippery work surfaces

- Slip, trip, fall due to steep working surfaces

Manual Task Injury

- Manual task/musculoskeletal injury from incorrect manual handling

Vehicle Collision or Pedestrian Injury Collison

- Vehicle collision/accident from incorrect/faulty operation of Sentrigates

- Injury to workers or public from incorrect/faulty operation of SentriGates

PRE-OPERATIONAL SAFETY CHECKS

- Complete, Site-Specific Risk Assessment (SSRA)

- This Procedure is to be used secondary to the requirements outlined in SWMS 48 Temporary Traffic Management

- Ensure all workers are trained in “Manual Handling”

- Ensure the Traffic Management Plan and Traffic Guidance Scheme is available and accurate for the site.

- Ensure all Traffic Controllers (TC’s) are trained and certified in:

- RIISS00058 TC1

- RIISS00060 TMI1

- RIISS00059 TC2

- RIISS00061 TMI2

ABOUT

Viatec’s vehicle mounted SentriGates are used to separate traffic controllers from direct interaction with traffic to prevent harm on the worksite.

The SentriGate (boom-gate) must be operated by a traffic controller who is trained, competent and authorised by Viatec Management to operate a manually controlled gate.

Our gates must be operated in accordance with the any local jurisdiction approved procedure that may be in place.

OPERATING PROCEDURES

- Complete Viatec Traffic Solution SSRA in Lucidity

- Complete Tailgate Meeting with TC’s and client

- Road Measurement Survey – Gauge 3.5m gate and vehicle width

- Implement Traffic Guidance Scheme (TGS) & or Traffic Management Plan (TMP) in accordance with SWMS 48

- Extend yellow post retaining bar from chassis of utility, select hole length, tighten locking bolt and remove D-Pin

- Remove the SentriGates from the Ute storage cupboard, this must be undertaken by two persons

- Locate the support pole into the centre of the retention slot ensuring the correct alignment has been achieved

- Tighten locking pin leaving the D-ring off, ensure the gate is on an appropriate angle 90 degrees to the ute

- Connect the 12V Power supply to the electrical port near Ute chassis

- Ensure all gate and extension bar retaining pins are tight, get your co-worker to verify this

- Open the yellow SentriGate cover plate by using the key from the SentriGate cabinet

- Turn the rocker switch on the LHS of the inside of the box to the ‘on’ position

- Test the voltage level of the battery by pressing the black button switch at the rear of the cabinet it should be 12V or near to it.

- Replace or charge battery if necessary (<9V) – spares are in plastic yellow box inside the ute cab

- Close the yellow panel and relock the cover to a fixed position

- Remove gate blade locking/retaining bar at base of blades if its locked/engaged

- Test the SentriGate to ensure correct operation of all functions

- Ensure the controller used is for that gate (1&3) or (2&4) #’s are written on controller and gate cupboard

- Move gate back to the vertical position

- Move up to the secondary or additional gate sites

- Repeat 5 – 19 above

- Traffic Controllers to take their posts at each gate

- Check radios, ensure communication is clear and concise

- Begin temporary traffic management operations

# ENDING OPERATIONS

- Reverse procedures above - starting at 24 through to 4.

DO NOT

- Do not use if gate is faulty. Attach an Out of Service tag and report to management immediately

- Do not lift SentriGates if you are alone – must always be a two-person lift

- Do not leave gates unattended whilst in use

- Do not drive the vehicle with the gates, deployed and positioned within the engagement slot