ABRASIVE CUT OFF SAW (METAL)

DO NOT use this machine unless you have been instructed in its safe use and operation.

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

Pre-Operational Safety Checks

- Ensure the saw is properly secured to a worktable by bolts/ clamps at approximately hip height.

- Ensure the saw is operated on an RCD protected circuit.

- Use abrasive cut off wheels with the correct size arbor hole.

- Use abrasive cut off wheels which have a maximum safe operating speed greater than the “no load RPM” marked on the machine’s nameplate.

- Before each use inspect the cut off wheel for chips and cracks.

- Check workspaces and walkways to ensure no slip/trip-hazards are present.

- Ensure all guards are in place, in working order, and in proper adjustment and alignment.

- Ensure the depth stop is properly adjusted.

- Ensure you are familiar with the operation of the ON/OFF starter.

- Keep table and work area clear of all tools and off-cut material.

- Faulty equipment must not be used. Immediately report suspect equipment.

### This machine and abrasive wheel is NOT intended to cut wood or wood products.

Operational Safety Checks

- Ensure all adjustments to machine are secure before making a cut.

- Do not hold a workpiece by hand. Workpiece will become very hot while being cut.

- Use the vice to clamp the work and properly support the over-hanging portion or the workpiece level with the base of the machine.

- Do not use a length stop on the free off-cut end of a clamped workpiece.

- Allow the machine to reach full speed before contacting the workpiece.

- Do not have any part of your body in line with the path of the abrasive disc.

- Ease the abrasive disc against the work piece when starting to cut. Do not force tool.

- Keep hands away from cutting area. Do not attempt to remove cut material while wheel is moving.

- After finishing the cut, release the switch, hold the saw arm down and wait for the disc to stop before removing work or off-cut piece.

- Disconnect the plug from the power source and bring the machine to a complete standstill before making any adjustments.

- MACHINE MUST ALWAYS BE SWITCHED OFF BEFORE ANY FURTHER TASKS ARE UNDERTAKEN.

### Housekeeping

- Ensure ventilation openings and switch levers are kept clean and free of foreign matter.

- Leave the machine in a safe, clean and tidy state.

- Clean up area of any debris and put unwanted/ surplus material away.

- Inspect cutting wheel and replace if worn or damaged.

### Potential Hazards

- Metal sparks

- Noise

- Sharp metal burrs

- Contact with rotating disc

- Eye injuries

- Burns from workpiece

### A Cut Off Saw MUST NOT BE USED FOR:

- Use other than recommended tasks.

- Do not grind on the side of abrasive cut off wheels

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.