# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-09

SAFE WORK PROCEDURE

FLOOR JACK

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Wear protective Gloves. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. |  |

Only one person may operate this machine at any one time.

Hydraulic and pneumatic jacks are the most common jacks. They can be mounted on slides or on a wheeled trolley. The size of jack you use will be determined by the weight of the vehicle you want to lift. The objective of this procedure is to show you how to lift and secure a vehicle with a floor jack and jack stands.

### Preparation and Safety

#### Objective

- Lift and secure a vehicle or item with a floor jack and jack stands.

#### Personal safety

Whenever you perform a task in the workshop you must use personal protective clothing and equipment that is appropriate for the task, and which conforms to your local safety regulations and policies. Among other items, this may include:

- Work clothing - such as coveralls and steel-capped footwear

- Eye protection - such as safety glasses and face masks

- Ear protection - such as earmuffs or earplugs

- Hand protection – such as suitable gloves and barrier cream

- Respiratory equipment – such as face masks and valved respirators If you are not certain what is appropriate or required, ask your supervisor.

#### Safety check

- Make sure the jack and stands you are using are suitable for the job.

- Never lift a vehicle or item that is heavier than the jack’s rated capacity.

- When supporting an item/vehicle always use matched pairs of jack stands.

- Never support a vehicle on anything other than jack stands.

- Do not use wood or steel blocks to support the vehicle. They may slide or split under the weight of the vehicle.

- Do not use bricks to support the vehicle. They will shatter under the weight of the vehicle.

SWP- Floor Jack   V10 31.08.2025	Authorised & Approved by Management	Page 1 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-09

- Make sure that you understand and observe all legislative and personal safety procedures when carrying out the following tasks. If you are unsure of what these are, ask your supervisor.

#### Points to note

- There are three types of workshop jacks: hydraulic, pneumatic and mechanical.

- Hydraulic and pneumatic jacks are the most common. They can be mounted on slides or on a wheeled trolley.

- The size of jack you use will be determined by the mass of the vehicle you want to lift. Most workshops will have a jack that has a lifting capacity of about 2 ½ tons/tonnes. If the vehicle is heavier than that, or if the vehicle is loaded, you will need to use a jack with a greater lifting capacity.

- Always check the vehicle service manual or owner’s manual to determine the best position to lift and support a vehicle. Some vehicles require special attachments to be fitted before they can be lifted.

- Do not jack or support a vehicle under any independent suspension components. They are not strong enough to support the weight of the vehicle.

- Make sure the vehicle is positioned on a firm level surface.

- Make sure the jack stands are in good condition before you use them to support the vehicle. If they are cracked or bent, they will not support the vehicle safely.

Step-by-step instruction

#### Position the vehicle

Position the vehicle on a flat, solid surface. Place vehicle in first gear or park and set the emergency brake. Turn off engine. Place blocks in front of and behind the wheels that aren’t going to be raised off the ground.

#### Inspect the floor jack

Before you use the jack, check for leaks in the hydraulic system. Check the pad, or saddle, and the wheels of the jack. They should rotate freely and be undamaged. Check the manufacturer’s label on the jack. The specifications will tell you the maximum load weight it will bear, so it must suit the vehicle you want to raise.

#### Check the vehicle handbook

Refer to the owner’s manual to find out where you can safely place the jack without damaging the vehicle. This is usually a major point on the chassis, a cross member or axle unit.

#### Select the jack safety stands

Before operating the jack, select two safety stands of the same type, suitable for the weight of the vehicle. Check the stands for any cracks, and if necessary, lubricate the threaded adjusting post with a few drops of oil. Place one stand on each side of the vehicle at the same point. Adjust them so that they are both the same height, and high enough to slip under the vehicle once you’ve raised it.

#### Position the jack

Roll the jack under the vehicle, and make sure the pad, or saddle, is positioned correctly under the frame or cross member. Turn the valve on the top of the jack handle clockwise and begin pumping the handle up and down until the jack pad touches the vehicle and begins to lift it.

#### Check position of jack

Once the vehicle’s wheels lift off the floor, stop and check the placement of the jack pad under the vehicle to make sure there’s no danger of slipping. Double check the position of the wheel blocks to make sure they haven’t moved. If the vehicle is stable, continue lifting it until it’s at the height where you can safely work under it.

SWP- Floor Jack   V10 31.08.2025	Authorised & Approved by Management	Page 2 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

#### Position the safety stands

PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-09

Slide the two safety stands underneath the vehicle. Make sure they’re positioned at a point that can sup- port the weight. Both stands should be adjusted to the same height and placed as far apart as practical.

#### Lower the vehicle onto the stands

Turn the valve on the jack handle counterclockwise and gently lower the vehicle onto the stands. When the vehicle has settled onto the stands, lower the jack completely and remove it from under the vehicle. Repeat this process to lift the other end of the vehicle if required. Be aware that the vehicle is now supported on jack stands and will not be as stable as it would be if the wheels were on the ground.

#### Working underneath vehicle or equipment

Before working under a vehicle or equipment, inform other workers in the area that you are working under the vehicle. Always remove the vehicle’s ignition keys and keep them on you to ensure no-one else can start the vehicle. Tag-out the ignition and lock-out if possible.

#### Raise the vehicle off the stands

When you’ve finished working under the vehicle, make sure you’ve removed all tools and equipment before you attempt to lower it. Use the jack to raise the vehicle off the safety stands. Slide out the safety stands from under the vehicle.

#### Lower the vehicle

Turn the valve on the jack handle counterclockwise very gently to lower the vehicle to the ground. Do not allow the vehicle to drop quickly or you may cause serious damage. Return the floor jack, the safety stands and the wheel wedges to their storage area before you continue working on the vehicle. Remove all tag-out labels and reinsert ignition keys.

SWP- Floor Jack	Authorised & Approved by Management	Page 3 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.