# PANELTEC PTY LTD

A.B.N. 12 128 689 412

SAFE WORK PROCEDURE

WORKING ON FOOT IN PROXIMITY TO MOVING PLANT

2025 - 2027

SWP-19

Reference: Based on new guidelines for Working on Foot in the Vicinity of Moving Plant set out in

section 9.26 of the RMS (RTA) Traffic Control at Worksites Manual.

#### This SWP sheet has been developed to ensure personnel are safe while working on foot in the vicinity of mobile plant or vehicles.

Vehicle Movements

Vehicle movements within worksites must be planned, taking into account:

- All mobile plant.

- Delivery vehicle.

- Other site traffic; and

- Site workers on foot and other pedestrians.

Key hazards identified:

- Reversing plant

- Delivery vehicles - (asphalt trucks & cover trucks)

### Risk factors to be considered:

- Unauthorised persons entering the work site.

- Proximity of people to mobile plant.

- Vehicles being loaded or unloaded.

- Plant being operated in reverse.

- Noisy work locations masking the sound of approaching plant.

- Inappropriate location of compounds or site amenities.

- Lack of adequately defined and safe crossing points.

- Working at night.

- Limited sight distances.

- Crews unfamiliar with the site.

- Working beneath power lines.

- Servicing and/or floating of plant items.

- Parking plant or vehicles in inappropriate locations ie too close to moving plant or in blind spots.

- Workers jumping onto, out of or riding on plant.

- Volume, speed and clearance to any adjacent traffic.

SWP- Working on foot in proximity to moving plant	Authorised & Approved by Management  Ver 10 – 31.08.2025	Page 1 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

### Safety controls

Clearances From Ground Personnel

PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-19

All operators/drivers of mobile plant/vehicles must:

- Maintain a 3 metre front & rear clearance and a 1 metre side clearance from all ground personnel; and

- For Skid steer loaders, maintain a 5 metre side and rear clearance from ground personnel; and

- If clearance distances are breached the operator/driver must STOP, sound horn twice and wait for personnel to move outside the clearance distance. Exceptions are pavers and rollers rolling Asphalt adjacent to or up to the rear of the paver.

#### All Ground Personnel must maintain a:

- 3 metre front and rear clearance and a 1 metre side clearance from moving mobile plant and vehicles; and

- 5 metre side and rear clearance from skid steer loader/s.

Exceptions are pavers, profilers, spotters and rollers rolling asphalt adjacent to or up to the rear of the paver.

#### Exclusion zones

- As a general rule, workers are not to work within 3 metres of moving plant.

- Where the activity requires workers within 3 metres or within the area influence, you must ensure that:

  - Workers are properly trained

  - A spotter is assigned to the plant or vehicle

  - Plant Operators and vehicle drivers are advised of the proximity of workers on foot (or pedestrians etc).

  - Workers not to use mobile phones

- Barriers and exclusion zones should be considered to delineate plant movement areas and prevent access.

#### Reversing Exclusion Zone

The direct reversing path of any vehicle with restricted rear vision is an exclusion zone. No personnel are allowed in the exclusion zone while the vehicle with restricted rear vision is reversing.

#### Spotters/observers

- Use of spotters is required for plant and vehicles in the vicinity of workers on foot.

- Ensure communication systems are established between observers, workers on foot and plant/truck operators.

- Spotters must have the authority to stop work where a danger exists for workers on foot from the plant or vehicles

#### Alarms

- Ensure that plant items and delivery vehicles are fitted with operational and automatically activated, clearly audible reversing alarms and warning lights before they are allowed into the site.

#### Personal protective equipment

- Approved high visibility clothing and other PPE must be worn by all site personnel and pedestrians/visitors to the site.

SWP- Working on foot in proximity to moving plant	Authorised & Approved by Management	Page 2 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.

# PANELTEC PTY LTD

A.B.N. 12 128 689 412

## SWP-19

#### Central reporting point for delivery vehicles eg Truck marshalling area

- Have a designated central point where all delivery drivers must report on arrival at the site.

#### Site induction

- Site induction is a critical means of communicating site specific safety rules including vehicle movements to all workers and visitors to asphalting/spray sealing sites.

- Site inductions should be immediately repeated as site conditions and safety measures change.

- Toolbox meetings provide an alternate and/or supplementary means of ensuring that all workers and visitors on site are kept up to date with changes.

#### New technology

Technological solutions that could be considered include:

- Rear-mounted vehicle cameras for reversing plant although factors such as night work, light levels outside and inside the vehicle must also be considered.

- Sensor alarms that warn drivers and plant operators of their proximity to physical objects and people.

- Interlock devices that prevent vehicle or plant movement outside a pre-determined range.

SWP- Working on foot in proximity to moving plant	Authorised & Approved by Management	Page 3 of 3

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.