PANELTEC PTY LTD

A.B.N. 12 128 689 412

SWP-16

SAFE WORK PROCEDURE

POWER DRILL

DO NOT use this machine unless you have been instructed in its safe use and operation.

2025 - 2027

Personal Protective Equipment

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Sturdy footwear must be worn at all times in work areas. | Close fitting/protective clothing must be worn. |
| Hearing protection must be used when using this machine. | Gloves must not be worn when using this machine. |
| Rings and jewellery must not be worn. |  |

Only one person may operate this machine at any one time.

PRE-Operation:

- Task is clearly understood.

- Drill bit is tight in chuck and chuck key has been removed (if applicable).

- Drill has been tested & tagged (3 mthly for off site construction, 6 mthly in workshop).

- Work piece is secured & correct drill speed has been set.

- Where necessary, depth stop for drill has been set or wood to be drilled is rested on scrap.

- Correct bit has been selected for material being drilled.

- Identify ON/OFF switch and emergency stop button.

# Operation:

- Check that drill bit runs ‘true’ and does not wobble.

- Keep hands clear of work piece and away from rotating tool.

- Back out frequently on deep cuts to clean and cool drill bit.

- Turn off drill before speed settings or bit is changed.

- Exercise caution when removing hot drill bit.

# POST-Operation:

- Switch off drill before removing waste material from the drill table.

- Make sure good housekeeping practices are in place to minimise dust build-up.

- Return clean drill bit to drill rack and clamps to their storage area.

SWP- Power Drill   V 10 – 31.08.2025	Authorised & Approved by Management

This SWP does not necessarily cover all possible hazards associated with the machine/equipment.

It is designed to act as a reminder to users/operators prior to machine/equipment use.                                                           Page 1 of 1