# Environmental Risks

Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites.

# Objective

To work cooperatively with the clients representative to preserve site of heritage including cultural heritage and/or archaeological significance.

# Target

Not to lose, destroy or deface any sites of historical or archaeological significance.

# Control Measures

- Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist.

- Develop a project specific procedure based on the findings and recommendations of the preliminary survey, which takes into account the nature, and timing, of construction. This procedure should address delineation of significant area, restricting disturbance to required area, imposition of safe working distances from significant sites, and how construction work will be undertaken to minimise impact.

- Ensure that the appropriate permits/ authorisation (if required) have been received prior to undertaking work in these areas. Ensure relevant representatives are advised of construction  program in advance so that they can be on-site during construction if necessary.

- Rare trees may need to be protected or relocated which would require a permit.

- Comply with any special requirements of the job specifications.

- Explain all requirements clearly to workers at site induction.

- Co-operate with any directions provided by the relevant authority representatives while in significant areas.

- If any Aboriginal archaeological sites or artefacts are discovered during the works, ensure work ceases immediately in the vicinity and that relevant government department is contacted for further advice and action.

# Monitoring & Measurement

By observation and communication with client and/or specialist consultants. Toolbox meeting minutes should also be used to ensure that all workers on-site have been made aware of their responsibilities with respect to this matter.