# Environmental Risk

Some works may require the removal of vegetation. Native fauna may be at risk from excavation works.

# Objective

To reduce the impact that operations have on the flora and fauna.

# Target

- Limit disturbance to the design area and relocate fauna if possible

- Comply with the project specifications

# Control Measures Vegetation

- Assess work area for significant vegetation: size of trees, faunal habitat

- Identify any vegetation you may think is important that has not been identified in the specification i.e. old trees and habitat trees. Advice from the Department of Sustainability & Environment (DSE) may be obtained if unsure.

- Assess the design impact on vegetation, i.e. it may be possible to alter the design slightly to save vegetation

- Pruning should be undertaken by an approved person

- If revegetation is required, ensure plants for revegetation are from the local area.

# Seek Expert Advice

- If native fauna are present or thought to be, then advice should be sought from the DSE or the Environmental Representative

- Injured fauna should not be handled unless safe to do so. The welfare of the animal must be given top priority. Be aware that;

o	All native animals including snakes are protected

# Operations

- Define work and exclusion areas using fencing and signage.

- Ensure all machinery is to be thoroughly washed down prior to commencing works on-site to prevent spread of foreign seed.

- Vehicles exiting the site should also be thoroughly washed down.

- Use defined access tracks and roads for entering and moving around the site

# Soil

- Strip topsoil, if any, for turn around points, parking areas etc. prior to using these areas. Stockpile soil for site rehabilitation/re-use.

- When works are complete, replace topsoil and dress to allow native seeds to revegetate the area disturbed.

- Clearly explain and identify all requirements to staff at site induction.

# Maintenance

Maintain barricades and marking & inspect regularly for compliance.