# Environmental Risks

During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways.

# Objective

To contain prime and bitumen products and comply with EPA and work cover requirements at all times.

# Target

- No pollution from prime or bitumen.

# Control Measures

  - Plan prime or bitumen operation so that it is conducted during fine weather.

  - Have material available to clean up a spill if it occurs and dispose of contaminated material according to the Disposal of Prescribed Wastes Work Instruction.

  - Protect storm drains during and after application to prevent pollution.

  - Closely monitor prime/bitumen during and after application to prevent pollution.

  - Closely monitor prime profiles so that excessive prime is not used during paving operations.

The information above is included in the EPA & AAPA guidelines for environmental management titled “Spraying Bituminous Materials”.

# Maintenance

  - Waste disposal areas should be regularly inspected to ensure adequate disposal area is available. Storm drain protection prior to prime should be inspected.

# Monitoring & Measurement

Site inspection using the Daily/Weekly Environmental Inspection and Risk Assessment Form and review of documentation to demonstrate waste has been disposed of in accordance to EPA guidelines.