# Environmental Risks

The potential for wind-blown dust to be generated on construction work sites could be high. The amount of dust generated depends on planning, weather, activities, materials being worked, controls in place, etc.

The other form of air pollution potentially generated on site is from plant emissions. Such emissions are usually as a result of poorly maintained or old equipment being used.

# Objectives

Dust generated to be minimised and controlled at all sites

Ensure all plant & equipment is maintained to the standard that there are no adverse plant emissions.

# Targets

- No dust moving off-site and minimum dust on-site.

- Zero complaints from residents, public, client, EPA or council

- Reduced emission levels on plant.

- Plant to be operating according to manufacturer’s specifications.

# Control Measures

The following control measures can be used to limit the amount of dust generated on-site and plant emissions Planning

# Assessing whether dust-generating activities should be stopped in circumstances where control measures are not controlling the problem, i.e. during periods of dry weather and high wind speeds.

# Roads & Traffic

- Suitable measures should be used to prevent dirt/mud from being tracked onto public roads.

- Use of water carts, sprinkler systems or handheld water sprays on bare areas and stockpiles. Also, can be used on trafficked areas for dust suppression.

- Restricting traffic/plant movement to specified and defined tracks/roads.

- Use of watering system on tow along road broom.

# Scheduling of Works

Scheduling work so that large sections of bare areas are not exposed at any one time.

# Covering & Screening Material & Stockpiles

- Covering trucks if dust generation from the load is a problem.

- Early rehabilitation and revegetation.

- Use of dust screens (shade cloth or similar on boundary fences).

# Operational Activities

- Maintaining machinery in accordance with manufacturer’s specifications so that exhaust emissions would comply with the State Environment Protection Policy (The Air Environment).

- Replacing old machinery when no longer operating efficiently.

- Keeping dust suppression equipment on hand for use when it is needed.

- Ensure smooth surface areas are ripped or left rough to lower velocity at soil surface.

# Burning

- No fires permitted on-site unless necessary approval has been obtained.

- Vegetation is to be mulched for re-use on-site, not burned.

# Maintenance

- Dust control measures are to be maintained.

- Soil will not be allowed to accumulate behind dust screens or other controls.

- Plant & Equipment to be serviced regularly.

- If machinery is emitting smoke continuously for longer than 10seconds, during normal operations, then that piece of equipment will be serviced or replaced.

# Monitoring & Measurement

Dust measurement is to be by observation by the site manager and by comment from affected workers and residents. Every-one on-site is capable of measuring the effectiveness of dust management techniques.