# Environmental Risks

Some activities that pose a significant risk to water quality include;

- Increased sediment load in storm water drains and waterways.

- Contribution of oil/grease from refueling/workshop/storage areas.

- Oil/chemical spills.

# Objective

To keep all waste, particularly sediment and petrochemicals, out of the drains and waterways.

# Targets

Targets should include but not only be limited to the following;

- Zero soil leaving the site on vehicles or from site drains.

- No complaints from neighborhood about muddy water in waterways.

- Road at site access is kept clean.

# Control Measures

Planning

- Assess the existing features of the land including the contour, existing vegetation, stormwater drains, proximity to waterways etc.

- Where possible program works to minimise the impact on the environment, e.g. plan to work in waterways in summer when mud and silt will not be generated.

- Define where risk activities are going to take place, ie. Entry/exit points, stockpile.

- Install soil erosion and sediment control measures prior to the commencement of works based on as assessment of existing features and location of risk activities. Re-assess site over time to assess the need to change control measures during works.

- Assess the possibility of installing cut off drains to divert clean stormwater around the construction site.

- Assess the extent to which construction will affect local groundwater quality or flow during the design phase.

# Vegetation

- Retain vegetation where possible as it minimises exposed surfaces and assists in reducing runoff.

- Stockpiles will be cleaned up and revegetated if applicable.

- Vegetation remaining on-site will be handled accordingly.

# Machinery & Fuel Storage

- On-site service of machinery should be undertaken in a controlled manner and in a designated area.

- Ensure wash down and fuel storage areas are located away from stormwater drainage lines and waterways.

# Prime & Bitumen

- Prime, bitumen needs to be controlled to prevent it from entering the stormwater system.

- Spill kits and suitable materials should be available on-site to respond to spills immediately.

- Follow guidelines in procedure EM-03 Prime & Bitumen.

# Avoiding Soil Erosion

- Minimise soil erosion by compacting and trimming all fill surfaces prior to any chance of rain.

- Locate stockpile away from drainage lines or waterways.

- Sediment control structures need to be adequate in size to cope with the amount of water anticipated.

- Regular maintenance or discarding of sediment control devices is essential.

- Site entry pits should be covered to prevent soil and litter entering waterways.

# Access to the Site

- Access on-site will be limited to designated areas to avoid moving soil around and off site.

# Rehabilitation

- Site rehabilitation will be completed to meet client requirements. Where this is not specified the site will be revegetated using local seed where possible.

- Alternative construction methods may need to be assessed when working in, adjacent to or over waterways, to minimise the adverse impact on the environment.

- During and at completion of works, build up of silt and detritus behind silt traps is to be captured and removed to a suitable waste area (if available, use of vacuum trucks will assist in this action).

# Monitoring & Measurement

- A visual inspection needs to be undertaken.

- A review of files to ensure no negative correspondence from EPA or water authorities.

- Daily checks on drains to ensure sediment is not entering waterways.

- Installation and maintenance of sediment control devices to ensure they are functional along all waterways and at drains.