Environmental Impacts Aspects Register - Appendix 1	31/08/2025 – V10.0

Environmental Risk Assessment

| Business Name | Paneltec Pty Ltd | ABN | 12128689412 |  |  |
| --- | --- | --- | --- | --- | --- |
| Address | 19 Connector Park Drive, , TAS, 7249 , KINGS MEADOWS, TAS, 7249 |  |  |  |  |
| Emergency Contact : | John Guy | Phone : | 0408 176 442 | After Hours : | John Guy |

| DETAILS |  |  |  |
| --- | --- | --- | --- |
| ID : | SRRA-1 | Title/Name : | Environmental Impacts & Aspects Register - Appendix 1 |
| Description of hazard/activity/task : | Environmental Aspects Register - Site 1 |  |  |
| Location : 1 | Paneltec Pty Ltd - 19 Connector Park Drive, Kings Meadows, Tasmania, 7249 | Project : Paneltec Depot & Office ERA |  |
| Location : 2 | Paneltec Pty Ltd – 30 Remount Road, Mowbray Tasmania, 7248 |  |  |
| Assessor Name : | Craig Large | Assessor Contact Details : | Craig Large 0458 650 338 |
| Assessment Date/Time : | 28/08/2025 09:00:00 | Status : | Approved |
| Assessment Reason : | Environmental Aspect Register |  |  |

# HAZARD IDENTIFICATION

|  | Hazard Type | Hazard | Hazard Source |
| --- | --- | --- | --- |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust from mobile plant traffic - Pit |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust carried by wind - Pit |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust from mobile plant traffic - Wash Plant |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust carried wind - Wash Plant |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust from mobile plant traffic - Loading Area |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust from cleaning machines - Workshop |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust created from transferring product - Admin/Parking |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust created from product spilling during Transport |
|  | Air Quality - Dust | Dust carrying off-site and onto adjoining properties | Dust carried by wind - Pit |
|  | Biodiversity | Death of Fauna | Natural Fauna |
|  | Biodiversity | Spread of noxious weeds to cleared areas | Noxious weeds |
|  | Energy/ Climate Change | Energy use | Electricity used for operations |
|  | Energy/ Climate Change | Energy use | Mobile equipment fuel usage |
|  | Land Contamination | Destruction of Top Soil. | Soil removed as part of Operations |
|  | Land Contamination | Land Contamination | Aggregate discharged from trucks |
|  | Land Contamination | Land/ water contamination. Hydrocarbon plume intersecting water table | Leaking underground tank |
|  | Land Contamination | Rubbish allowed to blow off site | Rubbish thrown on ground |
|  | Land Contamination | Spills | Vehicles carrying material off-site |
|  | Land Contamination | Spills onto soil | Spills associated with equipment maintenance |
|  | Land Contamination | Spills onto soil | Spills from leaking vehicles |
|  | Noise | Noise levels over EPA Levels | All site processes |
|  | Noise | Noise levels over EPA Levels | Fixed Plant- compressors, pumps and valves, air-operated controls |

|  | Noise | Noise levels over EPA Levels | Mobile Plants |
| --- | --- | --- | --- |
|  | Noise | Noise levels over EPA Levels | Wash Plant/ Work Shop/ Quarry Operations horns, other audible alarms such as Conveyor Start-up alarms |
|  | Noise | Noise levels over EPA Levels | Light vehicle traffic |
|  | Rehabilitation | Stockpile of reject material | Process Plant |
|  | Rehabilitation | Unusable land | Site End of Life |
|  | Rehabilitation | Unusable land | Topsoil loss |
|  | Waste Management | Build-up of inventory | Waste material generated from Quarrying processes |
|  | Waste Management | Build-up of inventory | Scrap metal from redundant equipment, projects, maintenance work. |
|  | Waste Management | Land contamination | Tyres removed from service |
|  | Waste Management | Land contamination | Batteries-Old equipment being replaced |
|  | Waste Management | Land contamination | Waste oil/ Filters/ Equipment Maintenance |
|  | Waste Management | Land contamination | Fuel Spills |
|  | Waste Management | Litter on ground. | Rubbish carried into pit |
|  | Waste Management | Over-use of Landfill | Paper/ Plastic/ wood |
|  | Waste Management | Resource consumption | Fuel - overuse |
|  | Water | Contamination from hydro-carbons | Pressure washing equipment |
|  | Water | Erosion | Rainwater run-off - Admin/Parking |
|  | Water | Erosion | Rainwater run-off - Pit |
|  | Water | Erosion | Rainwater run-off - Loading Area |
|  | Water | Potable/Non-potable water cross contamination | Personal use - amenities |
|  | Water | Water heavy with sediments | Washing sand |

RISK ASSESSMENT

| Hazards | Risks | Controls | Risk Assessment |
| --- | --- | --- | --- |
| Air Quality - Dust - Dust carrying off-site and onto adjoining properties | Dust blown off-site leading to complaints or fines. | ELIMINATION | 3 |
| Dust generated during normal operations. |  | OPERATIONAL |  |
|  |  | * Buildings regularly swept to collect dust. |  |
|  |  | ISOLATION |  |
|  |  | * Water Cart used to suppress dust. |  |
|  |  | * Wind Screens at Property edge to block wind. |  |
|  |  | * Product kept moist to limit dust generation. ADMINISTRATIVE |  |
|  |  | * Vehicle speed reduced to minimise dust generation. |  |
| Biodiversity - Spread of noxious weeds to cleared areas Noxious weed seeds carried off site or onto site by vehicles. | Noxious weeds can be spread by soil transfer or mobile plant being moved between sites. | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Regular site inspections to identify and destroy noxious weeds. | 3 |
| Land Contamination - Land/ water contamination. Hydrocarbon plume intersecting water table | Fuel leaks contaminating soil or waterways. | ELIMINATION OPERATIONAL | 3 |

| Leaking fuel storage tanks. |  | - Leaking tanks removed from use. SUBSTITUTION - Fuel storage tanks regularly inspected for leaks. |  |
| --- | --- | --- | --- |
| Land Contamination - Rubbish allowed to blow off site Rubbish thrown on ground. | Household rubbish causing environmental damage. | SUBSTITUTION OPERATIONAL * Rubbish bins must be provided on the worksite and regularly emptied. | 3 |
|  |  | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Workers made aware of their duty of care. |  |
|  |  | * Regular Site Inspections. |  |
| Land Contamination - Spills onto soil Spills associated with equipment maintenance and leaking vehicles. | Leaking vehicles/equipment causing soil contamination or environmental damage. | ISOLATION OPERATIONAL * Where possible maintenance and repair performed in workshops where spills can be more easily contained. | 3 |
|  |  | ADMINISTRATIVE EMERGENCY PREPAREDNESS * Emergency management plans in place for the containment and clean up of all spills. |  |
| Noise - Noise levels over EPA Levels Excessive noise levels generated during normal operations. | Excessive noise leading to complaints, fines or prosecution. | ENGINEERING MAINTENANCE * All equipment regularly maintained to minimise noise. | 3 |
|  |  | OPERATIONAL * Mobile Plants fitted with squawkers to reduce noise level. |  |
|  |  | - Visual alarms to replace audible alarms where possible. ADMINISTRATIVE ADMINISTRATIVE / TRAINING - Site Noise Assessment conducted to determine noise levels. |  |
|  |  | * All personnel trained on the importance of noise management. |  |
| Rehabilitation - Stockpile of reject material Excessive stockpile of reject materials. | Excessive material to be disposed of at site End of Life leading to fines or litigation relating to site rehabilitation. | ELIMINATION OPERATIONAL * All material not sold used to re-fill areas of site. | 3 |
| Waste Management - Build-up of inventory Excessive stockpile of material at project End of Life. | Excessive material to be disposed of at end of project. | ELIMINATION ADMINISTRATIVE / TRAINING * Develop markets for waste products. | 3 |
|  |  | OPERATIONAL * Use over-burden to rehabilitate site at end-of-life. |  |

|  |  | - Material not recyclable will be disposed of through a reputable salvage agency. ADMINISTRATIVE ADMINISTRATIVE / TRAINING - Projects include assessment of all old equipment/ material identify recycle potential. |  |
| --- | --- | --- | --- |
| Waste Management - Land contamination | Waste materials improperly disposed of leading to contamination of waterways or | ELIMINATION | 3 |
| Waste material generated from maintenance operations. | environmental damage. | OPERATIONAL |  |
|  |  | * All old tyres are removed by reputable recycling agency. |  |
|  |  | * Procedure to have batteries removed by reputable recycling agency. |  |
|  |  | * All waste oil tracked and removed by reputable contractor for recycling. ISOLATION |  |
|  |  | * Designated storage area for old batteries. |  |
| Waste Management - Litter on ground. Litter in work site | Improper rubbish disposal leading to environmental damage. | SUBSTITUTION OPERATIONAL * Rubbish bins must be provided on the worksite and regularly emptied. | 3 |
|  |  | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Training for workers to dispose of rubbish properly. |  |
| Waste Management - Over-use of Landfill Recyclable materials entering landfill. | Paper/ Plastic/ wood sent to landfill or recycling bins contaminated with non recyclable materials. | ISOLATION OPERATIONAL * Site has dedicated recycle bins. | 3 |
|  |  | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Site Procedures include identification of opportunities to recycle material. |  |
| Waste Management - Resource consumption Excessive fuel consumption causing pollution and contributing to climate change. | Use of vehicles and plant with poor fuel efficiency. | ENGINEERING MAINTENANCE * All vehicles regularly maintained to ensure good fuel efficiency. | 3 |
| Biodiversity - Death of Fauna Plant/Vehicle movement on site. | Loss of habitat and traffic movement on site may cause loss of Fauna. | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Traffic Management contains management of traffic/fauna interaction. |  |
| Energy/ Climate Change - Energy use Excessive electricity usage. | Excessive energy usage contributing to climate change. | ENGINEERING MAINTENANCE * All equipment regularly maintained to improve fuel efficiency. |  |
|  |  | ADMINISTRATIVE ADMINISTRATIVE / TRAINING |  |

|  |  | * All workers trained in energy-conservation tactics. |  |
| --- | --- | --- | --- |
| Land Contamination - Destruction of Top Soil. Soil removed as part of Operations | Destruction of contamination of topsoil. | SUBSTITUTION OPERATIONAL * Topsoil for use in site rehabilitation segregated from contaminated soil. |  |
| Land Contamination - Land Contamination Soil removed as part of Operations. | Destruction or contamination of top soil. | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * All personnel trained on the importance of hazard reporting. |  |
| Land Contamination - Spills | Risk of sand spills. | ADMINISTRATIVE |  |
| Vehicles carrying material off-site. |  | ADMINISTRATIVE / TRAINING |  |
|  |  | * Loader Operators trained to control material movement and inspect trucks being |  |
|  |  | loaded. |  |
| Rehabilitation - Unusable land | Site does not get rehabilitated at end of life resulting in degraded land use. | ELIMINATION |  |
| Unusable land at Site End of Life. |  | OPERATIONAL |  |
|  |  | * All material not sold used to re-fill areas of site. |  |
|  |  | * All Topsoil kept in stockpiles and protected for use at site End-of-Life |  |
|  |  | Rehabilitation. ADMINISTRATIVE |  |
|  |  | ADMINISTRATIVE / TRAINING |  |
|  |  | * End-of-life business plan required as part of Site Management Plan. |  |
|  |  | * Bund height limited to 2m. |  |
| Water - Contamination from hydro-carbons Contamination of environment by hydrocarbon spills. | Hydrocarbons washed off mobile plant. | ISOLATION OPERATIONAL * All water from washing collected via drains to waste water dam. |  |
|  |  | ADMINISTRATIVE CLEANING UP * Clean-up spills and leaks as per prior to washing equipment. |  |
| Water - Erosion Rainwater run-off causing erosion. | Rainwater carries contaminants into waterways. | ENGINEERING MAINTENANCE * Regular maintenance of drainage. |  |
|  |  | OPERATIONAL * Rainwater channelled into dams. |  |
|  |  | ADMINISTRATIVE ADMINISTRATIVE / TRAINING * Daily site inspections to monitor erosion. |  |

| Water - Potable/Non-potable water cross contamination Wate/recycled water used for personal amenities. | Cross contamination into other water lines servicing amenities. | ISOLATION OPERATIONAL - Re-cycle water segregated from other water sources and signed. ADMINISTRATIVE ADMINISTRATIVE / TRAINING - All water lines clearly marked as to content. |  |
| --- | --- | --- | --- |
| Water - Water heavy with sediments Water with heavy sediment. | Water with heavy sediment leaving site damaging waterways or contaminating soil. | ISOLATION OPERATIONAL * Water collected into dams to allow sediments to settle prior to use. |  |
|  |  | * Flocculation used to remove solids. |  |