Work that entails a risk to exposure to Asbestos covers a range of activities that are specified in Chapter 7, Part 7.2 of the Tasmanian WHS Regulation. This work must be assessed and identified against the WHS Regulation requirements. Compliance with this legislation is mandatory.

The primary route of asbestos entry into the body is by inhalation. The Dust Diseases Authority respiratory screening service, or a registered medical practitioner specialising in occupational asbestos exposure, provides a service to examine and document the respiratory health status of workers if there is a risk of exposure.

Managers must consult with and provide workers with relevant information

# Health screening

Baseline screening is required to establish a reference level when a risk assessment determines there is a potential for exposure to asbestos.

The need for health monitoring should be determined based on:

- Potential for exposure

- Frequency of potential exposure

- Duration of work being undertaken where the potential for exposure remains.

# Testing for exposure

WHS Regulation Clauses 435–444 and the code of practice How to Manage and Control Asbestos in the Workplace provide guidance for health monitoring for potential exposure to asbestos and include:

- Demographic, medical and occupational history

- Records of worker’s personal exposure

- Physical examination with an emphasis on the respiratory system

- Standardised respiratory function tests, unless another form of health monitoring is recommended by the registered medical practitioner.

# Engaging a testing provider

Health monitoring must be carried out under the supervision of a registered medical practitioner with the relevant competencies. Managers are required to refer workers to a registered practitioner and ensure the following processes are established with the provider.

The medical practitioner will identify the options available to test for asbestos exposure, such as:

- Availability of an onsite mobile service if a number of workers may be exposed

- Clinic facilities with capacity to do all tests required (x-ray, spirometry, etc).

The medical practitioner will be provided with details about the exposure, worker and workplace including:

- Company details, including address and contact person

- Full name, date of birth and address of worker

- Description of the work or activity leading to the exposure

- How long the worker has been exposed (potential or actual).

Reports from the medical practitioner will include:

- Advice to the worker about their specific medical results, including any personal follow-up required with their own doctor to manage their respiratory health

- Details of abnormal findings (if relevant for the work unit) with clear identification of any actions or ongoing management required.

When all actions are complete or no actions are required, the manager’s copy and any other relevant information is retained confidentially in the worker’s file.

## NOTE: Records must be kept for 40 years after the last action date.

ALL test results positive for asbestos exposure must be reported to WorkSafe Tasmania.

Managing exposure

The medical practitioner advises a worker if they recommend further actions (especially further medical treatment). A personal asbestos management plan can be developed in collaboration with the worker and their treating doctor if the report indicates abnormal findings.

As part of the close out process managers should ensure that a review of the risk control measures is conducted and that appropriate measures to eliminate or minimise exposure are in place.