# Procedure

What is Fire? - Three elements are necessary to have fire.

- Fuel comes from anything combustible, solid, liquid or gas that can burn.

- Heat can come from flames, sparks, cigarette butts, friction electrical sources, hot pipes and equipment.

- Oxygen comes mainly from the air and also from chemical reactions. Take away any of these elements and the fire will extinguish.

# Fire Prevention

To prevent fire from occurring it is important to:

- Remove rubbish oils and greases from floors and work areas.

- Comply with safety and warning signs.

- Use welding, cutting and soldering equipment correctly.

- Store flammable liquids in designated areas.

- Replace work boots which have worn and exposed steel caps which could spark.

- When open flames or sparks are generated in the course of our work, (i.e. grinding, welding, soldering or cutting operations), the work area shall be suitably protected from causing fire. The use of fire blankets, extinguishers or water hose reels may be used where applicable.

# Note: All mobile grinding, welding and or soldering sets will be accompanied by a fire extinguisher specifically for the work to be undertaken.

General Housekeeping

To assist in the control of accidents and fires, every worker is responsible for the good housekeeping and general orderliness of his/her workplace.

# Fire Fighting

- Before you can attempt to extinguish a fire you must first determine the type of fire, so that the correct extinguishing agent and method is used (refer to ‘Portable Fire Extinguisher Guide’)

- Use of an incorrect agent or method may be ineffective or dangerous eg if you put water on an electrical you may be electrocuted.

- Other factors to be considered before fighting a fire are:

  - the size of the fire;

  - accessibility;

  - the risk of escape being blocked;

  - hazards in or produced by the fire;

  - material around the fire that could be at risk from the fire (exposures);

  - possible side effect of fighting the fire and the equipment available.

- If you discover a fire, without risking your own safety, attempt to extinguish it with available fire fighting equipment.

- If you cannot extinguish the fire quickly, notify your Supervisor and:

  - evacuate any persons not directly involved in fighting the fire,

  - ensure all persons are accounted for,

  - fight the fire providing your life is not endangered,

  - always keep a clear line of retreat behind you,

  - use the appropriate extinguisher,

  - remember - you have to remove at least one of the elements of a fire: heat, oxygen or fuel,

  - never attempt to fight a fire around fuel tanks or mobile equipment tyres,

  - if it is unsafe to fight the fire or you are not sure how to use the extinguisher – evacuate,

  - always follow the fire safety guidelines set down by the site you are working at.

# Fire Equipment

  - Management will ensure that an adequate number and type of fire extinguishers are available at the workplace.

  - Paneltec Pty Ltd will ensure that all workers carrying out hot work will have a fire extinguisher within 5 metres of the work being carried out.

  - Fire fighting equipment must never be abused or removed unless to fight a fire, or for maintenance and recharging purposes.

  - If a fire extinguisher is used the incident must be reported to management.

  - Used extinguishers must be recharged regardless of how full it is.

  - It is every worker’s responsibility to know the location and how to use all extinguishers and hose reels available at the depot or the worksite.

# Mobile Site Requirements (as a minimum)

  - A dry chemical fire extinguisher will be available for use onsite where hot works are being conducted.

  - All designated work vehicles will carry a least one small dry chemical fire extinguisher.

  - When working in high risk fire areas knapsacks will be available on site.

# Depot Requirements (as a minimum)

  - A suitable number of fire extinguishers will be available in the workshop.

  - Office at least 1 dry chemical fire extinguisher, kitchen area to have a dry chemical extinguisher and a fire blanket.

# Inspection & Testing

  - A list and current service history of all fire fighting equipment will be provided refer to form SF-23 Fire Equipment Register.

  - This record will be maintained in accordance with Australian Standard AS – 1851.

# Training

  - All workers of Paneltec Pty Ltd will be fully trained in the use of fire extinguishers and which types of extinguishers are suitable for the various types of fire.

  - A record of training attendance will be recorded on the appropriate form.

# Emergency Procedures

  - All personnel will be made aware of the site specific emergency procedures and phone numbers prior to commencing work.

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| SF-23 | Fire Equipment Register |
| Skills Matrix | Excel spreadsheet |