# Procedure

This procedure outlines the management of hazardous substances and dangerous goods on the worksite in compliance with the Work Health and Safety Regulation 2022 and the Codes of Practice for Managing the Risks of Hazardous Chemicals in the Workplace and the Labelling of Workplace Hazardous Chemicals.

There are many kinds of chemical products and other substances in the workplace. Not all of these substances are hazardous according to the regulations – but some are.

Hazardous substances are chemicals and other substances that can harm people’s health. They can cause injury or illness.

Dangerous goods can affect property, the environment or people. Dangerous goods and hazardous substances are covered by separate legislation, each focusing on the different kinds of risks that apply.

Hazardous substances and dangerous goods are classified according to different reasons. Hazardous substances are classified only on the basis of health effects (whether they be immediate or long-term), while dangerous goods are classified on the basis of immediate physical or chemical effects, such as fire, explosion, corrosion and poisoning.

Many hazardous substances are also classified as dangerous goods, and the requirements of both pieces of legislation will apply in these cases.

# Safety Data Sheets (SDS)

A Safety Data Sheet (SDS) is an information sheet that is used to supply detailed information relating to chemicals and related products. The manufacturer of the products that may be used in the workplace prepares Safety Data Sheets. SDS’s are available from the manufacturer through the supplier.

The information contained on a Safety Data Sheet tells you:

- What is in the product

- Any health affect the product may cause

- First aid procedures

- Precautions to follow when using the product

- Safe handling and storage information

- Emergency procedures to be followed in case of exposure or skills

Safety Data Sheets provide vital information for the user of the product. They should be read and understood before using any substance.

The workplace will keep copies of Safety Data Sheets for all chemicals used in the workplace. SDS’s will be kept in a location that all workers can gain access to.

Refer to Safety Data Sheets – FAQ’s (frequently asked questions) in SDS folder for further information.

# Selection of chemicals and other substances

The PCBU will consider the following when selecting products to be used on site.

- Flammability and explosivity;

- Toxicity (short and long term);

- Carcinogenic classification if relevant;

- Chemical action and instability;

- Corrosive properties;

- Extent of PPE required;

- Environmental hazards;

- Storage requirements.

# Storage and Handling

- All storage and handling of hazardous substances/dangerous goods will be in accordance with the SDS and storage and handling regulations. Dangerous Goods Safety Management and Codes of Practice for Managing the Risks of Hazardous Chemicals in the Workplace and the Labelling of Workplace Hazardous Chemicals.

- All hazardous substances/dangerous goods will be stored in suitable containers with the label intact and legible at all times.

- Hazardous substances/dangerous goods of any quantity will not be stored in lunch rooms, container sheds or offices.

- Fuels for equipment will be carried in safe legal containers.

- Incompatible goods will be separated.

- A spill kit will be available at the depot for minor spills also be carried on trucks where required.

# Handling of Hazardous Substances & Dangerous Goods

- Where practicable the material with the lowest possible hazard capability that meets the technical requirements for the job will be used.

- Prior to using the hazardous substance or dangerous good all workers involved in its use will be provided with adequate information and training to allow safe completion of the required task. One form of training will be through the implementation of Safe Work Method Statements (SWMS).

- First Aid provisions on site will be suitable for Hazardous Substances and Dangerous Goods.

# Training

All workers shall be given training in the use of the various substances that they use. Care should be taken to adequately identify any health hazards to workers or members of the public. This training shall be in sufficient depth and frequency that personnel:

- Have an understanding of the hazards of the substances that they are using;

- Know safe working and handling procedures, including the correct protective equipment for the particular workplace substance and limitations of such equipment;

- Know the storage (including reactive), transport, spills and disposal requirements of the materials they are using.

# Protective Equipment

Adequate supplies of all safety equipment (including first aid materials) required for use with a workplace substance shall be readily available to workers prior to work commencing, as per relevant Australian Standards or the SDS. The basic personal protective equipment available to guard against risks from hazardous substances may include respirators, goggles, face shields, gloves, footwear, and aprons.

# Decanting

- Where a quantity of hazardous substance or dangerous good is decanted or otherwise taken from an original container, the new container must be marked in accordance with the Codes of Practice for Managing the Risks of Hazardous Chemicals in the Workplace and the Labelling of Workplace Hazardous Chemicals.

- End-use containers must not be used to indiscriminately carry different products. Such Containers must be disposed of in an approved manner (refer SDS).

- Containers used for decanting should normally be purchased as new containers. Where used containers are utilised, the container must be cleaned and completely re-labelled.

- Food containers must not be used to store decanted hazardous substances.

# Hazardous Substances and Dangerous Goods Risk Assessments

- Risk assessments can be completed using form SF-27.

# Spillage

- Spillage of any liquid or solid materials shall be promptly cleaned up. Appropriate spillage materials must be available for containment and clean-up purposes (refer SDS).

# Exposure

  - Refer to WHS-28 Health Surveillance Procedure for information on any workers exposed to Hazardous Substances or Dangerous Goods.

# Disposal

- Waste material must be disposed of appropriately. EPA waste disposal requirements may apply.

# Transport

- Transportation of hazardous substances from one area to another must conform to the requirements of the SDS.

- Transportation of dangerous goods must conform to the Dangerous Goods Transport Regulations.

# Compliance

- Compliance to this procedure by all individuals is required. Where there is non-compliance this must be investigated to ascertain the reason(s) and handled in accordance with personnel procedures.

# Records

- A register of all products SDS’s that may be used on site will be recorded in the WHSEQ Software Platform. And QR code links to those SDS’s shall be available onsite.

# Review

- Safety Data Sheets will be no older than 5 years.

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| WHS-28 | Health Surveillance Procedure |
| SF-26 | Chemical Register & Safety Data Sheet list |
| SF-27 | Hazardous Substances & Dangerous Good Risk Assessment |