# Purpose

Management has a duty of care to provide a safe workplace for all workers and the public. Reversing vehicles with restricted rear vision on the worksite are a risk to the safety of all if not controlled.

# Scope

This procedure is to be followed by all persons on worksites, including subcontractors.

# Procedure

To reduce the risk from reversing vehicles with restricted rear vision, reversing on the worksite is to be kept to a minimum and vehicles are not permitted to reverse on site unless the following controls are implemented.

- Where practical all vehicles including trucks, dogs & trailers with restricted rear vision should be fitted with a reversing camera along with flashing orange light and reversing beeper.

- A person/s will be nominated at the tailgate meeting to act as a spotter/s to guide reversing vehicles with restricted rear vision when reversing cameras are not operating or in addition to reversing cameras in specific circumstances as directed below.

- Spotters’ names will be documented on the SSRA form within Lucidity.

- Each nominated spotter will have a two-way radio to use as communication.

- If you are required to reverse on a worksite and you are alone, you must:

  - Step out of your vehicle and assess the site to ensure there are no persons or objects in your reversing path.

  - Ensure reversing beepers are working correctly.

  - Proceed with caution.

# Drivers of Reversing vehicles with restricted rear vision responsibilities

- It is responsibility of all drivers of vehicles with restricted rear vision under guidance to follow the instructions of the nominated spotter where applicable.

- If continual hand signaling or two-way radio communication ceases with the nominated spotter the reversing vehicle must stop immediately and not resume reversing until visual and or two-way radio contact has been made by the nominated spotter.

# Spotter/s

- The nominated spotter is to be visible to the reversing vehicle driver or in contact using a two-way radio.

- The nominated spotter is to have a clear line of site to the area the vehicle is reversing to.

Controlled Document	Authorised & Approved by Management	Page 1 of 1