# Procedure

The purpose of this procedure is to provide practical guidance on how to meet the requirements of Work Health and Safety Regulations, the objective of which is to protect people at work against risks to health or safety from plant and systems associated with plant.

Equipment including static (stationery) and mobile plant can be hazardous to workplace safety. In order to comply with Work Health and Safety Legislation the PCBU will carry out regular inspection and maintenance of plant and equipment. The inspection and maintenance history of each item will be documented on the appropriate form. Where relevant Australian Standard is appropriate the inspection, use and maintenance of the plant will comply as a minimum with the Standard. Where no Australian Standard is provided, the inspection, use and maintenance of the plant will comply as a minimum with the Manufacturers Recommendations. The effect of plant and equipment on the workplace will also be considered.

# Risk Assessment

Management or their representative will carry out a Safety assessment of all mobile plant using the Plant/Equipment Hazard Assessment form SF-31. The assessment will include the identification of potential hazards, level of risk and the provision of appropriate controls to eliminate or minimize the risk to health and safety of workers. This process will include both, the plant and/or equipment itself and its impact on the surrounding workplace.

When identifying potential hazards consideration will be given to all aspects of the plant and equipment including design, work environment, operational conditions, abnormal conditions, ergonomic principles, transportation, storage, installation and erection, access and egress for maintenance, adjustments, repairs, cleaning, use, operator competencies, dismantling and disposal. Risk assessments to be reviewed every 5 years/earlier if the plant has had any major changes made to it.

# Hired Plant & Equipment

Where plant and equipment is hired the same requirements for workplace health and safety as those required on site will be specified by the PCBU to the Hire Company as a condition of the Hire Agreement.

# Daily Pre-Start Inspection

A Pre-Start Safety Checklist (Plant Operator Pre-Start Checklist & Fault Report SF-32 or online form) will be completed on a daily basis or prior to use for each item of mobile plant. Any fault or maintenance requirement will be noted for action on the report by the operator and forwarded to the supervisor.

Use of Plant and Equipment

All operators of plant and equipment must be appropriately trained, competent & ticketed as required in the operation and safe use of the plant and equipment they operate. Plant and equipment must be used in accordance with the manufacturer’s instructions and other relevant information such as operating procedures and guidelines. At no time should the operation and use of plant and equipment place persons in unsafe situations.

Hand tools

‘Hand tools’ refers to `a tool used with a person’s hands’, these include:

- tools designed for force to be applied by the person, such as screwdrivers, hammers, chisels, and hacksaws, and

- tools where force is applied by a power source, but a person is still needed to hold the tool, such as portable electric or battery drills, grinders, saws etc.

# Safe Work Method Statements and Safe Work Procedures will be written for hand tools as required and available for workers to refer to.

# Maintenance

  - All plant and equipment will be maintained in accordance with manufactures specifications and required ‘Service Schedules’.

  - Testing requirements will be fulfilled as per specific requirements

  - Details of maintenance and tests performed will be recorded.

  - Are you carrying out regular inspections on your Oxygen, Acetylene and LPG hoses? All equipment should be inspected before work commences regardless of whether the equipment has been inspected before. Hoses should also undergo a thorough inspection every 3 months (refer to the Safe Work Instruction - Quarterly Hose Inspection).

# Plant and Equipment Register

    - All plant and equipment will be listed on a central register and identified by plant number or licence number, refer SF-33 Powered Mobile Plant Truck Trailer List or your SmarterScreen.

# Machine Guarding

- No item of plant will be operated without the correct guarding in place as per AS40124.1 Safeguarding of Machinery Part 1: General Principles.

No guard should be removed from any plant or equipment unless is for maintenance purposes.

- The guard must be replaced immediately maintenance has been completed and prior to the restarting of a machine.

- The lock out and tag process should be used prior to any guard being removed.

- DO NOT attempt to remove a guard while the machinery is running.

# Mobile Plant

- A certified ROPS canopy shall be fitted to all rollers, tractor brooms, skid steers and front end loaders conforming to AS2294, ISO 3471/1 along with a quick release style seat belt in each operating position.

- Reverse alarms must be fitted to all mobile equipment

- Revolving flashing amber lights (not strobe lights) must be fitted to all mobile equipment.

- Warning signs should be fitted to all mobile equipment where applicable for example,

  - “Warning Vehicle May Reverse At Any Time”

  - “Hearing Protection Must Be Worn”

  - “Seat Belts Must Be Worn”

  - “Maintain 3 Points of Contact”

# Floats/Trailers

- All floats/trailers used to transport plant and equipment should have non skid surfaces applied to the ramp sections of the unit when required.

Lifting Equipment (Lifting Chains and slings)

# Routine Inspection

Before using lifting equipment workers shall carry out a visual inspection which shall encompass the following:

- Checking for excessive wear of the item

- Checking for obvious damage or defects

- Checking of harnesses and belts for excessive wear

- Checking the operation of fall arrest locking mechanisms

- Checking the length of ropes and slings for serious cuts and abrasions

Where an item is identified as damaged or faulty it shall be removed from service and tagged with a “Do Not Operate” tag until such time as it has been inspected by a competent person and determined if it is safe to continue to be used.

When making a chain sling, all chains, hooks, links and couplers are to be of the same grade, SWL, and in a good state of repair.

All chains and slings are to have SWL and a current inspection tag.

# Plant & Equipment General Safety Information Operating Plant & Equipment

- All operators must be trained and hold appropriate certification or licence.

- Check plant daily or before use, report any damage or fault immediately.

- All warning devices must be checked and operating.

- Faulty or damaged plant must be isolated and Danger Tags used.

- Always carry out a pre-start check, when taking over from another operator.

- Always read the manufacturer’s operating instructions.

- Never leave plant running and unattended.

# Further guidelines can be found in the Safe Work Method Statements (SWMS) and Safe Work Procedures (SWP).

Lasers

Invisible direct or reflected laser beams can permanently damage unprotected eyes.

Lasers and their use should comply with AS 2397 —1993 Safe use of lasers in the building and construction industry.

# Laser Safety:

- Do not stare into any visible laser beam: No matter what the specifications are!

- Do not point any visible laser beam at another person or work colleague.

Hot Work

- On some client sites a Hot Work Permit (SF-36) is required for any work, in a restricted area, where naked flames, sparks or heat may be generated in the area. Hot work includes welding, power cutting, power drilling, concrete cutting or chipping, using electric hand tools, internal combustion engines etc.

- A hot work permit is only valid for the period stated on the permit but may be re-endorsed by the permit writer. The hot work permit is not valid until it has been signed by the permit writer and the permit recipient.

- Workers carrying out works on those client sites where a hot work permit is required shall follow the rules of that site.

- Suitable fire extinguishers must be available in the immediate hot work area.

- Combustible materials and flammable liquids must be removed from the work area.

# Colour Coding

- Australian Standard 1318 - 1985 recommends specific colours for the marking of physical hazards and the identification of certain equipment in industry. Colours used are red, yellow, green and blue.

- RED AND WHITE: Signifies danger and prohibition, fire protection equipment, stop buttons for electrical switches and emergency stop controls for machinery.

- YELLOW AND BLACK: Signifies caution: draws attention to such hazards as unfastened or removed machinery guards.

- GREEN AND WHITE: Denotes safety and location of safety or first aid equipment.

- BLUE AND WHITE: Is for information signs where there is no specific hazard.

# Information, Instruction and Training

Workers will be given information, instruction and training in the correct method of operation of plant and equipment as required.

No person will operate a piece of plant or equipment unless they have been trained, they are competent and have the required licences to operate that piece of plant or equipment.

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| SF-31 | Plant/Equipment Hazard Assessment form |
| SF-32 | Plant Operator Pre-Start Checklist & Fault Report |
| SF-33 | Powered Mobile Plant Truck Trailer List |
| SF-36 | Hot Work Permit |