# General Guidelines

The Paneltec Group (Paneltec) works upon above ground and underground assets that can potentially contain asbestos containing materials (ACM).

This procedure provides instruction for employees repairing or removing asbestos containing materials under the Paneltec’s Integrated management system (IMS).

People with management and control of a workplace as well as employers and self-employed persons have 
duties to control exposure to airborne asbestos fibres in the workplace.  Exposure to airborne particle exposure 
is to be reduced so far as is reasonably practicable to an acceptable level.

This procedure reinforces the requirements Safe Work Australia’s Removing Asbestos in Workplaces and Managing Asbestos in Workplaces, Code of Practice (2020).

# Consultation

You must consult your workers who are involved in carrying out work in or near asbestos removal during the process of identifying hazards, assessing risks, and implementing control measures.

Definitions & Abbreviations

ACM – Asbestos Containing Material

AC – Asbestos cement

AMIS – Asset Management Information System

Class B Specific Non-Friable Asbestos Removalist – people licensed to remove a specific type(s) of

non-friable asbestos containing materials

Friable Asbestos – asbestos containing materials that when dry may be crumbled, pulverised or

reduced to powder by hand pressure or as a result of work process becomes such that it may be

GIS – Geographical Information System

Immediate and adjacent area – an area near where asbestos removal will take place or an area

adjoining or directly facing the removal area

SSRA – Site specific risk assessment

Non-Friable Asbestos – asbestos materials that have been compounded from asbestos mixed with

cement or other hard bonding materials and are not able to be reduced to dust through the application of

Tapping – a sealed process for ‘screwing’ a ferrule into an existing main while still under pressure

Procedural Steps

Asbestos removal work that requires a license Nonfriable asbestos

Asbestos removal work involving nonfriable ACM must be performed by a Class A or B

Asbestos Removal licensed person if:

•  The area of ACM to be removed exceeds 10 square metres in total, or

•  The total time over which asbestos removal work is performed in any period of seven days

exceeds one hour (this is the cumulative total time the asbestos removal work is carried out)

License to operate as an asbestos removalist

WorkSafe Tasmania has issued Paneltec a ‘license to operate as an asbestos removalist’,

under the Work Health & Safety Act 2012.

- This license type is a Class B – Non-Friable Asbestos

- The license lists Paneltec’s nominated site supervisor and conditions.

Requirements for Asbestos Cement Pipe Repairs and Removal

Training Operators and Supervisors

All employees operating under Paneltec’s Class B license or those that are

at risk of exposure must have completed a WorkSafe Approved Class B asbestos training course.

The training shall cover:

- Health effects

- Dangers of smoking

- Decontamination process

- Disposal Procedures

- Methods of removal

- Controls

- Legislation requirements of personal protective equipment

No employee shall be permitted to remove or repair AC Pipes unless they have completed

the appropriate training and have been duly authorised to conduct this type of work.

Respiratory protective equipment

All persons engaged in asbestos removal work must wear respiratory protection equipment

(RPE) conforming to the requirements of AS/NZS 1716:2006

Non-disposable respirators need to be maintained in a clean and good working condition by

the person responsible for their safe working condition (the removalist).  All parts including

filters, valves and seals, need to be inspected before and after each use.

Respirator replacement. (DO NOT PROCEED WITH ASBESTOS REMOVAL ACTIVITIES IF YOU

IDENTIFY A DEFECT WITH YOUR RESPIRATOR)

The respirator must be worn in accordance with the manufacturer’s instructions and the

coverall hood must go over the respirator straps.

Disposable P2 respirators for asbestos can be used for removal work.  If used, they must be disposed of 
as asbestos waste after a single use.

Non-disposable respirators must be cleaned and stored in an appropriate container in a safe place away from contamination.

IMMEDIATELY PRIOR TO COMMENCEMENT OF WORK AND ON EVERY USE, a

respirator fit check in accordance with training and the manufactures instructions must be undertaken.

For effective protection via negative-pressure respirators, asbestos removal employees

must be clean shaven to achieve a face seal.  Employees with beards, stubble or facial hair should shave first.

Supervisors

Supervisors have been nominated and trained to oversee AC pipe repair/removal work and

provide additional guidance as required. To act as a supervisor for asbestos removal works listed on

Paneltec’s license by the WorkSafe licensing branch.

The nominated supervisor for a particular job is to be accessible by phone, and able to

attend the repair/removal site within 60 minutes if required.

Medicals

Medicals shall be provided to all employees operating under Paneltec’s Asbestos

Removal license for Class B – Non-Friable Asbestos

- Before the employee commences asbestos removal work for the first time

- At intervals of not more than two years

- The employee has had a medical examination within the preceding year.

Results of employee medical examinations must be retained by The Paneltec Group for 30

years.

The medical examination should be performed in accordance with the Guidelines for Health

Surveillance [NOHSC:7039(1995)]

Notification to WorkSafe

Notification requirements under The Paneltec Group licence are as follows:

- Removal work in excess of 10 m   - notify WorkSafe at least 5 days prior to work

commencing (10 m translates to about 17.9 lineal metres of 150 mm pipe, less for

- Removal work less than 10 m - notify at least 24 hours before work commences.

- Tappings – No notification required

NOTE: TasWater has a specific exemption regarding notification for unexpected breakdown of essential water services that requires immediate removal.

Notification forms may be downloaded from the WorkSafe website or obtained from Safety Personnel.

Barricades and signage

Ensure adequate barricades and signage to protect the public from hazards associated with excavation work and asbestos removal.

- Signs are to be placed in prominent locations.

  - The type of ACM assessment needs to take account of:

  - around the asbestos removal area (eg other workers, visitors, neighbours, the public) – ie other people’s exposure

  - ACM removal (ie walls, doors, fences)

  - the quantity of ACM to be removed

  - the type of barrier used (eg hoarding or tape)

Reliance on signs and barricades alone to deny unauthorised access to the removal area is

not sufficient.  Staff should maintain a heightened awareness of those outside the removal area.

All barriers and warning signs must remain in place until a clearance to re-occupy the site

has been granted by the removalist.

Informing people in immediate and adjacent area

In line with the changed requirement to inform persons occupying premises, including

domestic premises, in immediate and adjacent areas, the necessary details will be available

(‘postcard’) and it will be left for the occupier of the premises immediately facing the works

and the premises either side of the removal works.

The same postcard notification will be used for planned and reactive works.

The postcard will direct occupiers and those in immediate and adjacent areas to a list of

Frequently Asked Questions. See TasWater’s community engagement officer.

Site Specific Risk Assessment & Safe Work Method Statement

The asbestos procedure does not take away from the requirement to perform a signed SSRA and have a copy of our SWMS-63 prior to the commencement of works.

The SSRA must consider emergency planning.  Eg: In the event of an emergency and where it is not reasonably practicable to decontaminate, as fire or serious injury or sick personnel’.  As soon as reasonably practicable there after decontamination is to occur.

Pipe Location & Uncovering

Determine depth of pipe from drawings, probe, vacuum pothole or manual excavation. Using a combination

of mechanical and manual excavation, carefully uncover section of pipe to be removed, DO NOT HIGH-PRESSURE WATER JET AGAINST THE PIPE.

Removal Preparation

- Put on PPE including Disposable Coveralls, Class P2 Respirator or half mask is the work procedure might generate fibres, or create asbestos dust or pipe fragments.

- No employee shall commence any such work unless they have achieved a satisfactory fit test with the respirator positioned on the face.

- Prior to commencing removal, staff shall ensure that there is an adequate supply of asbestos waste bags and/or plastic for the disposal of contaminated PPE and asbestos waste within the barricaded work area.

- Prior to the commencement of cutting operations, staff must ensure that there is an adequate supply of clean water on hand for the entire operation.

Pipe cutting and removal

- Pipe cutting shall be performed using hand tools, wheel cutters, or compression cutters.

- Make the cuts with a low-speed hand tool such as a hand saw. Abrasive disc power saws

must not be used.

- Clean water must be run over the area of the pipe to be cut.  This can be achieved by using

a water saturated cloth above the saw blade and squeezing water over the blade and pipe

- Removalist shall ensure the cloth is regularly re-saturated to achieve dust suppression.  The

removalists must ensure that selection of the appropriate dust suppression and elimination

type of ACM and pipe size.)

- Remove the section of pipe to the side of the trench. Perform a visual inspection to and

remove any visible fragments from the trench and place in an asbestos waste bag.

- Where reasonably practicable, the section to be removed should go back to the pipe collar

or joint i.e. remove whole lengths of pipe.

- Where not practicable to remove back to the collar or joint, employees should ensure the

length removed extends past the pipe defect, and employees are to avoid standing on the

pipe to prevent damage to the remaining in-situ pipework and fittings.

Plugging of “decommissioned pipe in place”

When removing asbestos pipe under this procedure for the purposes of placing a pipe into “decommissioned in 
place” status, The Paneltec Group requires the decommissioned pipe to be plugged with sand for 500mm down the remaining pipe from the cut annulus.

If the section of pipe is for TasWater than the supervisor or The Paneltec Group responsible officer is to

approve the plugged length. TasWater may require a grout compound.

Disposal & Decontamination

- Cut a piece of 0.2mm heavy-duty plastic wrapping to a slighter longer length than the piece of pipe. 
Alternatively, a purpose-made asbestos waste plastic bag can also be used.

- Ensure rolls of duct tape are at the ready to seal the plastic or bags.

- Wrap the pipe, fold the overlapping plastic over the pipe at both ends and secure with tape.

- Place wrapped pipe in an asbestos labelled plastic bag and tie or tape the ends. Note: all waste must be effectively “doubled wrapped”.

- Non-disposable protective clothing e.g. gumboots and waterproof gloves should be rinsed clean, preferably 
with the person still in the trench to prevent run-off.

- All used disposable clothing, respirators filters, cartridges, gloves, used saws etc. shall be treated as

asbestos waste and disposed of into asbestos labelled waste bag. Respirator filters shall be used once

only and then disposed of.

- Employees shall ensure that they carry out personal decontamination each time they leave

the asbestos work area (ie. ensure personal cleaning methods are used such as a wet wipes and hand 
sanitizer.

- Personal decontamination needs to be performed within the asbestos work area where re- contamination cannot occur.

Employees who undertake work which requires strenuous effort, involves possible exposure to infectious 
agents or other contaminates or leaves them dirty or smelly, need to removal aspect of the work may not require personal decontamination to include:

- decontamination units (decontamination units are required for friable asbestos removal),

- access to showers before leaving work may still be required.

Personal washing and hygiene are essential, with particular attention to be paid to the hands, fingernails, face and head).

Transportation and Disposal

The double wrapped asbestos waste shall be securely and safely loaded (consider manual handling aspects) onto an EPA approved Paneltec Group vehicle and transported from the worksite to either dedicated, secure and locked skip bin or container:

- TasWater Rocherlea – TasWater Infrastructure

- Yellow Depot Asbestos Skip – Overnight Storage Only

The material must go directly from the worksite to either of the Asbestos Waste Bin Depots listed above.

# For further information refer to Safe Work Australia – Code of Practice – How to safely remove Asbestos – 2020

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| SWMS-63 | Asbestos Pipe Removal |
| Asbestos Register | Excel spreadsheet |