# Lockout & Tagging

- The basic principle for an effective isolation procedure is to Positively Isolate the equipment or machinery by locking all energy sources, so the equipment can’t possibly be started, engaged, or moved while anyone is in a position which would expose them to injury. The key elements in this are “locking” and “ALL energy sources”.

- If you don’t have some way of locking the equipment out with a lock and key, or in the case of mobile equipment, keeping the key in your pocket, there is a possibility someone else can start it while you are at risk. All energy sources must be isolated. Other energy sources include petrol, gas and diesel engines, hydraulics, compressed air systems, the effects of gravity and stored energy,  in  a  fly  wheel  for  example. All energy sources must be positively isolated to protect you when you are working on equipment.

- The other major principle of isolation is that the first person who isolates a piece of equipment ensures they have achieved positive isolation by testing the isolation. Isolation switches fail regularly for a variety of reasons, such as contacts welding together, broken switch handles, misaligned switch handles due to ill fitting doors, faulty valves etc. These failures can and do cause serious injuries and fatalities, so  the  added precaution of testing the isolation is essential.

- Remember, never remove a machine guard or commence work on a piece of equipment where you are personally exposed to risk, without first placing your own individual lock and Personal Danger Tag. There are never any circumstances where your safety and good health is worth being placed at risk by taking a short cut.

- If the equipment you are working on isn’t safe to use when you leave the job, always remove your Personal Danger Tag and replace it with an Out of Service Tag, to protect other people from injury.

# Danger Tags

- Danger tags are used for a variety of hazards that may be involved in equipment and machinery maintenance, such as moving parts, electricity, steam, gas, liquid or toxic substances.

- If any work is to be carried out on a particular piece of machinery or equipment and there is a possibility of somebody inadvertently turning it on, the person carrying out the work must personally fasten a red and black "DANGER" tag or lock to the main isolating switch or valve.

- As with locking out, if more than one person is undertaking the work, each person must fit their own personal lock or tag to the machine. Each tag must be printed with the person's name and their section, and give the date and time it was attached to the switch or valve. No one must operate a switch or valve that has been locked or tagged "DANGER" until the lock or tag has been removed, and only the person who attached the lock or tag is allowed to remove it.

# Out of Service Tags

- Yellow and black "OUT OF SERVICE" tags are to be used to prevent accidents or damage to machinery that is out of service for repairs or alterations. These tags are used in the same way as "DANGER" tags, except that only a supervisor can remove an "OUT OF SERVICE" tag. If possible, both tags AND locks shall be used. This will provide both visual and physical protection. As a further precaution, a portable sign can also be placed in front of the equipment or machine, or it can be cordoned off with a barrier or rope.

Examples of Tags