# Procedure

Where traffic management is not under the control of an external Traffic Control Company, temporary traffic management shall be set up by qualified & competent workers.

Traffic control at work sites is provided to maintain a safe workplace for workers and to safely guide road users through work sites.  It is essential for safety that the credibility of traffic control at work sites is maintained.  This can be achieved by ensuring that arrangements are as simple and predictable as possible, that devices are correctly installed, and that the measures applied match the road environment and work activities being undertaken.

# Traffic control at work sites in Tasmania must be installed in accordance with the relevant Australian Standard

– AS1742.3, Manual of uniform traffic control devices, Part 3: Traffic control for works on roads. Staff involved in installing and managing traffic control at work sites must understand the requirements of the Standard and be appropriately trained and qualified in its use.

(Referenced from Department of State Growth-Transport Traffic Control for Work on Roads-2023 guidance notes)

# What is a temporary traffic management?

- Temporary traffic management (TTM) is a system of controlling traffic movement through or past a worksite to achieve a maximum of safety and a minimum of inconvenience for both the road worker and the road user.

- A TTM system is required when working on public roads, road verges, road medians, footpaths, bicycle paths, construction sites and where any other work related activities require temporary controlling of traffic.

- This means preventing injury to workers due to hazards within the worksite, protection of workers from oncoming or passing traffic and the protection of road users and pedestrians from hazard within the worksite.

- Implementing a temporary traffic management system, which is designed to protect workers and road users, is a method of meeting your duty of care under Workplace health and safety laws.

- Traffic shall be controlled in accordance with the following depending on the contract or site conditions and requirements:

  - Department of State Growth-Transport (DSCT) Traffic Control for Work on Roads-2023 and Australian Standard AS 1742.3: Manual of uniform traffic control devices - Part 3: Traffic control devices for works on roads.

  - SAA HB81.1 to HB 81.6 Field guides for traffic control at works on roads.

Part 1 to Part 6 (cover various examples of work on different roads and under different conditions).

This procedure assumes works shall be carried out entirely during daylight hours, and whenever possible, traffic management signs and apparatus shall not be left out overnight unless the work is unfinished.

The need for traffic management during the night is a particular Hazard and will be considered individually as the situation arises and the appropriate.

Management of Paneltec Pty Ltd is responsible for the provision of traffic management plans, temporary signs and their location along with checking them throughout the day and documenting checks.

TRAINING (referenced from Department of State Growth-Transport Traffic Control for Work on Roads-2023 guidance notes) Those involved in providing traffic control at worksites, both in operational and managerial roles, must be competent. This means they must be appropriately trained, hold relevant qualifications, and have relevant field experience for the tasks they undertake.

- Personnel undertaking traffic management activities in the road reserve must have satisfactorily completed the Australian Qualifications Framework Resources and Infrastructure Industry training package unit RIIOHS302D – ‘Implement Traffic Management Plan’ or equivalent.

- In addition to the above qualification, where manual traffic control is required it shall be performed by those who have also satisfactorily completed the Australian Qualifications Framework Resources and Infrastructure Industry training package unit RIIOHS205D – ‘Control Traffic with a Stop/Slow Bat’ or equivalent.

As a minimum, traffic management plans must be certified by a person who has satisfactorily completed the Australian Qualifications Framework Resources and Infrastructure Industry training package unit RIICWD503D – ‘Prepare Work Zone Traffic Management Plan’ or equivalent. Satisfactory completion of the training

- specified at 1. (Implement Traffic Management Plan) is a pre-requisite.

- For works involving more complex traffic arrangements, or staging, or both, traffic management plans shall be prepared by a person who has satisfactorily completed the Australian Qualifications Framework Resources and Infrastructure Industry training package unit RIICWD503A – ‘Prepare Work Zone Traffic Management Plan’ or equivalent and the Australian Qualifications Framework Resources and Infrastructure Industry training package unit RIIRIS301A – ‘Apply Risk Management Processes’ or equivalent. Satisfactory completion of the training specified at 3. (Prepare Work Zone Traffic Management Plan) and at least 12 months experience in the previous 3 years in selecting/modifying traffic control plans are pre-requisites.

Personnel are required to complete refresher training at three year intervals to keep up with ongoing changes to standards and legislation. Employers are required to ensure that employees’ qualifications are sufficient and current.

Persons must carry traffic management tickets, on their person always whilst conducting traffic management.

# MANAGEMENT OF TRAFFIC

The management of traffic is an essential requirement of any work undertaken on roads. The objectives of traffic management are to;

- Provide a safe environment for all persons working on, and traffic traveling along, roads.

- Minimise the impact of the works on traffic (and where possible, give priority to public transport conveyances) and adjacent landowners/occupiers.

- Cater for the needs of all road users; and

- Communicate the arrangements for, and impacts of, any activities affecting traffic.

# TRAFFIC MANAGEMENT PLANS

The traffic management plans shown in the Australian Standard AS 1742.3: Manual of uniform traffic control devices - Part 3: Traffic control devices for works on roads. cover a variety of situations and can be modified to accommodate other circumstances.

The Standards Australia Field Guides also provide a large number of examples of other plans associated with a range of works on roads.

Published field guides for traffic control at works on roads include the 10 parts of the ‘Guide to Temporary Traffic Management’ which details contemporary temporary traffic management practice for application in Australia.

- Part 1: Introduction (PDF)

- Part 2: Traffic Management Planning (PDF)

- Part 3: Static Worksites (PDF)

- Part 4: Mobile Works (PDF)

- Part 5: Short Term Low Impact Worksites (PDF)

- Part 6: Field Staff – Implementation and Operation (PDF)

- Part 7: Traffic Controllers (PDF)

- Part 8: Processes and Procedures (PDF)

- Part 9: Sample Layouts (PDF)

- Part 10: Supporting Guidance (PDF)

If none of these plans cover the circumstances of the particular works being contemplated, then it may be necessary to have the traffic management plan prepared by a person who is suitably experienced and competent in traffic management.

Where a standard diagram as contained in the Standards Australia Field Guides is used as a traffic management plan, or forms part of a traffic management plan, the standard diagram should clearly identify any relevant variations necessary to relate the standard diagram to the specific conditions of the worksite.

In preparing a traffic management plan, especially for those works having a high site risk rating, consideration should be given to the following matters where relevant (in addition to any prescribed requirements).

- Overall strategy for the management of traffic.

- Arrangement of traffic control signs and devices for each stage of the works.

- Emergency access – for both workers and any emergency services vehicles travelling through the worksite.

- Any unusual hazards or job specific requirements eg. nearby school or access to shops.

- Use of alternative routes as required.

- Provision for over-dimensional vehicles.

- Provision of safe passage for pedestrians, cyclists and people with disabilities.

- Provision for, and impact on, public transport (eg. delay to buses/trams, potential for traffic to queue across an adjacent railway crossing), including where possible, priority for public transport conveyances.

  - Provision for access to abutting properties.

  - Duration and times for conducting the works (eg. Day or night operation).

  - Emergency contact details.

  - Communication arrangements.

A traffic management plan must be prepared by a person who is suitably qualified, experienced and competent in traffic management. Having regard to the nature and complexity of the works, and the type of traffic management plan required. Consultation with workers who have experience in working on trafficked sites can be beneficial in ensuring that a practicable traffic management plan is prepared.

Where a traffic management plan proposes the use of certain traffic control devices as part of the subject works that require a written authorisation or consent, a copy of the traffic management plan should be included with the authorisation or consent application for the use of those traffic control devices (refer specific council or road authority procedures).

# Traffic Management Plan Checklist

The checklist SF-42 can be used to undertake the risk assessment of the works to be carried out (on either the road or roadside). The checklist should be used to record the key decisions made when determining the traffic management plan to be used for the works.

The completed form should be filed for record purposes.

# Traffic Management - Daily Routine-Inspections-Audits General

Supervisory personnel should establish a daily routine which allots specific tasks to personnel, including supervisors, so that;

- loss of production time is minimized;

- plant operations are not disrupted;

- signing at all times is adequate for the safety of personnel and traffic; and

- the surface of the travelled path is maintained in a satisfactory condition. The supervisor’s role in this routine procedure is coordination, inspection and correction.

# Setting Out Roadwork Signage for Traffic Control

Roadwork signage is critical to the safety of work site personal and the travelling public.

- Great care must be taken when setting up traffic control as personnel are working unprotected, except for the sign vehicle with flashing lights on.

- A traffic control plan must be used at all times.

- Two operators are required to set up traffic control the following rules must be observed.

- Do not attempt to cross the road without looking both ways first.

- Communicate with each other and always warn of approaching traffic.

- Always walk, do not run.

- Traffic lights should be set up on Flash prior to erection of signs.

- Signs must be set up level and fitted with two orange flags.

- Signs must be clean and easily read.

- No personnel or machinery are to commence work until the person in charge of Traffic Control has given the all-clear and the lights are turned to red.

# Before work starts

The following routine should be undertaken before work starts each day:

- An inspection of all traffic signs and devices should be made and a note made of signs out of place or damaged during the night for subsequent rectification.

- All lamps should be switched off and checked and cleaned, if necessary.

- An inspection should be made of all water ballasted safety barrier or containment fence modules and a note taken of any out of position modules, low water levels and damaged modules for subsequent rectification.

- After adjustments have been made to the traffic management provisions for the day

  - they should be checked for safety and effectiveness by an inspection drive through

  - the job, and a record made of the signs erected and their locations.

# During hours of work

The following routine should be followed while work is in progress:

- Periodically drive through the work site to check that all signs, markings and delineating devices as seen by other road users are satisfactory and in their correct position. Using a digital camera to take photos is an excellent way of recording the set out of signs for a particular day.

- Complete the ‘Roadside Worksite Traffic Management Checklist’

- Attend to minor problems as they occur.

- During work breaks, e.g. tea breaks. Move personnel clear of the work area; park mobile plant/vehicles clear of traffic lanes. Remove from view or cover inappropriate signs such as ‘PREPARE TO STOP’ or ‘Workers Ahead’ if workers leave the site.

- Where there are traffic hazards or where only one lane is open to traffic, instruct traffic controllers to remain on the job and relieve them as necessary.

- Reposition barriers, signs and tapers as necessary, e.g. adjust the length of single lane traffic operation as necessary to keep it to a minimum and keep records of changes made and the time these occurred.

- Coordinate maintenance of the travelled path with other job operations.

# Closing down at the end of the day

These could include traffic controllers working in shifts for the full period of the obstruction, illumination of the site or the installation of portable or temporary traffic signals.

In general, the following action is required at the end of a day’s work:

- Upon completion of works, all personnel must be clear of the road work site, lights are to be turned to Flash and signage can then be dismantled.

- Carry out a pre-closedown inspection allowing time for urgent maintenance to the travelled path.

- Remove PREPARE TO STOP, Workers Ahead and other inappropriate signs.

- In cases where barriers are to remain, affix and light lamps.

- Drive through the work site to confirm that signs and devices are in position and operating before leaving the site.

- Using a digital camera to record the position of signs and devices prior to leaving the site is advisable.

- Finally, record any changes that have been made to the previously recorded sign arrangement or traffic guidance scheme.

# After hours

During the hours when work is suspended:

- Make arrangements for personnel to check lamps after dark and to maintain the lamp system during weekends and holidays.

- Provide after hours contact so that arrangements can be made to replace damaged signs, delineators or barriers.

- Ensure that a record is kept of signs found damaged, missing or out of place (and their location) at night, weekend or holiday inspections.

- Carry out periodic after-dark inspections on low headlight beam to ensure that all devices are visible and performing their correct function.

# Night Works

Compared to works during day time, night-works are significantly more hazardous.

Where Traffic Controllers are required to work at night, special equipment needs to be used. A safety vest suitable for nigh-time use.

  - Reflective night use hand-held “STOP/SLOW ” sign

  - Reflective cones

  - Night wands

  - Barrier boards

  - Night signs

  - Ripple/flashing lights

  - Two-way radio

At all times Traffic Controllers must be well illuminated by portable floodlights, street lighting, car headlights etc. The light source should be shielded to minimise glare to approaching traffic.

# Inspections and Audits of Traffic Management Plans

Audits of any traffic management plans to ensure worksite safety, should be undertaken by the works manager using the Roadside Worksite Traffic Management Checklist SF-43.

The audits are undertaken to verify compliance with a traffic management plan when in operation.

- When to Audit – audits should be completed at the following stages of the works:

  - At the start of the works.

  - At each major change to the traffic management plan.

  - During both day and night operation for long term works.

  - Whenever the operation of a traffic management plan results in unexpected significant disruptions to traffic.

- Items to Consider – when auditing compliance with a traffic management plan, and having regard to the nature and complexity of the works and associated safety risks, consideration should be given to the following:

  - Safety of workers on the worksite, road users and the public.

  - Signs, road markings, temporary safety barriers, lighting, and facilities for pedestrians and cyclists.

  - Traffic compliance with the implemented traffic management plan.

  - Access to abutting properties.

  - Effect of the works on surrounding land use (eg. residential, commercial/industrial, car parking).

  - Differences in weather conditions.

# Traffic Management - Records Management

Supervisory personnel engaged on works which require the use of a traffic management should keep the following records:

- Daily records of the sign arrangement or traffic guidance scheme should be kept in a diary or in work sheets (eg SF-44 Traffic Management Road Signage Check Sheet)

- This record should include details of the traffic control signs and devices arrangement, the use of traffic control devices (eg. When and where speed limit signs are displayed), and any changes to the traffic management plan made during the conduct of the works.

- Special attention should be given to recording the installation, alteration and removal of all regulatory signs and devices, including speed restriction signs.

- For short-term works, reference to the figure number(s) in the relevant field guide which generally applies to the layout used or to a documented procedure will usually be sufficient.

- The records should include the hours of operation and the surface condition of the road. Any significant departures from, or additions to, the signs and devices included in the relevant figure(s) should also be noted.

- In the case of accidents, either witnessed or reported, involving the public or from which legal proceedings might arise, the actual type, size and location of signs and devices in use at the time of the accident should be recorded and the sign arrangement photographed for subsequent reporting. Details of the actual width and condition of the travelled path and weather conditions should also be recorded.

- An Incident/Hazard report needs to be completed for all road accidents, near misses or hazardous situations.

NOTE: This information may be critical should legal proceedings result from an accident. In such cases, the organization is often requested to provide details of signs and devices erected at works carried out in the distant past and which cannot be accurately answered unless detailed records are kept.

Taking photos of the traffic management set out is an excellent way to record the traffic control system used on any particular day along with written records.

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| SF-42 | Traffic Management Plan Checklist |
| SF-43 | Roadside Worksite Traffic Management Checklist |
| SF-44 | Traffic Management Road Signage Check Sheet |