# Procedure

This procedure outlines the requirements of the Noise Regulations along with management and its worker’s responsibilities under these regulations.

# Regulation

The Work Health and Safety Regulations in relation to the control of noise exposure require that;

- The exposure of workers to noise in the workplace is controlled so as to reduce the incidence and severity of hearing loss resulting from excessive exposure to noise;

- There is consultation within the workplace in relation to the control of workers’ exposure to noise.

# Provisions of the Regulations are:

- The acceptable level of noise exposure is less than the 8 hour equivalent sound level of 85 dB(A) and a peak sound level of 140 dB(A).

- Assessment of noise exposure must not take into account hearing protection devices worn by workers;

# Noise over time

- Currently 85 dB(A) is set as the upper limit 8 hour dosage. Noise dosage doubles with every 3 dB(A) increase in the sound level. Therefore the time exposure to noise must be halved for every 3 dB(A) increase, that is if 85 dB(A) for 8 hours is OK then:

- at 88 dB(A), exposure should only be 4 hours;

- at 91 dB(A), exposure should only be 2 hours;

- at 94 dB(A), exposure should only be 1 hour;

- at 97 dB(A), exposure should only be 30 minutes and

- at 100 dB(A), exposure should only be 15 minutes.

(every 10 dB(A) increase is equivalent to 10 times the noise level)

# Exposure

- Refer to WHS-28 Health Surveillance Procedure for further details on exposure to Hazardous Noise.

# Management Duties

The principal duty on Management is to ensure worker’s exposure to noise does not exceed the exposure standard. Other duties are as follows;

- Consult with workers where practicable when conducting noise assessment and risk control activities.

- Assess worker exposure to noise where exposure may exceed exposure standard.

- Review and where necessary revise a noise assessment if circumstances change, or at least every 5 years.

- Employers must ensure workers’ exposure to noise does not exceed the exposure standard by implementing control measures using the Hierarchy of Control.

  - Eliminate the source of the noise

  - Substitute quieter plant or use engineering controls

  - Use administrative controls

  - Provide hearing protectors (signs must be fitted where hearing protection is required)

- Documented noise test assessments and control measures and to be implemented.

- Provide information, instruction and training to workers in relation to the effects of noise, the control of noise, the use of hearing protectors and the purpose of audiometric tests.

- Provide and pay for hearing tests for workers if they are required to wear hearing protection every 2 years. Audiometric test records are to be kept indefinitely. These records are subject to confidentiality. In the event of the business closing down, these records will be sent to the Worksafe Tasmania.

- Ensure copies of audiometric tests are kept confidential and are provided to relevant workers.



# Duties of Workers

- If a worker has been given information, instruction and training in how to use noise control measures (for example, wearing hearing protectors) he or she must use the measures in accordance with the information, instruction and training provided.

- Workers are required to wear hearing protection at all times where equipment or workshops are signed.

# Documentation

- Noise Test Assessments of plant/equipment and work processes shall be completed, documented and filed.

- Refer to form SF-29 Noise Test Register.

| PROCEDURE RECORDS |  |
| --- | --- |
| Number | Detail |
| WHS-28 | Health Surveillance Procedure |
| SF-29 | Noise Test Register |