| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | FRONT END LOADER - SAFE OPERATION | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025. SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Training | To prevent injuries or illness to workers | 1 | - Machinery operators will have the necessary current licence or competency certificate for the equipment they will operate. - All persons will have a current construction induction card. | Supervisor All | 2 |
| Arriving on site | Entry unsafe or unauthorised areas | 1 | - Report to site office for site induction if required. - Risk assessment of site conditions and Site specific induction & toolbox meeting to be conducted. - Consider all environmental protection issues and the impact caused by entry to site. | All Supervisor | 2 |
| Unloading Plant & equipment | Falls, slips & trips from vehicle, trucks and floats Machinery plant tipping over or falling from ramps | 1 | - Ensure all vehicles are parked in a clear level area for unloading plant and equipment. - Extra care taken in wet conditions, use a ramp to load and unload plant. - Persons unloading equipment to wear seat belt. - Operators familiar with SWMS- Loading, Transporting and Unloading Mobile Plant-Equipment. - Only suitably trained personnel to work with plant | Operator | 2 |
|  | Traffic hazards, hit by vehicles or other on site plant and equipment | 1 | - Ensure hazard & warning signage is in place as required when unloading plant. - Training in the use of PPE equipment such as fluorescent safety vests/clothing. | Operator Supervisor | 2 |
|  | Hand, foot & hearing damage, crushing of limbs | 2 | 1. Training in manual handling & the correct protection, gloves and safety footwear. | Supervisor | 3 |
| Personal Protective Clothing & Equipment (PPE) | Personal injury. | 1 | 1. Minimum PPE to be worn on site is: - Hi visibility vests/clothing - Steel toe-capped boots - UV skin protection minimum 15+ UV sunscreen - Head protection as applicable eg sun/hard hat - Long pants and long sleeves | Operator | 2 |
|  |  |  | 2. Seat belt to be worn at all times. |  |  |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Pre Start Check | Unsafe working conditions. Potential for accident | 1 | - Pre start checklist forms filled out & defects documented, notify supervisor/leading hand of any defects. - Where defects are of a safety hazard park up machine and put an out of service tag on it and discuss hazard with supervisor/leading hand. They will have the hazard rectified. - Ensure all controls are in neutral or park. - Adjust the seat and use the seat belt. | Operator | 2 |
| Starting checks | Unsafe working conditions Increased likelihood of accident during operation | 1 | - The operator must be seated and in full control of the machine when starting. - Check the operation of all of the controls and gauges. - Listen and watch for defects. - Ensure all grounded attachments are safely retracted. - Check safety locking pins in quick hitch are locked for buckets or attachments. - Test brakes & hand brake. - Test lights, horn and backup alarm - Test steering, limiting devices and all equipment controls. | Operator | 2 |
| Set out work area. | Struck by road or construction plant. | 1 | - Barricade area off to restrict access and minimise plant working area. - All workers to wear safety vests. | Supervisor All | 2 |
|  | Persons falling or tripping over on site. | 2 | - Maintain good housekeeping. - Barricade off unsafe areas. - Clearly mark protruding objects using paint, caps or hazard tape. | All Supervisor Supervisor | 3 |
|  | Dust entering eyes | 2 | - Suppress dust using water truck. - Eye protection to be worn when required. | Supervisor All | 3 |
| FEL Operation | Striking personnel and plant | 1 | - Hold toolbox meeting to discuss hazards including all persons to stay well clear of operating FEL. - Reflective safety vests to be worn at all times. - Where possible minimise plant & personnel in work areas. | Supervisor All Supervisor | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Separate trafficable areas. - Install giveway and stop signs where necessary. - FEL to have operational flashing light and reversing beeper. | Supervisor Supervisor |  |
|  | Exposure to noise | 2 | - Suppress noise using mufflers. - Ear protection must be worn by personnel working near noisy equipment. | Workshop All | 3 |
|  | Struck by moving vehicle or objects, driving over objects on the ground that may flick up and strike another object, worker or the machine itself. Steel re-enforcement that may protrude outside the width of the bucket | 1 | - Do not carry passengers. - Keep machine under control at all times. - Carry the bucket as close to the ground as possible and racked back to allow greater stability & vision. - Travel at a safe speed for the operation being undertaken. - Be aware of what objects on the travel path that may flick up or be dislodged whilst traveling over them. - Be aware of objects that may protrude outside the width of the bucket. - Ensure that no persons are within range of being struck by protruding objects. | Operator | 2 |
|  | Uncontrolled slippage | 1 | - Do not travel across the slope on a steep incline. - Assess ground surface conditions eg; wet ground conditions, steep slope. | Operator | 2 |
| Fitting & Removing Attachments | Crush hazards Manual handling injury | 2 | - Take car when fitting or removing attachments. - Seek assistance from a workmate where required. | Operator | 3 |
| Using the FEL with bucket attachment | Various including possible death if risk controls not followed. | 1 | - Before loading, check the cubic capacity of the bucket and the type of material to be handled to ensure overloading does not occur. - At all times carry a loaded bucket as close to the ground as possible, to just clear ground obstructions. Do not raise the bucket to unloading height until reaching the dumping point. The dangers of a high bucket are instability, obstructed vision and the possibility of striking overhead obstructions. - When travelling on sloping surfaces with a loaded | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | bucket, always keep the bucket facing uphill and never travel sideways on a steep incline. If necessary, prepare the sloping surface by cut and fill before taking loads up or down. - Never travel in reverse before looking behind to check for persons or obstructions in your intended path. - Major factors in accidents during truck loading operations include truck position, driver location and traffic flow. Loader operators should move cautiously, especially in congested areas. - The loader operator and the truck driver should agree where the driver will stay when the truck is being loaded; either in the cab, or away from both loader and truck – never on the truck body. - Persons are not to ride in the bucket. - To avoid cave-in, never work close to loose edges. Remember the loaded machine is extremely heavy when back filling deep trenches, work at 90 degrees to its edge. - Buckets should always be grounded when the operator leaves the cab. A bucket left elevated for any reason should be securely blocked. - Equipment left unattended should be locked or made inoperable so that any unauthorised person, particularly a child, cannot start the engine. |  |  |
| Log grab attachment | Striking other persons onsite - death | 1 | - Operator is to ensure no other persons are in the vicinity of operating area of FEL with log grab attachment. - All persons on site to keep well clear of operating FEL. - If a person is assisting in the work process the operator and offside are to keep within eye contact at all times and the offside is to move to a safe area when logs or items are being moved by the FEL. | Operator Offsider | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Road travel | Tyre puncture Struck by moving parts Steering / brake failure | 1 | - Ensure the tyres or tracks are clear of mud & rocks. - Ensure the hinged clip between the two brake pedals is locked into position (where applicable). - Raise the bucket to a safe traveling height (but also keep vision clear. - Apply park brake if brake failure occurs. - Apply brake and stop if steering failure. | Operator | 2 |
| Excavating | Engine failure | 1 | 1. Warm up engine and hydraulic oil before starting work. | Operator | 2 |
|  | Obstruction to other personnel | 1 | 2. Carry the bucket close to the ground & racked backwards. | Operator | 2 |
|  | Ground failure leading to machine rollover Personnel being crushed | 1 | - Only competent operators to use machinery. - Operator to complete visual assessment of ground conditions. - All unbattered excavations to be barricaded off. - Do not operate close to an overhang or a deep ditch or excavation the weight could collapse the sides of the trench. | Supervisor Operator Supervisor Operator | 2 |
|  | Disruption to services Accidental entry of personnel may be struck. | 1 | - Locate all utilities in the area. - Barricade of the working area. - Clear everyone from the operating radius of the bucket. | Supervisor | 2 |
|  | Equipment damage Plant rollover , ground failure | 1 | - Always approach embankments or trenches from a 90 degree angle. - Do not use the bucket as a hammer. - When working on a slope determine whether to dump the spoil on the high side or low side of the trench. | Operator | 2 |
| Lifting | Sling failure | 1 | - The safe working load (swl) of the machine should be marked on the machine. - Do not exceed the safe work load and work radius limits of the machine. | Operator | 2 |
|  | Struck by moving load/part | 1 | 1. Check the SWL of the machine and the lifting lugs, slings & chains to ensure the load is correctly slung and balanced and is within the SWL. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Ground failure | 2 | 2. If there is any sign of instability lower the load immediately. | Operator | 3 |
|  | Failure during lifting | 1 | - When the load is raised keep all people at a safe distance. - Do not swing loads over people’s heads. - Do not leave the load suspended in the air. Lower it as soon as possible. - Always lower the load to the ground if leaving the machine. - Make allowances for materials when they are wet. They will weigh more than normal. - If you do not know the weight of the load do not lift. - When working on sloping or flat surfaces be aware of the safe bucket capacity eg; rocks falling from an overfull bucket and damaging other items or persons - Do not reeve or nip slings around buckets as the sling can be cut by the cutting edge or teeth of the bucket. - Do not place a sling over a tooth or teeth. - Place the lifting hook onto the approved lifting lug normally attached to the quick hitch. - The person who slings and directs the load must have a dogman’s certificate. - Inspect all slings prior to use to ensure they are serviceable. | Operator | 2 |
| FEL shut down | Personnel struck by rolling plant. | 1 | - Access parking area as to site vision for other works in the area (eg traffic situation on roadworks site). - Park on firm, level ground where possible. - Park the machine up and down the slope when parking on a grade. - Park in a non operating area. - Park away from deep excavations and trenches. - Place the transmission in neutral or park position. - Apply the park brake or hand brake. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Lower the bucket to the ground with the front cutting edge flat on the ground. - Lower other attachments (rippers) to the ground. - Idle the engine for gradual cooling before turning it off. - Turn the steering wheel back and forward and cycle the hydraulic controls to dissipate residual pressure in the hydraulic system before leaving the machine. - Remove the ignition key and secure the machine. |  |  |
| Maintenance | Personal injury | 1 | - Move machine to a position away from the work site. - Remove keys from ignition. - Place a lock out danger tag on the steering wheel of the FEL to alert others that you are working under or behind machine and not to start the machine. | Operator | 2 |
| Environmental Issues | Transmission of material off site to public domain eg mud or to main road. | 2 | 1. Where possible endeavor to not park in muddy areas. | Operator | 3 |
|  | Hydraulic oil spill | 3 | - Make sure there is a visual check of all hydraulic hoses is part of your pre start check. - Should there be an extensive oil leak- clean it up immediately. | Operator | 3 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.