| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | STABILISER - SAFE OPERATION | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025 SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Training | To prevent injuries or illness to workers | 1 | - Machinery operators will have the necessary current licence or competency certificate for the equipment they will operate. - Heed and obey warning signs. - Do not operate or work on this machine unless you have read and understand the instructions and warnings in the Operation and Maintenance Manual. - Failure to follow the instructions or heed the warnings could result in injury or death. - All persons operating equipment and working on site will have a current construction induction card. | Supervisor All | 2 |
| Arriving on site | Entry unsafe or unauthorised areas | 1 | - Report to site office for site induction if required. - Risk assessment of site conditions and Site specific induction & toolbox meeting to be conducted. - Consider all environmental protection issues and the impact caused by entry to site. | All Supervisor | 2 |
| Set out work area | Struck by road or construction plant. | 1 | - Barricade area off or sign appropriately to restrict access and minimise plant working area as per traffic management plan. - All site personnel to wear PPE as per site induction. - All vehicles and equipment to have operational flashing lights and reversing beepers. - UHF radios available where required. | Supervisor All Operator | 2 |
|  | Persons falling or tripping over on site. | 2 | - Maintain good housekeeping. - Barricade off unsafe areas. - Clearly mark protruding objects using paint, caps or hazard tape. | All Supervisor Supervisor | 3 |
|  | Dust entering eyes | 2 | - Suppress dust using water truck where applicable. - Eye protection to be worn at all times. | Supervisor All | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Pre Start Check | Unsafe equipment Potential for accident | 1 | - Pre start checklist forms filled out & defects documented, supervisor notified of any safety hazards or defects. - Make sure that all protective guards and all covers are secured in place on the equipment. - Make sure that the machine horn, the backup alarm and all other warning devices are working properly. - Refer to Operation and Maintenance Manual, “Daily Inspection” before you operate the machine. | Operator | 2 |
| Access/egress on plant | Falls | 2 | - Mount the machine and dismount the machine only at locations that have steps and/or handholds. - Before you mount the machine, clean the steps and the handholds. Inspect the steps and handholds. Make all necessary repairs. - Face the machine whenever you get on the machine and whenever you get off the machine. - Always maintain 3 point of contact with the steps and with the handholds when climbing up and down from stabilizer. | Operator | 3 |
| Before operation of stabiliser | Striking personnel and other plant. Plant roll over. Electrocution | 1 | - Hold toolbox meeting to discuss site hazards. - Make sure you are aware of any potentially low overhead wires and not these on toolbox meeting form and appoint spotter if required. - Use Dial Before You Dig to highlight if underground services will be a hazard. - Clear all personnel from the machine - maintain a minimum distance of 10 m (33ft). - Reflective safety vests to be worn at all times. Wear a hard hat, protective glasses, and other protective equipment, as required. Do not wear loose clothing or jewelry that can snag on controls or on other parts of the equipment. - Clear all obstacles that are in the path of the machine. - Beware of hazards such as wires, ditches, etc. - Know the appropriate work site hand signals and the personnel that are authorized to give the hand signals. - Accept hand signals from one person only. | Supervisor Operator All | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Stabiliser operation | Striking personnel and other plant. Plant roll over. Electrocution | 1 | - Only competent operators to operate the stabiliser. - Stabiliser to have operational flashing light and reversing beeper. - The operator must satisfy himself that no one will be endangered before moving the machine. - Never operate a stabiliser at speeds that are unsafe. - Only operate the machine while you are in a seat. - The seat belt must be fastened while you operate the machine. - Only operate the controls while the engine is running. - While you operate the machine slowly in an open area, check for proper operation of all controls and all protective devices. - Do not allow riders on the machine unless the machine has an additional seat. - Note any needed repairs during machine operation. - Report any needed repairs. - Lower the rotor. - Slowly propel the machine when you start a cut. This operation prevents the machine from riding up on the rotor and lunging backward. This operation also prevents excessive bouncing. - Do not allow anyone to stand or walk behind the machine while the machine is in operation. - The machine may lunge backward if the rotor hits an obstruction. - Carry attachments approximately 40 cm (15 inches) above ground level. - Do not go close to the edge of a cliff, an excavation, or an overhang. - If the machine begins to sideslip downward on a grade, immediately remove the load and turn the machine downhill. - Avoid any conditions that can lead to tipping the machine. | Supervisor Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - The machine can tip when you work on hills, on banks and on slopes. Also, the machine can tip when you cross ditches, ridges or other unexpected obstructions. - Avoid operating the machine across the slope. - When possible, operate the machine up the slopes and down the slopes. - Maintain control of the machine at all times. - Do not overload the machine beyond the machine capacity. - Be sure that the hitches and the towing devices are adequate. - Only connect the trailing equipment to a drawbar or to a hitch. - Know the width of your equipment in order to maintain proper clearance when you operate the equipment near fences or near boundary obstacles. - Be aware of high voltage power lines and power cables that are buried. If the machine comes in contact with these hazards, serious injury or death may occur from electrocution. |  |  |
|  | Cutter Hazard and Rotor Movement Hazard, Stay Back | 1 | - Rotor movement can cause injury or death. Stay clear of the rotor when the engine is running. - Shut off the engine before performing any service on the rotor or its housing. | Operator | 2 |
|  | Flying Debris Hazard, | 1 | 1. Flying debris may cause serious personal injury or death. Maintain a minimum distance of 10 m (33ft) | Operator | 2 |
|  | Crush Hazard | 1 | 1. Do not operate this machine if the rotor service panel door is not closed and secured. | Operator | 2 |
| Stabiliser shut down | Personnel struck by rolling plant Explosion Crushing | 1 | - Assess parking area as to site vision for other works in the area. - Park in a non operating area. - Park away from deep excavations and trenches. - Never leave the Stabiliser unattended with the engine running. Park the machine on a level surface. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - If you must park the machine on a grade, you must block the machine. - Move the propel lever to the NEUTRAL position in order to stop the machine. - Place all controls in the NEUTRAL position. - Engage the parking brake by pushing the parking brake control to the ON position. The parking brake control will light. - Lower the rotor and the hood until the rotor rests on the ground. - Stop the engine. - Turn the engine start switch to the OFF position and remove the key. - Turn the battery disconnect switch to the OFF position. |  |  |
| Maintenance Refueling | Personal injury Fire/explosion Burns Crush hazard | 1 | - Do not operate or perform any lubrication, maintenance or repair on this product, until you have read and understood the operation, lubrication, maintenance and repair information. - Attach a “Do Not Operate” warning tag or a similar warning tag to the start switch or to the controls before you service the equipment or before you repair the equipment and take out the key . - Support the equipment properly before you perform any work or maintenance beneath that equipment. - Do not depend on the hydraulic cylinders to hold up the equipment. Equipment can fall if a control is moved, or if a hydraulic line breaks. - Do not work beneath the cab of the machine unless the cab is properly supported. - Do not leave the engine running while making adjustments or repairs unless specifically recommended and a safe management plan is in place and a second person is in attendance. - Never refuel when the engine is running. | Operator Maintenance person | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Do not smoke while filling the fuel tank or servicing the fuel system. - Do not oil, grease, check for faulty wiring or loose connections or adjust any part of the Stabiliser while it is in motion. - Refer to Operation and Maintenance Manual for the procedure for placing the equipment in the servicing position. - Do not touch any part of an operating engine. - Allow the engine to cool before any maintenance. |  |  |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.