| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | WORKING IN ISOLATED & REMOTE AREAS | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group - Operations Manager and in consultation with management and workers – 2025. SWMS Review Date: August 2026 unless required earlier. Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Pre planning | Untrained persons No Communication Unsafe working conditions Working alone - increased likelihood of incident. Operation of motor boats. Drowning | 1 | - No person shall travel and work in remote areas unless they are competent in the work to be performed and do not have any current health issues that may cause a concern. (if a person has any medical condition which may require medication e.g. asthma, heart condition, is susceptible to fits or any other condition which may be unsuitable for bush walking). - It is the responsibility of all workers to inform their Supervisor prior to leaving for a remote location of any medical health issues they have. - The primary form of communication in isolated & remote areas is the Satellite phone- Supervisor to ensure all persons in the work crew understand how to use the satellite phone in an emergency situation. - No person shall work alone in isolated & remote areas. - Supervisor phones in at the start of work, and at the end of the shift (and at other times as deemed necessary due to working conditions) to base noting day’s work activities, location and welfare of all workers. - Only trained and competent operators with a boat licence operate motor boats refer SWMS-Aluminium Boat - Safe Working From. - Boat operators and passengers must be competent swimmers and use life jackets in areas deemed necessary e.g. fast flowing water or deep water. - Train all site personnel in emergency procedures and ensure that an appropriately stocked first aid kit is available on site. - A first aid person with Cardio-Pulmonary Resuscitation (CPR) training is to be available on site where there is a risk of drowning to workers. | Management Supervisor Workers | 2 |
| Driving and unfamiliar driving conditions | Vehicle crashes Vehicle breakdown Personal injury or death | 1 | - No person should drive or operate vehicles or equipment unless they are licensed and have been competently trained to do so including the driving of 4WD vehicles. - No person shall operate plant or equipment under the influence of alcohol, illicit drugs or prescription drugs that affect their ability to work safely. - All vehicles and equipment will have a pre-trip inspection undertaken and documented. | Management Supervisor Workers | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Daily pre-start checks will be conducted on all mobile equipment prior to the start of work. - Seat belts are to be fastened when vehicle is in motion. - Drivers Not to exceed speed limits and slow down to a speed appropriate for the driving conditions, e.g. poor visibility, road surface. - Drivers & passengers to remain alert for animals, slow moving vehicles and vehicles entering the road from farms or side roads. - Drivers not to take undue risks e.g. driving across flooded waterways. - If working extended hours take breaks or alternatives, i.e. accommodation or camp out. |  |  |
| Total Fire Ban and Extreme Weather | Starting a fire with petrol driven equipment. | 1 | - No petrol driven equipment is to be used on days of Total Fire Ban. - Supervisor to decide if works shall proceed on days of extreme weather conditions in consultation with workers and management. | Management Supervisor Workers | 2 |
| Undertaking tasks in heavily vegetated areas along waterways, floodplains or private land locations | Personal injury or death from striking or being crushed by falling trees or branches. | 1 | - Supervisor to monitor the weather condition and do not walk in forested areas and work under trees in high winds or in heavy rain. - All persons to be aware of overhead/hanging hazards - LOOK UP & LIVE! - Where overhead/hanging hazards are identified all persons wear a hardhat at all times. - Crew to carry a portable first aid kit if working a long way from the vehicle. | Supervisor Workers | 2 |
| Walking too close to a steep waterway bank or gully. Trying to jump across a gully or depression. | Personnel Injury/ Death Slips, trips, falls | 1 | - No person is to walk too close to steep banks (keep at least 2m distance). - No persons to attempt to jump steep gullies or depressions (look for a safer route). - All persons to avoid walking on logs/fallen debris - KEEP YOUR FEET ON THE GROUND! - All persons to watch out for holes/mineshafts/burrows and loose rocks. - Crew to carry a portable first aid kit if working a long way from the vehicle. - All persons to wear robust walking boots with non-slip sole. | Supervisor Workers | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Difficult terrain / long distance and physical activity or high work load in the hot weather | Personnel Injury: High air temperature. Exposure to the sun. Intense physical activity or high work load. | 1 | - Supervisor to reorganise work if possible so that more of the work is done in Cooler parts of the day, for example, early morning. - Workers to carry drinking water with them at all times and make sure they drink on a regular basis during hot weather. At least 2 litres of water a day is a good measure. - Suitable Work Clothing should be worn at all times- long pants. Long sleeves, work boots, gum boots if working in water. - No loose clothing to be worn when working with drills. - Sun hates or wide brimmed attachments on helmets to protect from direct sunlight. - All persons to take rest breaks if walking long distances. | Supervisor Workers | 2 |
| Working in or near bodies of water | Drowning | 1 | - All persons walking/working in or near bodies of water must be competent swimmers and wear life jackets where within 1.5m of the edge. - All persons are to wear appropriate clothing and gum boots. No waders are to be worn in streams. - All persons are to be aware of their surrounding at all times and be extra careful where you place your feet and watch out for deep holes in unclear waters. - All persons are to watch out for their workmates and alert them to possible dangers. | Supervisor Workers | 2 |
|  | Leeches | 3 | - All persons to wear long clothing and gumboots when walking & working near or in water or dense bush. - Check yourself at all times for leaches and follow emergency guidelines to remove leeches. The simplest way to remove leeches is with salt: a shake onto the body and most will quickly drop off. Tea tree oil or vinegar dabbed onto the body are also effective alternatives or simply pull them off. It is a good idea to get a fresh wad of cotton wool and dab a little tea tree oil onto the open wound to prevent infection. While leeches are not known to spread disease, a particular type of bacteria has been identified in their stomachs which may be passed onto the host. | Supervisor Workers | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
| Camping Out | Fallen limbs Flood water Escaping camp fire | 1 | - Do not camp under trees. - Do not camp in flood zone areas, as water may rise quickly overnight. - No camp fires are to be lit during months of high fire danger Nov-April or a designated by local fire authority. - Follow guidelines in ‘LIVING AWAY FROM HOME RESPONSIBILITIES’ as attached. | Supervisor Workers | 2 |
| Other Site Hazards | Snake bites Allergic reaction Fatality | 1 | - Snake bite bandages should be carried with each work group and in first aid kit. - All workers should be trained in correct use of bandage if bitten and emergency procedure. - Be aware of surroundings (snakes can be found anywhere, especially under rocks, logs, planks, long grass etc. and near water sources) - Check ground before exiting vehicle. - Visually inspect where you are working. - Check the immediate surroundings before Sitting / kneeling or squatting. - Ensure adequate lighting at all times. - Use a stick to move stones, logs etc. if required. Do not use fingers/hands. - Make noise. - Do not approach a snake (dead or alive). Treat every snake as if it is venomous. | Supervisor Workers | 2 |
|  | Wasp/Bee Stings Personal Injury: Sting Allergic reaction Localised pain/swelling | 1 | - Be aware of surroundings (bees hives, wasp nests) - Wasps nest in logs, walls or underground. They are generally more aggressive than bees and attracted to food and drink. - Do not contact/disturb nests/hives. If nests/hives are located in work area – contact professional to remove before undertaking work. - Do not drink directly out of cans if left unattended (use a straw). - Avoid wearing perfumes and scents. - Avoid dark or brightly coloured clothing. - Drive with windows up. - Consider head-nets if required. | Supervisor Workers | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls in place |
| --- | --- | --- | --- | --- | --- |
|  | Fall injury | 2 | 1. Maintain 3 points of contact when getting up and down from equipment and climbing in and out of steep banks. | Supervisor Workers | 3 |
|  | Exposure to noise | 2 | - Hearing protection must be worn at appropriate times when working near noisy equipment or where equipment is suitably signed. - No person shall use personal radios whist at work this will impair their ability to hear in an emergency situation. | Supervisor Workers | 3 |
|  | Sunburn may cause skin damage. | 1 | 1. Workers to wear long pants, long sleeves, sun hats, sunscreen, sunglasses and lip protection as per ‘Personal protective equipment’ procedure. | Supervisor Workers | 2 |
|  | Crew members falling or tripping over | 3 | 1. Check for hidden objects/holes. | Supervisor Workers | 3 |
| Evacuation procedures | Fire Storm Medical emergency | 1 | - Evacuation point to commence at vehicle / exit area via quickest route. - Evacuation routes to be discussed at daily toolbox meeting and noted on toolbox meeting form. - Follow all directions given by Supervisor or most senior person on site during evacuation. | Supervisor Workers | 2 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions e.g. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.