| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor: Aaron Foster | Date: | 14/05/2026 |
| High Risk Construction Work: | Loading, staging and installation of precast concrete panels | Project: | Paneltec Development |
| Craig R. Large SWMS prepared by Craig Large - Paneltec Group – Safety and Compliance Manager and in consultation with management and workers – 2026 SWMS Review Date: August 2027 unless required earlier Signature: |  | Location: | Paneltec, 30 Remount Road |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 01/05/2026 – V1.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Training | To prevent injuries or illness to workers | High | - Machinery operators will have the necessary current licence for the equipment they will operate. - All workers to be site inducted, briefed on this SWMS and authorised by supervisor. - All persons will have a current construction induction card. - Precast erection supervisor - Competent person experienced in precast erection, lift sequencing and temporary works controls. - Brace installation / alteration / removal workers - Trained and deemed competent in the specific bracing system, anchor installation and engineer’s requirements. - Spotter - Competent and authorised under site traffic management arrangements. | Supervisor All | Medium |
| Pre-start Planning and Documentation |  |  |  |  |  |
| Review design, drawings, lift plan and bracing requirements | Incorrect erection sequence; brace failure; panel collapse; conflict between trades | Extreme | - Confirm current approved drawings, panel layout, erection sequence, lift plan and engineer’s bracing design are available on site. - Conduct pre-start meeting with crane crew, riggers, panel installers, mobile plant operators and supervisor. - No substitutions, brace changes or sequence changes unless authorised in writing by engineer or competent temporary works designer. | Supervisor | Medium |
| Confirm ground conditions and crane set-up area | Crane/forklift/Excavator instability; ground bearing failure; outrigger failure; panel dropped | Extreme | - Verify ground bearing capacity, crane pad design and outrigger matting. Inspect for excavations, trenches, voids, soft ground, underground services and recent rain effects. - Crane set-up must comply with lift plan and crane manufacturer requirements. | Supervisor Crane Operator | Medium |
| Confirm weather conditions | Wind loading on suspended panel; uncontrolled panel movement; lightning; reduced visibility | High | - Check forecast and current conditions. Do not lift panels where wind exceeds crane/lift plan/panel manufacturer limits. - Stop work during lightning, severe weather, poor visibility or unsafe gusting conditions. | Supervisor All | Low |
| Confirm services and work area | Contact with overhead powerlines; underground service strike; public interface | High | - Locate and mark services. Establish no-go zones for overhead electrical assets. - Use spotters where required. Coordinate with traffic management where work interfaces with vehicles or pedestrians. | Supervisor All | Low |
| Delivery, Unloading and Storage of Panels |  |  |  |  |  |
| Inspect panels and lifting points | Damaged panel; failed insert; incorrect panel lifted | High | - Verify panel ID against drawings. - Inspect for cracks, damaged edges, damaged inserts, missing hardware or transport damage. - Do not lift damaged or unidentified panels until assessed by competent person/engineer. | Supervisor | Low |
| Lift panels | Crush injury; suspended load; rigging failure; panel instability | Extreme | - Only licensed dogger/rigger to sling loads. - Use certified lifting gear. - Establish exclusion zone around truck, crane and suspended panel. - No person under suspended load. - Use tag lines only where safe and not within fall zone. - Follow lift plan. | Supervisor Crane Operator Dogger/Rigger | Medium |
| Store/stage panels temporarily | Panel topple; unstable supports; ground failure | Extreme | - Store only in engineered racks or approved support system. - Do not lean panels against structures or plant unless specifically engineered. - Install and mark out “No Go Zone” - Keep people out of potential fall zone. - Inspect supports and ground conditions. | Supervisor All | Medium |
| ## Crane Lifting and Panel Erection |  |  |  |  |  |
| Set up crane and lifting zone | Crane overturn; collision; unauthorised entry | Extreme | - Crane set up as per lift study. Establish barricaded exclusion zones covering crane slew radius, lifting path and panel fall zone. - Confirm communication protocol between crane operator, dogger and erection crew. | Supervisor Crane Operator Dogger/Rigger | Medium |
| Attach lifting gear | Incorrect rigging; lifting insert failure; hand crush | High | - Dogging/rigging only by licensed competent persons. Confirm lifting clutch compatibility, correct orientation and full engagement. - Keep hands clear of pinch points. Use approved access where needed. | Supervisor Crane Operator Dogger/Rigger | Low |
| Lift panel from truck or rack | Panel swing; shock loading; load failure; crush | Extreme | - Trial lift to confirm balance. Lift slowly and smoothly. No one under or in front of panel. - Maintain clear escape paths. - Use tag lines only where they do not place workers in line of fire. | Supervisor Crane Operator Dogger/Rigger | Medium |
| Rotate and position panel | Panel collision; struck-by panel; wind sail effect | Extreme | - Maintain exclusion zone. - Use dogger as single lift director. - Use controlled crane movements. - Monitor wind. - Do not place workers between panel and fixed object. - Use EWPs or engineered access systems for connection tasks. | Supervisor Crane Operator Dogger/Rigger | Medium |
| Land and temporarily secure panel | Panel collapse; misalignment; brace delay; foot crush | Extreme | - Panel must not be released from crane until minimum temporary supports/braces are installed and verified as per engineer’s design. - Keep non-essential workers outside fall zone. - Use packers/shims as designed. | Supervisor Crane Operator Dogger/Rigger | Medium |
| Install permanent fixings as required | Falls; dropped tools; pinch/crush injuries | High | - Use approved access system. - Tool lanyards where working at height. - Maintain exclusion zone below. - Follow connection design and inspection hold points. | Supervisor All | Low |
| Temporary Bracing Installation |  |  |  |  |  |
| Install temporary braces | Panel collapse; incorrect brace angle; anchor failure; crush injury | Extreme | - Install braces strictly to engineer’s bracing design, including brace type, quantity, location, angle, anchor type, embedment, torque and substrate requirements. - Do not improvise or relocate brace points without written approval. | Supervisor All | Medium |
| Drill and install brace anchors | Silica dust; noise; hand injury; anchor failure | High | - Use dust suppression or extraction. - Wear respiratory protection where required. - Use correct drill size, depth and cleaning method. - Install anchors to manufacturer and engineer requirements. Verify torque/pull testing if specified. | Supervisor All | Low |
| Tension/adjust braces | Overloading panel; brace instability; hand pinch | High | - Adjust gradually and evenly. - Do not use braces to force panel alignment beyond design allowance unless authorised. - Use correct tools and maintain clear body position. | Supervisor All | Low |
| Inspect installed bracing | Missed defect; loose fixings; wrong brace | High | - Supervisor/competent person to inspect before crane releases panel. Record sign-off where required. - Confirm braces are protected from impact by mobile plant and other works. | Supervisor All | Low |
| ## Mobile Plant Operating Around Erected or Braced Panels |  |  |  |  |  |
| Operate mobile plant near panels or braces | Impact with brace or panel; panel collapse; crush injury | Extreme | - Establish plant exclusion zones around panels, braces and brace footings. - Use physical barriers or bollards where practicable. No mobile plant to travel within designated brace protection zone unless authorised by supervisor and controlled by spotter. | Supervisor All | Medium |
| Travel under or beside erected panels | Panel fall; crush; limited escape route | Extreme | - Avoid travel through panel fall zones wherever practicable. - Where unavoidable, supervisor must authorise route, confirm bracing is protected and stable, and use spotter/traffic control. - Do not park plant beside braced panels unless approved. | Supervisor All | Medium |
| Lift materials near braced panels | Collision; dropped load; brace damage | High | - Use designated loading/unloading zones away from braces. - Maintain slew and load clearance from panels. - Do not drag loads over brace feet, bolts or anchors. | Supervisor All | Low |
| Work around blind spots | Person struck by plant; collision | High | - Use spotters, radios, reversing alarms, flashing beacons and exclusion zones. - Workers must maintain eye contact with operator and stay out of blind spots. | Supervisor All | Low |
| low-light work | Reduced visibility; collision with braces/panels | High | - Provide adequate lighting to panels, braces, travel paths and exclusion zones. - Use illuminated plant and high-visibility PPE. - Stop work if visibility is inadequate. | Supervisor All | Low |
| Altering, Adjusting or Removing Temporary Bracing |  |  |  |  |  |
| Assess request to alter or remove brace | Premature brace removal; structural collapse; unauthorised change | Extreme | - HOLD POINT: No brace is to be altered, loosened, relocated or removed unless written approval has been provided by the project engineer/temporary works engineer or other authorised competent person. - Confirm permanent structural connections are completed and inspected. | Supervisor All | Medium |
| Inspect structure before brace change | Missing welds/bolts/connections; incomplete diaphragm; instability | Extreme | - Competent person to inspect permanent structural elements, fixings, grout, welds, bolts, roof/floor diaphragm and panel connections against engineer’s requirements. - Use sign-off checklist before work proceeds. | Supervisor All | Medium |
| Establish exclusion zone before brace removal | Struck/crushed by panel or brace; unauthorised entry | Extreme | - Barricade fall zone and brace work area. - Remove non-essential personnel and plant. - Communicate brace removal activity at pre-start or task briefing. - Maintain emergency access. | Supervisor All | Medium |
| Loosen or remove brace fixings | Brace release under load; sudden panel movement; hand injury | High | - Remove braces in engineer-approved sequence only. - Do not stand in line with brace or between panel and brace. - Use correct tools and access equipment. - Stop immediately if movement, cracking, unusual noise or resistance is detected. | Supervisor All | Low |
| Relocate or alter brace | Incorrect new location; inadequate anchor substrate; insufficient bracing | Extreme | - Relocated brace must be installed to engineered design before original support is removed unless approved otherwise. - Verify anchor installation, torque and brace angle. - Update drawings/records where required. | Supervisor All | Medium |
| Final brace removal | Delayed instability; undocumented change | High | - Record brace removal sign-off. - Remove anchors or make safe protrusions. - Reinstate barricades or open area only after supervisor confirms the area is safe. | Supervisor All | Low |
| Working at Heights During Panel Works |  |  |  |  |  |
| Access lifting points, brace points or connections at height | Fall from height; dropped tools; EWP crush | High | - Use EWP, scaffold or engineered platform. - Do not climb panels, braces or unauthorised structures. - Maintain harness use where required. Keep tools secured. - Maintain exclusion zone below. | Supervisor All | Low |
| Operate EWP near panels | EWP collision; entrapment; overturn | High | - EWP operator to be competent. - Ground conditions to be suitable. - Use spotter where required. - Maintain clearance from panels, braces, powerlines and crane operations. | Supervisor All | Low |
| ## Environmental and Public Interface Controls |  |  |  |  |  |
| Concrete drilling/cutting | Silica dust; noise; slurry runoff | High | - Use on-tool dust extraction or wet methods. Control slurry and prevent discharge to stormwater. - Wear hearing and respiratory protection as required. | Supervisor All | Low |
| Work near public roads, footpaths or occupied areas | Public struck by plant/load; unauthorised access | High | - Install site fencing, exclusion zones, traffic management, pedestrian detours and signage. Use qualified traffic controllers where required. - Coordinate deliveries outside peak periods where practicable. | Supervisor All | Low |
| Waste management | Trip hazards; environmental contamination | Medium | - Remove offcuts, packaging, failed anchors and debris. - Dispose of waste in designated bins. Keep access routes clear. | Supervisor All | Low |

| Mandatory Hold Points |  |
| --- | --- |
| Hold Point | Required Action / Approval |
| Before first panel lift | Lift plan, crane set-up, exclusion zones and communication method confirmed by supervisor and crane crew. |
| Before lifting each panel | Panel ID, lifting points, rigging and panel condition verified. |
| Before releasing crane from panel | Temporary braces installed, fixed, torqued and inspected as per engineer’s design. |
| Before mobile plant enters panel/bracing zone | Supervisor confirms route, exclusion controls and brace protection. |
| Before altering any brace | Written engineer/competent person approval obtained. |
| Before removing any brace | Permanent structural stability verified and signed off by competent person/engineer. |
| After brace alteration/removal | Area inspected, records updated and exclusion zones amended. |
| After any brace or panel impact | Work stops; engineer/competent person inspection before work resumes. |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Wet Concrete | During heavy rain, recently placed concrete could emulsify and wash into surrounding waterways. | 1 | - Plan concrete operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste products will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures – 2018 |
| Precast, Tilt-up and Concrete Elements in Building Construction 2008 |  |  |  |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form