| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman: Neville Young | Date: | 12/05/2026 |
| High Risk Construction Work: | TRENCH EXCAVATION SHORING & LAYING PIPES | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group - Operations Manager and in consultation with management and workers – 2023 SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: | Dry St, Invermay |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| - Training | To prevent injuries or illness to workers | 1 | - Machinery operators will have the necessary current licence for the equipment they will operate. - All persons will have a current construction induction card. | Supervisor All | 2 |
| - Arriving on site | Entry unsafe or unauthorised areas | 1 | - Report to site office for site induction if required. - Risk assessment of site conditions and site-specific induction & toolbox meeting to be conducted. - Consider all environmental protection issues and the impact caused by entry to site. | All Supervisor | 2 |
| - Unloading Plant & equipment | Falls, slips & trips from vehicle, trucks and floats Machinery plant tipping over or falling from ramps | 1 | - Ensure all vehicles are parked in a clear level area for unloading plant and equipment. - Extra care taken in wet conditions, use a ramp to load and unload plant. - Persons unloading equipment to wear seat belt. - Operators familiar with SWMS – Loading & Unloading Plant & Equipment. - Only suitably trained personnel to work with plant | Operator | 2 |
|  | Electrocution | 1 | 1. Be aware of overhead power lines | Operator | 2 |
|  | Traffic hazards, hit by vehicles or other on site plant and equipment | 1 | - Ensure hazard & warning signage is in place as required when unloading plant. - Training in the use of PPE equipment such as fluorescent safety vests/clothing. | Operator Supervisor | 2 |
|  | Hand, foot & hearing damage, crushing of limbs | 2 | 1. Training in manual handling & the correct protection, gloves and safety footwear. | Supervisor | 3 |
| - Machine Pre Start Check | Unsafe working conditions. Potential for accident. | 1 | - Equipment Prestart checklist forms filled out & defects documented, supervisor notified. - Ensure windows/ mirrors are clean. - Ensure all controls are in neutral or park. - Ensure all gear in the cabin is correctly stored. - Adjust the seat and use the seat belt. | Operator | 2 |
| - Machine Starting checks | Unsafe working conditions. Increased likelihood of accident during operation. | 1 | - The operator must be seated and in full control of the machine when starting. - Check the operation of all of the controls and gauges. - Listen and watch for defects. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Test hydraulic functions. - Ensure all grounded attachments are safely retracted - Test lights, horn and travel alarm. - Check safety locking pins in quick hitch are locked for buckets or attachments. |  |  |
| - Set out work area and check excavation levels. | Struck by road or construction plant. | 1 | - Barricade area off or sign appropriately to restrict access and minimise plant working area. - All workers to wear full length hi viz clothing. - Operational flashing lights and reversing beepers. - All trenches greater than 1.5 metres in depth are to be appropriately shored, benched, battered or otherwise protected from collapse. - Trenches 1.5 or greater must be accompanied by a suitable rescue/escape plan. | Supervisor All Operator | 2 |
|  | Striking underground services - electrocution | 1 | - Before digging starts, make sure you know the exact location of any underground electrical cables, gas lines, water, sewerage and telecommunications cables and Complete a Excavation permit. - Do not rely solely on site plans and drawings as these are sometimes not accurate or complete. - Seek assistance from the local services and distribution companies. - Use “Before you Dig Australia” - If services are identified in the excavation area non-destructive proving/potholing and marking is required before excavation commences. Emergency Procedure to be used: - If the contact point is under the ground there is an area around that point which can be electrified. - Jump well clear of that point and move away from the machine and warn others to keep away - Stay near the site until help arrives - Report the incident to your supervisor immediately who may need to report to WorkSafe. | Supervisor All | 2 |
|  | Persons falling or tripping over on site. | 2 | - Maintain good house keeping. - Barricade off unsafe areas. - Clearly mark protruding objects using paint, caps or hazard tape. | All Supervisor Supervisor | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Dust entering eyes | 2 | - Suppress dust using water truck. - Eye protection to be worn when required. | Supervisor All | 3 |
| - Excavator operation | Engine failure | 2 | 1. Warm up engine and hydraulic oil before starting work. | Operator | 3 |
| - Clearing & grubbing Excavation placement and compaction fill | Striking other objects whilst slewing excavator, materials on the ground flicking up as excavator travels over site. Overbalance of machine at reach. | 1 | - Be aware of what obstacles are around the site especially when slewing excavator. - Assess what weights the machine can handle at reach safely before attempting to load trucks. - Do not carry passengers. - Keep machine under control at all times. - Carry the bucket as close to the ground as possible whilst traveling. - Travel & operate at a safe speed for the operation being undertaken. | Operator | 2 |
|  | Attachments or bucket disconnecting from hitch. | 1 | 1. Ensure quick hitch safety pins are engaged whilst bucket or attachments are engaged into quick hitch. | Operator | 2 |
|  | Release of hydraulic hoses when changing work implements such as tilt bucket, pulverisor, rockbreaker and compaction plate. | 1 | 1. Ensure hydraulic pressure is taken off hydraulic hoses before releasing couplings. Pressure is normally released if engine is shut down and work tool pedal is pressed back & forth or two way flow ball valve is switched off. | Operator | 2 |
|  | Uncontrolled slippage | 1 | - Assess ground surface conditions eg; wet ground conditions, steep slope. - When working on top of mound make sure excavator is not taken too close to the edge. | Operator | 2 |
|  | Rockbreaker & pulverizer operation Flying fragments or reo steel from concrete | 1 | - Be aware that fragments can fly from chisel of the rockbreaker, and pulverizer . - Whilst operating excavator be aware that demolition debris such as reo. and pipe etc… can be flicked up by the tracks and could hit a worker or machine. - Use eye & hearing protection & keep others well away from work area. | Operator | 2 |
|  | Rock & log grab | 2 | 1. Material or objects lifted can become insecure due to crumbling or breaking be aware of the different hydraulic pressures that can be applied to objects | Operator | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Compaction plate | 2 | - Be aware that vibration can effect nearby buildings or unstable ground or trench walls & excavations. - Assess the situation & surrounding areas that may be affected. | Operator | 3 |
|  | Striking overhead services- electrocution | 1 | Operators need to keep; - Clearly mark overhead assets with flagging/signage - At least 3 metres from 132,000 volt lines. - At least 6 metres from 132,000 to 330,000volt lines. - Areas closer than 3m are no go zone. - Use spotter if working near powerlines. 5. (3m to 6.4m). - Clarify safe working distances with site manager. - If in doubt of line voltage keep at least 8 metres away. If your machine makes contact with electrical wires- Refer to Emergency Procedures below and the A5 laminated card in the cabin. | Operator Supervisor | 2 |
|  |  | 1 | Overhead Wires – Emergency Procedure - Stay calm. - Do not leave your seat. - Warn others to stay away. - If possible, try to break contact with the wires, if possible try to lower the bucket without causing further damage to the power lines. - Make sure that the bucket does not make contact with the ground as this could earth the machine. - Phone emergency services to have power turned off. If it is unsafe to stay in/on the machine because the machine is in danger, such as from fire or if you are alone. - Jump well clear onto dry ground if possible, making sure you have both legs together and shuffle with feet together or hop (eg ; kangaroo hop) and hop at least 10 metres away from the machine. - Do not climb down because if you make contact with the machine and the ground at the same time you could be electrocuted. - If the contact point is under the ground there is an area around that point which can be electrified. | All | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Jump well clear of that point and move away from the machine and warn others to keep away. - Stay near the site until help arrives. - Report the incident to your supervisor immediately who may need to report to WorkSafe |  |  |
|  | Striking personnel and plant | 1 | - Hold toolbox meeting to discuss hazards. - All workers to wear full length hi viz clothing - Where possible minimise plant & personnel in work areas. - Separate trafficable areas. - No personnel are to be within the working radius of any plant equipment unless positive comms or visual contact is established with the operator” - Install giveway and stop signs where necessary. - Excavator to have operational flashing light and reversing beeper. - ‘Blind spots’ of plant to be identified and discussed with all workers at toolbox meetings. - Where ‘Blind spots’ are identified as a potential issue a spotter should be used to minimize hazard. - Where practicable, plant should always move in a forward direction. | Supervisor All Supervisor Supervisor Supervisor Operators | 2 |
|  | Exposure to noise | 2 | - Suppress noise using mufflers. - Ear protection must be worn by personnel working near noisy equipment. | Workshop All | 3 |
|  | Ground failure, Excavation collapse, Sloping ground or Rough unprepared ground, leading to machine rollover and personnel being crushed | 1 | - Only competent operators to use machinery. - Operator to complete visual assessment of ground conditions. - All unbuttered excavations to be barricaded off. - Do not operate close to an overhang or a deep ditch or excavation the weight could collapse the sides of the trench. - Always approach embankments or trenches from a 90 degree angle. - Do not use the bucket as a hammer. - When working on a slope determine whether to dump the spoil on the high side or low side of the trench. - Adjust the seat and use the seat belt. | Supervisor Operator Supervisor | 2 |

| Steps in Activity | Potential Hazards | Risk Class |  | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Lifting | Sling failure | 1 |  | - The safe working load (SWL) of the machine should be marked on the machine. - Do not exceed the safe work load and work radius limits of the machine. - All lifting equipment should be tested and tagged for compliance | Operator | 2 |  |  |
|  | Struck by moving load/part | 1 |  | 1. Check the SWL of the machine and the lifting lugs, slings & chains to ensure the load is correctly slung and balanced and is within the SWL. | Operator | 2 |  |  |
|  | Ground failure | 1 |  | 1. If there is any sign of instability lower the load immediately. | Operator | 2 |  |  |
|  | Failure during lifting | 1 |  | - When the load is raised keep all people at a safe distance. - Do not swing loads over people’s heads. - Do not leave the load suspended in the air. Lower it as soon as possible. - Always lower the load to the ground if leaving the machine. - Make allowances for materials when they are wet. They will weigh more than normal. - If you do not know the weight of the load do not lift. - When working on sloping or flat surfaces be aware of the safe bucket capacity eg; rocks falling from an overfull bucket and damaging other items or persons. - Do not reeve or nip slings around buckets as the sling can be cut by the cutting edge or teeth of the bucket. - Do not place a sling over a tooth or teeth. - Place the lifting hook onto the approved lifting lug normally attached to the quick hitch. - For slinging and lifting of loads where judgement is not required and where the load is always in view of the plant operator a dogger is not required. - A qualified dogman is to be on site or : - Load to always remain in view of the plant operator - Selection of the slinging methods to be pre-determined by a competent person - Lifting points to be pre-determined by a competent person and marked on the load where appropriate. - Weight of the load—or load within a weight range—is pre-determined e.g. may be marked on the load, and suitable lifting equipment (i.e. chains, swift lifts etc.) to be used - Inspect all slings prior to use. Are they serviceable? - Refer to SWMS – Using Powered Earthmoving Equipment as a Crane. | Operator | 2 |  |  |
| - Traveling equipment on roadway, private or public areas | Damage to roadway or public / private property | 2 |  | - Check ownership of properties or boundaries. - Travel machine as to avoid disturbance to roadway or public or private lands eg; lift machine to turn on undisturbed areas if required, avoid sharp turns on roadways. - Be aware of what damage the tracks can do to surfaces. | Operator | 3 |  |  |
| - Equipment shut down | Personnel struck by rolling plant | 1 | - Assess parking area as to site vision for other works in the area ( eg traffic situation on roadworks site, avoid parking where vision is blocked). - Park on firm, level ground where possible. - Park the machine up and down the slope when parking on a grade. - Park in a non-operating area. - Park away from deep excavations and trenches. - Place the hydraulic lock mechanism in the lock position. - Lower the bucket or attachment to the ground. - Idle the engine for gradual cooling before turning it off. - Move the steering levers back and forward and cycle the hydraulic controls to dissipate residual pressure in the hydraulic system before leaving the machine. - Remove the ignition key and secure the machine. |  | Operator |  | 2 |
| - Environmental Issues | Transmission of material off site to public domain eg mud or to main road. | 2 | 1. Where possible endeavor to not park in muddy areas. |  | Operator |  | 3 |
|  | Hydraulic oil spill | 3 | - Make sure there is a visual check of all hydraulic hoses is part of your prestart check. - Should there be an extensive oil leak- clean it up immediately. |  | Operator |  | 3 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| - Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| - Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| - Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| - Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| - Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| - Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| - Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| - Soil Management & - Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| - Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| - Visual Impact - Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.