| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | TRACTOR - SAFE OPERATION | Project: |  |
| SWMS prepared Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025 SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| General safety precautions. | Injuries could vary from minor to death if safe work practices are not adhered to. | 1 | - Only competently trained operators use machinery. - Before starting a tractor, know your equipment and its limitations, read and follow the manufacturer’s operating instructions. - Pay special attention to the warning and caution labels on the tractor itself. - DO NOT operate tractor or any implement attached to it while under the influence of alcohol, medication or other substances or while fatigued. - Check the layout of the land and watch for hidden objects, ditches, embankments and depressions, especially when the edge is unstable or slippery conditions prevail. - Tractors is fitted with ROPS and retractable seat belt. - Wear appropriate PPE – Protective clothing, footwear. Never wear loose, torn or bulky clothing around tractor or implements. - Never run the engine in a closed shed — the exhaust gases can be fatal. - No person should be allowed to stand or walk beside a machine when it is being operated. DO NOT allow any bystanders around or near tractor during operation. - DO NOT allow passengers on vehicles without an OEM seat with seatbelt.. | Supervisor Operator | 2 |
| Pre Start Check | Unsafe equipment Potential for accident | 1 | - Pre start checklist forms filled out & defects documented, supervisor notified. - Check brakes, clutch and other mechanical parts for faulty adjustment. - Ensure that the seatbelt, Roll Over Protective Structure (ROPS), Falling Objects Protective Structure (FOPS) (where fitted) and Power Take-Off (PTO) guard are in sound condition. - Faulty equipment must not be used. Report suspect machinery immediately. - Check the 3-point linkage, pneumatic and hydraulic systems are functioning. - Use only implements that meet the specifications listed in the vehicle operation manual. - Ensure tractor driver is trained and competent and if driving on public roads, appropriately licensed. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - If the tractor is used on public roads, ensure that all lights and warning devices are operational and the vehicle is registered. - Adjust the seat and use the seat belt. |  |  |
| Attaching implements | Tractor may flip or overturn if not hitched correctly or loose or broken ends can whip around violently and may cause severe injury. Injury to bystanders. | 1 | - Check the owner's manual for the proper hitching position. - Only attach implements to the drawbar, three-point linkage or other hitch points designed for that purpose. - Never hitch to points in front of, or higher than the drawbar as this could be extremely dangerous. - When using stationary PTO-driven implements, leave the implement hitched to the tractor so that the hitching distance doesn't change. - To attach any implement on the 3 point linkage, link up the left side hydraulic arm (facing the rear of the tractor), then the right arm, winding the adjuster up or down as necessary, then attach the top link, adjusting as necessary. - If the implement is PTO (power take off) driven, attach the PTO shaft last. - NEVER work around the rear of a tractor with the PTO running. - Do not remove any guards attached to PTO shafts or drive heads. - Make sure when connecting up the PTO shaft to the tractor spline that the lock button returns fully to the lock position. - It is a good idea to grease the universal joints on both ends of the PTO shaft before attaching the tractor, as it is easier to do so if the grease nipples are more accessible. - Once the equipment is hooked up properly, raise the hydraulics and tighten the sway chains or arms, set top link to suit the type of implement making sure the PTO shaft is at the correct angle. - Remember to check oil levels, grease nipples and all other parts before starting up the PTO. - When reversing the tractor to the implement to be hitched, ensure that other people who may be assisting are well clear. - Never use three-point linkages when there is someone between the tractor and the implement, and lower all implements and fork lifts when not in use. - When pulling heavy loads, such as pulling a bogged vehicle clear, it is safer to use reverse gear and hitch to the front drawbar. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Ensure that the weight applied to the three-point linkage by lifting jibs or other attached equipment does not exceed the manufacturer’s specifications or affect stability or steering. - Get expert technical advice before fixing counter weights or wheel weights (front or rear) to increase tractor stability. - Check the operating manual or supplier about the recommended weight of a trailer or implement that the tractor can safely tow before towing it. |  |  |
| Starting a tractor | Entanglement in PTO Noise Rollover | 1 | When starting a tractor, the following precautions should be taken: - Only start and use the tractor according to the manufacturer’s instructions. - Operate the self-starter only from the driving position and do not start the machine while standing on the ground. - Before starting a tractor engine, check that the handbrake is on and the vehicle is not in gear. - Engage the appropriate gear for the work being undertaken. - Do not operate or idle engine in a non-ventilated area. | Supervisor Operator | 2 |
| Tractor operation | Run over by tractor Injuries to operator and bystanders. | 1 | - Pull only from the drawbar or hitch. - Never get on or off a moving tractor, no matter how slow it is moving. Modify the task to remove the need for frequent stopping and dismounting. - Never carry passengers on tractors, especially children, unless they have a separate seat. - Wear sturdy, non-slip footwear. - If the engine is still running when getting off a tractor, ensure the park brake is on, the gear lever is in neutral, and take care not to hit the gear levers with a foot when getting on or off. - Drive tractors at speeds slow enough to keep control in the event of unexpected hazards and be cautious in wet conditions. - Do not operate near ditches, holes or embankments, which may collapse under the tractor’s weight. - Reduce speed before turning or applying turning brakes. - Descend slopes cautiously with tractor in low gear. For example, on downward slopes it is possible, in extreme circumstances, for one wheel to reverse, causing the tractor to roll over. | All Supervisor Supervisor | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Extra care needs to be taken if towing trailers or implements down slopes, as often the trailers will not have brakes. - Ascending steep slopes can cause a tractor to back flip in extreme circumstances or the front wheels to lift thus reducing or losing control of steering. - To increase stability when working on hillsides, set tractor wheels to the widest possible setting. - When a tractor is bogged in mud or in a ditch, drive out in reverse gear. - Logs and planks should only be used behind the rear wheels to increase traction, as using logs and planks in front of the rear wheels increases the chance of back flipping. - When an attachment becomes blocked, the tractor should be stopped, the drive to the attachment disconnected and the moving parts of the implement stopped before the obstruction is cleared. - Seek expert technical advice before fixing counter weights or wheel weights (front or rear) to increase tractor stability. |  |  |
| Public road travel | Vehicle Collision | 1 | - If the tractor is to be operated on public roads it should be fitted with the following to comply with the requirements of the traffic regulations: - A horn and brake. - Head, tail, and turn signal lights. - Reflectors and rear-view mirrors. - Where a tractor is operating in a confined area and other persons cannot be excluded, it should be fitted with reversing beepers. | Supervisor | 2 |
| Stopping a Tractor | Run over by moving tractor | 1 | When stopping a tractor, the following precautions should be taken : - Park on even ground. - Shift the gear selector to neutral or park position. - Disconnect power sources and secure implements. - Lower blades, buckets or any other attachments to the ground and/or securely block these attachments. - Lock the parking brake. - Stop the engine and remove the keys. | Operator | 2 |
| Bunker Silos | Roll over Crushing Hazards | 1 | 1. When tractors or other vehicles are used to pack silage into bunker silos, overturns are a definite possibility. | Supervisor Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - The vehicle chosen for this task should have a rollover protective structure and a seat belt. - Equipping a tractor with dual wheels may increase its lateral stability and help prevent a sideways overturn. - Crushing hazards are also present when bunker silos are unloaded. Workers unloading a bunker silo with a front-end loader have been crushed when silage from the top of the pile fell onto their tractor. - When removing silage from the pile, always start at the top of the face and work down. - Never let an overhang develop. - By removing silage from the top of the face first and sloping the face slightly toward the pile, the chance of being crushed is reduced. |  |  |
| Passengers on Tractors | Run over by Tractor | 1 | - Passengers SHOULD NOT be allowed to ride on tractors. - Passengers can not be effectively protected by ROPS and safe, adequate passenger seating is generally not incorporated into the design of tractors. - Only for the purpose of instruction and training or in situations of extreme emergency to get to or from the scene of a serious accident, should a passenger travel on a tractor. | Operator | 2 |
| Maintenance | Personal injury | 1 | - Tractors should be maintained in a safe operating condition by making regular inspections and following the manufacturer’s recommended servicing and maintenance procedures. - Logbooks should record scheduled maintenance and repairs performed and any modifications which might affect the safe operation of the tractor. - Before inspecting or working underneath a tractor, ensure that the operator has got off the tractor, the tractor cannot move and any movable attachments are lowered to the ground and/or safely blocked. - Do not leave the engine running while making adjustments or repairs unless specifically recommended and a maintenance person is assisting. - Do not oil, grease, or adjust any part of the tractor while it is in motion or check for faulty wiring or loose connections. - Stop all power sources to pulleys before removing or replacing belts. - Allow the engine to cool before removing the radiator cap, and be careful of escaping steam. - When jump-starting the tractor, connect the jumper leads as specified by | Supervisor Operator Maintenance person | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | the manufacturer, to avoid damage to the electrical system and the possibility of a battery explosion. 9. Do not smoke while filling the fuel tank or servicing the fuel system. |  |  |
| Refueling | Fire/explosion | 1 | - Never refuel when the engine is running. - Keep open flames, open lights, lighted cigarettes etc. away from the fuelling operation. - During refuelling, maintain some form of contact between the metal outlet of the refuelling hose and the fuel tank opening to reduce the risk of an explosion or fire due to a discharge of static electricity. - Always refuel in a well ventilated area. | Operator | 2 |
| Removing and Refitting Tyres | Crush hazard Tyres could exploded and injure persons nearby | 1 | - When removing and refitting tractor tyres, first remove the valve core to allow air to escape and make the tyres flexible. - Maintain a good grip on the tyre. - Lever and stand to the side of the tyre when removing the tube from the rim. - While inflating a tyre, continually check to ensure the locking ring is properly seated and locked. - The tyre should be inflated to its correct pressure, according to the tyre manufacturer’s load/inflation specifications. - Always stand to the side when inflating a tyre. - An inflation cage should be used when inflating large tyres where possible. - The ballasting of tractor tyres should be done in accordance with manufacturer’s recommendations. | Operator Maintenance | 2 |
| Tractors, Machinery and Children | Death | 1 | - Always remove the starting keys from tractors when parked. - Isolate or guard machinery so that it is impossible for children to play with or turn on machinery. - Separate younger children’s play areas from active farm working areas, with barriers if necessary. - Never carry children on tractors. - Never leave a tractor unattended with the engine running. - When children are in active farm work areas, their activities should be supervised. Ensure they wear sturdy non-slip shoes, their clothes are tight fitting and that long hair is tied back. - Apply similar standards to visitors and their children. | Supervisor Operator | 2 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.