| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | PRESSURE WASHER - SAFE OPERATION | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025. SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Training and skill. | Un-trained persons could cause injury to themselves and others. Possible property damage. | 1 | - All persons on site must carry with them a current ‘Construction Induction Card’. - All persons working on site must be suitably trained and competent in the work they are to perform. | Supervisor Operator | 2 |
| Personal Protective Clothing & Equipment (PPE) | Personal injury from high water pressure. Particles flicking up from ground surface. Getting wet. | 1 | 1. Minimum PPE when operating Pressure Washer - Hi visibility vests/clothing - Safety boots/gumboots - UV skin protection & head protection - Hearing protection - Eye protection - Gloves - Waterproof overalls | Operator | 2 |
| Setting up work area | Trips & falls | 2 | - Ensure work area is clear of all tripping hazards. - Mark all protruding objects to prevent trips & falls. - Be aware of slippery surface when using water. | Operator | 3 |
|  | Injury to public | 2 | 1. Prevent pedestrian access using signs, barriers or closed gates. | Operator | 3 |
| Unloading high pressure washer from back of vehicle. | Back strain | 2 | - Seek assistance to mover washer. - Use ramps provided. | Operator | 3 |
| Pre-start checks | Damage, injury | 2 | - Check water pump and tank for security -check for cracks on welded section and loose bolts and nuts. - Check water pump and motor. oil and fuel for leaks - ensure to check for crack and loose fitting dripping oil or fuel - Check water tank and filter- be sure to fill water tank before commencement of work. - Ensure sufficient fuel and oil. - Only operators to be in vicinity of water jetting operation. | Operator | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Operation of Water jetting equipment | Personal Injury | 2 | 1. Release the pressure from hand gun - depress the trigger for short period of time allowing the trapped air in the system to escape. This will assist in the prevention of a kick at the start of operation. | Worker | 3 |
| Start the equipment | Damage, injury | 2 | - Ensure all moving parts are free of loose clothing, hoses and body parts. - Be aware there may be a kick at the start. | Worker | 3 |
| Commence Operation | Damage, injury Repetitive movement | 2 | - Position yourself in a non-slippery area and use your legs for support. - Do 5 minutes of water jetting and then clean up area. - No horseplay – never point a water jet at another person. | Worker | 3 |
|  | Entanglement in hose | 2 | 1. Maintain tidy work area. | Worker | 3 |
| After completion shutdown the motor | Damage, injury | 2 | - Ensure ignition key for water pump is switched off. - Release the remaining water pressure in the system. | Worker | 3 |

## NEVER:

- Never point the nozzle of the gun close to any parts of your body or to somebody else

- Never point the nozzle of the gun closer to any breakable or fragile parts of the equipment

- Never use when there are leaks in the hose or fittings

- Never spray when you are not in a good position to hold the gun properly

- Never spray when the water tank is run dry

## ALWAYS:

- Always keep your balance - support your body with your legs

- Always grip the handle of the gun firmly before pulling the trigger

- Always counteract the high pressure spray force by securely positioning your hand and feet

- Always stop spraying when you feel fatigue and unsure of what you are doing

- Always keep a distance from you to your work

- Always wear a full face shield or eye protection

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste products will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

  - Brief each team member on this SWMS before commencing work.

  - Ensure control measures have been implemented prior to the start of works.

  - Observe work being carried out.

  - If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

  - Ensure team knows that work is to immediately stop if the SWMS is not being followed.

  - Retain this SWMS on site for the duration of the high-risk construction work.

  - All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.