| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | LAYING PIPES | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025. SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Training | To prevent injuries or illness to workers | 1 | - Machinery operators will have the necessary current licence for the equipment they will operate. - All persons will have a current construction induction card. - Follow guidelines in other SWMS for the safe use of equipment eg trucks tipping, grader, rollers etc. | Supervisor All | 2 |
| Arriving on site | Entry unsafe or unauthorised areas | 1 | - Report to site office for site induction if required. - Risk assessment of site conditions and Site specific induction & toolbox meeting to be conducted. - Consider all environmental protection issues and the impact caused by entry to site. | All Supervisor | 2 |
| Set out work area and check levels. | Struck by road or construction plant. | 1 | - Barricade area off to restrict access & minimise plant working area. - All workers to wear safety vests. - Operational flashing lights and reversing beepers on all plant. | Supervisor All Operator | 2 |
|  | Falling or tripping over | 2 | - Maintain good housekeeping. - Barricade off unsafe areas. - Clearly mark protruding objects using paint, caps or hazard tape. | All Supervisor Supervisor | 3 |
|  | Dust entering eyes | 2 | - Suppress dust using water truck. - Safety glasses must be worn. | Supervisor All | 3 |
| Excavate Trench | Striking underground services | 1 | - Check plans for services and ask for clarification from Site Manager/Superintendent. - Locate services within work limits using ‘dial before you dig’ if required 1100. | Supervisor Supervisor | 2 |
|  | Potential for physical injury. | 1 | 1. Read SWMS- General Excavation & Trenching in conjunction with this SWMS. | Supervisor Worker | 2 |
|  | Trench collapse | 1 | 1. Trenches exceeding 1.5m in depth must be shored or battered. Refer ‘Safety Alert’ attached. | Supervisor | 2 |
|  | Striking personnel and plant | 1 | - Use spotter when plant & or personnel are working near each other. Hold toolbox meeting. - Reflective safety vests to be worn at all times. - Where possible minimise plant & personnel in work areas. - Operational flashing light and reversing beeper on all plant. | Supervisor All Supervisor Operators | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
|  | Plant failure | 1 | 1. Ensure safety pin is inserted at all times when attachment is on quick hitch for excavators. | Operators | 2 |
|  | Exposure to noise | 2 | - Suppress noise using mufflers. - Hearing protection must be worn by personnel working near noisy equipment. | Workshop All | 3 |
|  | Plant overturning or tipping | 1 | - Only competent operators to use machinery. - Operate plant on stable ground - Operator to complete visual assessment of ground conditions. - All unbuttered excavations to be barricaded off. | Supervisor Operator Operator Supervisor | 2 |
|  | Unstable ground | 2 | - Complete visual assessment of ground conditions prior to commencing works. - Batter back, bench or shore unstable ground. | Supervisor & Operators | 3 |
| Delivery of bedding & crushed rock | Vehicle movements | 1 | - Workers wear hi-vis clothing or safety vests. - Correct signing and barricading in place. | Supervisor | 2 |
| Placement of bedding | Falling into trench | 2 | - Provide ladder access in and out of excavations. - Barricade trench when not in use. | Supervisor Supervisor | 3 |
|  | Fumes | 2 | 1. Ensure dimension of trench is adequate for ventilation of diesel equipment. | Supervisor | 3 |
|  | Personnel struck by back fill | 1 | - Use spotter during placement - Wear hard hats and safety glasses. | Supervisor All | 2 |
|  | Back or muscle injury | 2 | 1. Correct manual handling techniques if using shovels. | Workers | 3 |
|  | Uneven ground | 3 | 1. Plant operator to be aware of uneven surfaces and if possible, ensure level ground prior to plant moving bedding & crushed rock. | Operator | 3 |
| Unloading pipes | Road Traffic | 1 | - Workers wear hi-vis clothing or safety vests. - Correct signing and barricading in place. | Supervisor | 2 |
|  | Pipes slipping off truck | 2 | 1. Pipes are securely tied onto tray of truck. | Truck Driver | 3 |
|  | Moving pipes off truck | 2 | - Use correct manual handling techniques. - Use a mechanical lifting device if it is available. | Workers | 3 |
| Laying Pipes | Falling into trench | 2 | - Provide ladder access in and out of excavations. - Barricade trench when not in use. | Supervisor Operator | 3 |
| Manual lift | Lifting into trench – muscle injury | 2 | - Use correct manual handling techniques. - Use a mechanical lifting device if it is available. | Workers | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Machine lift | Crushing of plant or personnel by pipes. | 1 1 | - Ensure competent dogman secures load. - Use certified lifting chains and shackles. - Check lifting capacity ad SWL prior to lift. - Remove excavation bucket prior to lifting pipes. - Do not slew load over personnel. - Competent dogman required whilst lifting. - Refer SWMS - Using Powered Earthmoving Equipment as a Crane if applicable. | Supervisor Operator Operator Supervisor | 3 3 |
|  | Trench collapse | 1 | 1. Bench or shore all trenches that are over 1.5m in depth &/ or are deemed unstable. | Supervisor | 3 |
| Placement and compaction of backfill. | Exposure to noise | 2 | - Where possible suppress noise using mufflers. - Ensure ear protection is work by personnel working near noisy equipment. | Supervisor | 3 |
|  | Fumes | 2 | 1. Ensure dimension of trench is adequate for ventilation of diesel equipment. | Supervisor | 3 |
|  | Personnel struck by backfill | 1 | - Use spotter during placement. - Wear hard hats and safety glasses. | Supervisor All | 3 |
| Environmental Issues | Transmission of material off site to public domain eg mud on to main road. | 2 | 1. Where possible endeavor to not park in muddy areas. | Operator | 3 |
|  | Hydraulic oil spill | 3 | - Make sure there is a visual check of all hydraulic hoses is part of your pre start check. - Should there be an extensive oil leak- clean it up immediately. | Operator | 3 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Bitumen/Prime | During heavy rain, recently placed prime or bitumen could emulsify and wash into surrounding waterways. | 1 | - Plan prime or bitumen operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for buildup of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled, and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refueling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste asphalt will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.