| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | Combination Vacuum Truck | Project: |  |
| SWMS prepared Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025. SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

RISK MATRIX

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Set out work area. | Struck by road or construction plant. | 1 | - Barricade area off to restrict access and minimize plant working area as appropriate. - All workers to wear safety vests or high visible clothing with long sleeve shirt with sleeves down. - Operational flashing lights and reversing beepers on all plant. | Supervisor All Operator | 2 |
| General Safety Requirements | Personal injury to operator and other workers. | 1 | - All persons who are to operate or assist with the operation of the Combination Vacuum Truck are to be fully competent with Type B Jetting Operators certificate and have received information and instruction in the safe use of the machine and the potential hazards. - Read all instructions before using machine. - Know how to stop and start the vacuum and bleed pressure quickly. - Be familiar with controls. - Do not work in rain or during thunderstorms. - The vacuum is not to be operated by children, teenagers or persons under the influence of drugs or alcohol. - Operator and assistant to wear appropriate PPE – safety boots, safety glasses, long clothing, safety vests (done up to prevent entanglement) gloves, hearing protection and safety helmets where required. - If the vacuum is to be used with hazardous or dangerous materials- the operator must be adequately trained. - If there is a need to climb on top of truck ref to Working at Hight SWMS. | Operator | 2 |
| DBYD Verification | Electrocution Explosion | 1 | - Review all Dial Before You Digs including any conditions of work issued by the relevant asset owner. - Verify all underground assets has been located. - Locate or have your supervisor arrange the location any unidentified services. - Ensure Safe Operating Procedures (SOP) of the asset owner will be complied with regarding excavations. - Obtain any relevant excavation permits if the depth of the excavation is to exceed 1.5m. - If the above cannot be facilitated, please contact your supervisor immediately. - In the instance of contracting to a primary PCBU ensure they have DBYD on site (do not exceed 28 days from issue date) and all relevant locations have been located. - Once satisfied underground asset locations are known, proceed with works. | Supervisor All | 2 |
|  | Falling or tripping over on site. | 2 | - Maintain good housekeeping especially hoses. - Barricade off unsafe areas. - Clearly mark protruding objects using paint, caps or hazard tape. - Ensure open pits or manholes a guarded to prevent falling in. | Supervisor Operator | 3 |
|  | Manual handling of vac hoses | 2 | - Use boom where possible to move hose, shorten length of hose required, if possible, use correct manual handling techniques. - Must be a two-person operation to manage manual handling/fatigue risks. | Operators | 3 |
| Opening manholes/pits | Manual handling Open manholes/pits | 1 | - Use correct tools to pen manholes/pits. - Place guard over open manhole/pit. | Operators | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Confined space |  | - If working near open manhole/pit maintain three points of contact with hose and ground to prevent possibility of falling. - Follow Confined Space Entry SWMS when entry to confined space is required. |  |  |
| Pre start checks vacuum recycler truck | Unsafe machine potential for injury to operator or bystanders | 1 | 1. Prestart checks are to be completed as per the operations manual and documented as such. | Operator | 2 |
| Assemble the vacuum equipment | Manual handling Slips, trips, falls Failure of connections Kicking/moving hoses Vertical connections of hose People being caught on end of suction Static Electricity | 1 | - Use boom where practical. - Position hose trailer as close to worksite as possible. - Use team lifting. - Use vacuum nozzles and fittings to reduce bending. - Each travis lamp to be checked and tapped if required. - Hose to be restrained along length to stop whipping. - Vertical connections to be tied between joints to prevent hose falling if clamp fails. - Personnel not to step over unrestrained hose lengths. - Barricade paths and work area. - Use crane or similar to raise/lower hose lengths where possible - Vertical connections to be secured with rope across join. - Area underneath and near vertical connections to be barricaded out to a distance to ensure no-one is underneath the potential fall path of hose if it fails. - Vacuum breaker must be fitted at final length of hose on worksite. - If remote is fitted then remote is also to be secured with Operator onsite. - Walk vacuum line to ensure safety & security along length of line to worksite. - Earth out the unit using the static cable and stake. | Operators | 2 |
| Assemble Jetting Equipment | Infection Sharp edges Manual handling Falling equipment High pressure water | 1 | - Personal hygiene (wash hands prior to eating). - Wear gloves and remove sharp edges/burrs on equipment. - Use correct manual handling techniques. - Store equipment securely on vehicle. - Inspect hose daily and ensure it has been tested (6 monthly) | Operators | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Never cap jetting hose. - Inspect jetting hose for damage. - Never adjust pump pressures or regulators. - Maximum reducer on 1 inch hose is ¾ inch. - No reducers on ½ inch hose. - Must be 3 metre leader hose on front of jetting assembly to identify end of jet assembly when removing from drain. - Fittings to be firmly secured use |  |  |
| Operating the Vacuum recycler truck | Injury to operator or bystanders – high pressure- dirt and stones can be flicked around. | 1 | - Wear eye and ear protection. - Keep operating area clear of all persons. - Use a witch’s hat next to the water jet to prevent splashes. - Stay alert and beware of the force of the water pressure coming out the end of the hose and when using the vacuum hose be aware of the suction pressure. - Do not overreach or stand on unstable supports. - Never aim the jet in the direction of persons or animals as the water jet comes out of the nozzle at high speed with high pressure. - Do not leave the operating vacuum unattended. If the vacuum is not required switch off. - Do not operate the vacuum in enclosed spaces. - Follow directions as per the operations manual. - The pressure hose is operated with by trigger action. - When cleaning pipes etc. the hose must first be in the pipe prior to activating the trigger. | Operator | 2 |
|  | Fire hazard | 1 | - Do not use near flammable liquids. - Turn off machine prior to refueling. | Operators | 2 |
| Flushing and or vacuuming pits and pipes | Striking pedestrians, members of the public or other workers Spraying debris causing injury Spraying debris causing property damage | 1 | - Wear appropriate PPE for the site including gloves. - Ensure appropriate work area is cordoned off to prevent access by the public. - When deciding on appropriate work area take into account spray return and wind conditions. - Observe the hose as it is being fed into pipe or pit. - Watch reel to ensure hose winds on correctly. | Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Whipping hoses |  | - Ensure minimal slack in hose to prevent whipping. - Before hose leaves pit ensure the water is shut off. - Stand clear of hose when it is exiting pit or pipe as debris may be sprayed out. - Only uncover pits required for access as pit covers will prevent uncontrolled water or debris spray. - Never point pressure spray at any person. |  |  |
| Use of drain cleaning cutter | Hand injury Bodily injury from cutting blade | 1 | - Where possible investigate that drain obstructions are tree roots and are not other pipes, cables etc. using suitable equipment. - First run cleaner jetting head at obstruction. - If obstruction doesn’t clear and roots are evident in water then cutting head can be prepared. - Wear gloves when handling cutting blade. - Do not operate cutter until safely positioned inside the drain. - Maintain a grip on jetting hose to feel the reaction force of cutter. | Operators | 2 |
| Non-Destructive Digging – Coring hard surfaces, post-hole or trench, prove asset | Flying debris Extreme Noise High pressure water Damage to lance Damage to Assets | 2 | - Wear appropriate PPE. - Use double hearing protection (ear plugs & earmuffs) - Use Noise Dampening Box. - Do not allow the water jet to contact any body part. - DO NOT strike or force soil using the lance. - Use other tools to lift loosened or exposed soil/debris from hole. - Once an asset or marker tape is discovered slow digging process and steadily reveal the asset. | Operators | 3 |
| Hose blockage | Entrapped hand Manual handling | 2 | - Do not use hand to unblock hoses. - Keep hands clear of hose. - Use two operators when handling hose when clearing blockages. | Operators | 3 |
| Finishing | Waste not cleaned up Manual Handling Slips, trips, falls | 2 | - Ensure all waste is vacuumed up. - Use correct manual handling techniques. - Ensure manhole or pits are covered. | Operators | 3 |
| Shut down and Maintenance | Potential for personal injury | 1 | - When finishing work follow end of day maintenance and shut down requirements. - When disassembling any part of the machine or servicing make sure the machine is turned off. | Operator | 2 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers are available in WHS 05 emergency procedures. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tires/bodies for buildup of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled, and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refueling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat at all Times | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory Helmets if and when required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle License’s (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators license’s or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice and Other Guides |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |
| Guide for Managing Risks from High Pressure Water Jetting – Safe Work Aust 2012 |  |  |  |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Vacuum Recycler Truck

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.