| SAFE WORK METHOD STATEMENT |  |  |  |
| --- | --- | --- | --- |
| Persons responsible for supervising, inspecting work, work methods, plant & equipment and tools | Supervisor/Foreman | Date: |  |
| High Risk Construction Work: | CONCRETE PLACEMENT | Project: |  |
| SWMS prepared by Patrick Monaghan - Paneltec Group -Operations Manager and in consultation with management and workers – 2025 SWMS Review Date: August 2026 unless required earlier Signature: |  | Location: |  |
| Approval to use this SWMS: “I certify that the attached SWMS has been reviewed by me, is endorsed for compliance with WorkSafe guidelines and is approved for use” John Guy Director - Paneltec Pty Ltd 0408 449 023 31/08/2025 – V12.0 Name Position  Signature Date |  |  |  |

RISK MATRIX

| Level | Description of Consequence or Impact | Consequence | Likelihood / Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L Likely | M Moderate | U Unlikely |
| H (1) (High level of harm) | Potential death, permanent disability or major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | H (1) (High) | 1 | 1 | 2 |
| M (2) (Medium level of harm) | Potential temporary disability or minor structural failure/damage. On-site environmental discharge/release contained, minor remediation required, short-term environmental harm. | M (2) (Medium) | 1 | 2 | 3 |
| L (3) (Low level of harm) | Incident that has the potential to cause persons to require first aid. On-site environmental discharge/release immediately contained, minor level clean up with no short-term environmental harm. | L (3) (Low) | 2 | 3 | 3 |
| Level | Likelihood / Probability |  |  |  |  |
| Likely | Could happen frequently |  |  |  |  |
| Moderate | Could happen occasionally |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances |  |  |  |  |

HIERARCHY OF RISK CONTROL

The hierarchy of controls must be applied in the order below for all identified hazards.

Level 1 Eliminate the hazard

Level 2 Substitute the hazard with something safer.

Level 3 Isolate the hazard from people. Reduce the risks through engineering controls.

Level 4 Reduce exposure to the hazard using administrative actions.

Level 5 Use personal protective equipment

The control measures that you put in place should be reviewed regularly to make sure they work as planned

# ACTIVITY ANALYSIS

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Training | To prevent injuries or illness to workers | 1 | - Machinery operators will have the necessary current licence for the equipment they will operate. - All persons will have a current construction induction card. | Supervisor All | 2 |
| Arriving on site | Entry unsafe or unauthorised areas | 1 | - Report to site office for site induction if required. - Risk assessment of site conditions and Site specific induction & toolbox meeting to be conducted. - Consider all environmental protection issues and the impact caused by entry to site. | All Supervisor | 2 |
| General planning | Task specific injuries due to inexperience, inadequate consultation or failure to provide appropriate equipment. | 2 | - Workers to be have sufficient skills and are competent to complete concreting tasks. - Adequate consultation required between supervisor and workers on task to be undertaken. - Workers completing works to be adequately supervised. - Appropriate concreting equipment to be supplied for use by workers. - Workers not to improvise with inappropriate equipment. | Supervisor | 3 |
|  | Slips, trips, falls, abrasions, strains and sprains; manual handling injuries such as back damage. | 2 | - There should be a clear space of a min. 550mm wide around the work area. - Access ways to be suitably defined (there needs to be a clear access for concreters and their equipment). - Access to work area not to be cluttered – good housekeeping (keep stored materials and rubbish out of work area). | Supervisor Workers | 3 |
| Site Access | Potential physical injury to workers, truck drivers or the general public. Electrocution | 1 | - Ensure all planning and site preparation takes into account how concrete will be placed allowing room for trucks, ramps for wheelbarrows, space for a pump etc. - Clear access must be provided to transport the concrete. - If concrete is to be delivered by trucks make sure they have unrestricted access to the site in all weather conditions. - A spotter (traffic controller) to be provided where required to assist in conditions involving reversing, low clearance (eg powerlines), obstacles, tight access, high vehicle, concrete pump and/or pedestrian traffic. | Supervisor | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - Traffic control to be provided where necessary on public roads by competent personnel. - There needs to be a safe route to the unloading point (including adequate signage). - Stable ground surface (freshly filled ground can present a problem for heavy trucks). - Delivery point to be clear of any building rubbish or materials. - There needs to be a safe exit from site ensuring a safe area is selected to pull off the road and that no road rules are breached if road side delivery is required. |  |  |
| Laying Concrete | Skin cancer; sunburn, eye damage. Skin rash or allergy/dermatitis. Concrete splashed in eye. Short or long term health affect, e.g rash, allergy, Dehydration & dizziness. | 2 | - Workers on the site should always wear protective clothing, strong boots and, if required, helmets or eye protection. (AS rated sunglasses to be worn). - Always avoid direct contact with cement and never kneel in or touch the concrete mix directly. - Wear gloves or use barrier creams. - Refer to Safety Alerts on “Working Safely with Wet Concrete and Working Safely with Dry Concrete Materials” - Ensure that anyone using heavy equipment, such as screeds or vibrators, has been properly trained. - Water to be made available to flush concrete from eyes. - Eye protection to be worn for tasks where splashing occurs, e.g. vibrating. - Barrier cream to be made available for use and used as required by workers. - SDS to be available on site. - All Safety instructions to be followed. - Adequate supply of cold drinking water provided to the work area. | Workers | 3 |
| Pumping Concrete | Violent ejection of concrete from hose at first moment of delivery. Struck by concrete which may cause body injuries; | 1 | - Warning to be given prior to starting or re-commencing the pour. - Make sure there is no air in hose due to: a blockage; a change of supply; a poor mix – too bony; or delays in the pour. - Make sure the delivery hose is not damaged. | Delivery Operator | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | Delivery pipe/joint |  | - Competent person in control of Tbar at end of hose. - Make sure all connecting pipes are adequately secured, e.g. no safety pins for lever couplings. - Make sure joint or pipe section brackets are not fixed too far apart causing excessive movement. - Ensure pipe operating pressure is not exceeded and the correct diameter pipe is used. |  |  |
|  | failure. -Struck by concrete |  |  |  |  |
|  | or piece of pipe, which |  |  |  |  |
|  | may cause body injuries; |  |  |  |  |
| Person holding “Tbar” placing concrete & person behind holding “J” hook | Manual handling injuries Strains and sprains; injuries such as back damage. | 2 | - “Tbar” & “J” hook operators to be well coordinated to ensure hose can be manoeuvred with ease. - “Tbar operator to be experienced- work with recoil and not against it. - Supervisor to ensure enough workers allocated to assist with tasks. - Correct manual handling techniques to be employed when lifting and moving hose around. - Rotate jobs. | Operators Supervisor | 3 |
|  |  |  |  | Operators |  |
|  |  |  |  | Supervisor |  |
| Access | Walking on deck | 2 | - Persons should walk forward and not backwards during delivery. - Persons to watch where they are stepping so as not to get boots caught between steel reinforcement – different sized reinforcement occurs over the work area. - Ensure there are enough ties or chairs to prevent steel reinforcement saging or giving way. - Make sure Tbar doesn’t slip off the end of the hose. - Make sure there are caps on the ends of reinforcement bar/s, e.g. column starter bars. - Make sure Tie wire is turned down into reinforcement cage. - Be aware that Tie wire can puncture boots. - Planks need to be provided to cover deep beams. - Penetrations need to be covered and covers secured. | Persons on site | 3 |
| . | during pour-slip on wet ply |  |  |  |  |
|  | or reinforcement and fall; |  |  |  |  |
|  | trip or twist ankle or knee. |  |  |  |  |
|  | Sharp edges penetrations |  |  |  |  |
|  | or steps in the deck, |  |  |  |  |
|  | e.g. deep beams |  |  |  |  |
|  | Cuts from reinforcement |  |  |  |  |
|  | bar ends or tie wire. |  |  |  |  |
|  | Fall through penetration |  |  |  |  |
|  | or into deep beam. |  |  |  |  |
| Vibrator and | Hearing damage from | 2 | - Equipment that has a high noise level should be quietened where possible. - Hearing protection should be worn where equipment has a noise level above 85dBA. | Supervisor | 3 |
| Operator | continuous noise over a |  |  | Workers |  |
|  | long period |  |  |  |  |
|  | Splashes in eye. Explosion |  |  |  |  |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  | and/or fire; burns; rashes or allergies from contact with bare skin. Illness; breathing difficulties – overcome by fumes. Electric shock or electrocution. Slip on wet ply or Reinforcement and fall; trip or twist ankle or knee. Strains and sprains; injuries such as back damage. |  | - Eye protection to be worn. - Sufficient water to be made available in work area for first aid treatment eg to flush concrete from eye/s. - Fuel should be stored in appropriate fuel containers. - Funnel to be made available for refueling. - No Smoking near fuel. - Equipment to be switched off and cool prior to refuelling. - Correct PPE to be worn when refuelling. - When electric vibrator used an Earth Leakage switch to be installed on mains supply or portable generator. - Equipment to be checked prior to use to ensure it is not faulty or damaged. - Extension leads to be checked prior to use to ensure they are not damaged. - Extension lead to be secured above work area – so it is not lying in water. - All electrical gear used on site should have a current test tag. - Operator not to walk backwards while operating vibrator. |  |  |
| Shoveling | Manual handling injuries. Strains and sprains; injuries such as back damage. Slip on wet ply or Reinforcement and fall; trip or twist ankle or knee. | 2 | - Workers should be given sufficient rest periods between jobs. - Rotate jobs -repetitious twisting and shoveling, or raking can cause injuries. - Make sure concrete not too dry – this will make it difficult to work with and may cause injuries. - Workers not to walk backwards while shoveling. - Workers to be aware of there surroundings when getting in and out of concrete to make sure they don’t slip or fall or get their boots caught between steel reinforcement. | Supervisor Workers | 3 |
| Screeding | Manual handling injuries Strains and sprains; injuries such as back damage. Hearing damage from continuous noise over a long period. | 2 | - Mechanised screed to be used where practicable. - Sufficient rest periods between jobs. - Job rotation to prevent repetitious twisting and shoveling, or raking. - Clear access around the perimeter of the work area (min. 550 mm wide). | Supervisor Workers | 3 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
| Bull float | Strains and sprains; injuries such as back damage. Trip on float and fall onto the deck, onto rubbish or building materials. | 2 | - Sufficient rest periods between jobs to prevent repetitious arm and wrist movement. - Rotate jobs where possible. - Need to have clear access around the perimeter of the work area where access is not cluttered with stored materials and/or rubbish which leave no room to pull the float straight back. - Need to watch out for the float when walking behind the operator. - Operator needs to swap hand frequently. | Supervisor Workers | 3 |
| Hand trowelling | Slips, trips | 2 | - Need to have clear access around the perimeter of the work area (min. 550 mm wide). - Area around work area not to be cluttered with stored materials and/or rubbish. | Supervisor Workers | 3 |
| Helicopter float | Loss of control resulting in a strain or sprain or serious cut. Blade strikes body part causing a serious cut. Strains and sprains; injuries such as back damage. Explosion and/or fire; burns; Illness; breathing difficulties – overcome by fumes. Hearing damage from continuous noise over a long period. | 1 | - Operator to be trained and competent in the use of the helicopter float. - Operator is to watch what they are doing at all times. - Make sure there are no objects are protruding from the slab, e.g. reinforcement or stressing duct, that can stop the blade or catch on the guards. - Operator to be aware that the momentum may throw the operator. - Machine to be checked prior to operation to ensure it is in good condition. - Machine maintenance needs to be up to date - Machine guards need to be in place prior to the start of the job. - Make sure the Pin securing the machine blade is not loose or worn which could result in blade falling off. - Make sure the extension carry bar is in place and not too short. - Use a crane where possible to lift the machine. - Fuel should be stored in appropriate fuel containers. - Funnel to be made available for refueling. | Supervisor Workers | 2 |

| Steps in Activity | Potential Hazards | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class After Controls In Place |
| --- | --- | --- | --- | --- | --- |
|  |  |  | - No Smoking near fuel. - Equipment to be switched off and cool prior to refuelling. - Correct PPE to be worn when refuelling. - Make sure there is a scrubber, or catalytic converter fitted to exhaust. - Year hearing protection where equipment signed. |  |  |
| Sealing concrete curing | Sprayed sealing compound may cause skin rashes and/or allergies | 2 | - Correct PPE- protective clothing, gloves, boots, eye goggles and face masks to be supplied and worn when using sealing compound. - Workers to be careful not to breath in fumes from chemicals. - If chemicals come in contact with skin or eyes wash area with plenty of cool water and seek doctors advice. - Always read the safety directions on the label (or SDS) of any chemical container prior to use. - Take note of wind direction when spraying compound and warn other workers of potential drift. | Supervisor Worker | 3 |

| ENVIRONMENTAL RISK ASSESSMENT & CONTROL MEASURES THAT MAY BE REQUIRED ON SITE DEPENDING ON THE ACTIVITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| Emergencies | Accident/ Incident Fire Spill | 1 | - Personnel have been advised at toolbox meeting and are prepared in the event of an incident occurring. - Emergency numbers available on site. - Fire Extinguishers/First Aid Kits/Spill Kits on site. | Supervisor All | 2 |
| Air Quality | Exhaust Fumes Dust | 2 | - Control exhaust emissions eg. Regular maintenance, replace old machinery, fit appropriate emission control measures. - Dampen milled surface with water cart. - Use water cart where required. | Supervisor All | 3 |
| Wet Concrete | During heavy rain, recently placed concrete could emulsify and wash into surrounding waterways. | 1 | - Plan concrete operation so that it is conducted during fine weather. - Have material available to clean up a spill if it occurs and dispose of contaminated material accordingly. - Protect storm drains during and after application to prevent pollution - have spill clean-up materials available. | Supervisor All | 2 |
| Community Relations | Inconvenience as a result of works/traffic management | 2 | - Identify from the list of potentially impacted parties who may be impacted by the work being undertaken. - Letter drops could be considered (unless specifically required) to notify residents/businesses etc. about works to be undertaken and if it has an effect on them. | Supervisor All | 3 |
| Flora & Fauna | Some works may require the removal of vegetation. Native fauna may be at risk from excavation works | 2 | - Assess work area for significant vegetation: size of trees, faunal habitat - define work and exclusion areas using fencing and signage. - Machinery is to be thoroughly washed down prior to leaving site where noxious weeds are present. | Supervisor All | 3 |
| Heritage & Archaeology | Unmonitored excavation at significant sites can lead to loss or destruction of valuable artifacts/relics, and disturbance of historical sites. including cultural heritage and/or archaeological significance | 1 | - Undertake a survey of the site to identify any areas of significance. The client’s representative may undertake this. Government bodies may be able to assist. - Develop a project specific procedure based on the findings and recommendations. - Installation of exclusion fencing and signage if required. | Supervisor All | 2 |

| Work Activity Risk | Hazard | Risk Class | Risk Control Measure | Responsible Person/s | Risk Class after Controls in place |
| --- | --- | --- | --- | --- | --- |
| Noise Pollution, Community Relations & Vibration | Sensitivity to noise - to public & fauna | 1 | - Work undertaken during “normal” hours where possible. - Affected residents notified in person or in writing. - Plant and equipment regularly serviced & fitted with efficient mufflers. - Magnitude of vibration reduced by using smaller plant etc. | Supervisor All | 2 |
| Soil Management & Water Quality | Soil needs to be managed to ensure that; there are no off-site impacts. Damage to stockpile areas (not prepared properly). Mud deposited onto roadways. | 2 | - Wash down machinery in only nominated areas. - Not within 20 metres of water courses. - Use only approved and nominated stockpile areas. - Build bund wall out of material available on site. - Drivers to check vehicle tyres/bodies for build up of mud when leaving stockpile sites and remove if necessary. | Supervisor All | 3 |
| Fuels and Chemicals | Contamination of waterways due to spillages of oil, degreasers, diesel, etc. | 1 | - Store fuel and chemicals in accordance with relevant standards/guidelines. - Minimise storage on site and locate storage facilities away from drains and waterways. - All containers clearly labelled and Safety Data Sheets are available on site. - All drums stored securely and in a bunded area. - Spill kits or alternative spill containment materials available on site. - Regular maintenance of plant hoses. - Use correct refuelling equipment. | Supervisor All | 2 |
| Visual Impact Litter and Amenities Waste Management | Litter such as paper, plastic, and other refuse as well as being a possible safety risk may be carried off-site or washed into waterways resulting in pollution and danger to native fauna. Litter is also visually obtrusive. Production generated lighting, litter, mud, dirt and dust may also impact residents. | 2 | - Use site induction to communicate the requirement for a tidy site. Also stress consideration for local residents. - If as a result of our works, dirt, mud, or litter affects a resident, then we will rectify this problem immediately. - Inspect the site frequently and insist on tidiness. Ensure public roads are free of dirt/mud from the work site. - Broken-out paved surfaces shall be repaired as soon as possible. - Remove all rubbish from site each day. - Waste products will be recycled where possible. | Supervisor All | 3 |

| Personal Protective Equipment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All site personnel must wear the required PPE specified in the controls above, such as: |  |  |  |  |  |  |
| Safety Hi-vis Vests/Shirts | Safety Boots | Sunscreen | Broad Brimmed Sun Hat | Sunglasses | Gloves specific to the task | White reflective overalls/night |
| Hard Hat where required | Safety Glasses | Hearing protection where required | Long clothing | Dust masks | Respiratory equipment if required |  |

| Workers may require Task Specific Training depending on activity each worker will be required to fulfill. |  |  |  |
| --- | --- | --- | --- |
| - All workers shall hold a current Construction Induction Card |  |  |  |
| - If required Client Site Induction - Activity Induction (SWMS & SWP’s) | - Vehicle Licences (Car or Truck depending on class of vehicle or mobile plant driven) | - Plant Operators licences or competency assessments where required | - Competencies for Work Zone Traffic Management |
| - First Aid certificate training | - Manual Handling training | - Fire Extinguisher training | - Fatigue Management Training |

A Site Specific Induction & Toolbox Meeting will be held each day prior to the start of each job.

| Legislation & Codes of Practice (refer to company Legislation register for applicable Australian Standards applicable to works) |  |  |  |
| --- | --- | --- | --- |
| Work Health & Safety Act 2012 and Work Health & Safety Regulations 2022 |  |  |  |
| Codes of Practice |  |  |  |
| Confined Spaces - 2020 | Hazardous Manual Tasks - 2020 | Managing Noise & Preventing Hearing Loss at Work-2020 | Managing Risks of Plant in the Workplace - 2018 |
| Construction Work - 2020 | How to Manage WH&S Risks - 2018 | Managing Risk of Falls at Workplaces - 2020 | Managing Electrical Risks in the Workplace -2019 |
| Excavation Work - 2020 | First Aid in the Workplace - 2018 | Managing Work Environment & Facilities - 2020 | WHS Consultation Co-operation & Co-ordination - 2018 |
| Welding Processes - 2021 | How to Safely Remove Asbestos - 2020 | Managing Risk of Hazardous Chemicals - 2018 | Safe Design of Structures - 2018 |

Plant & Equipment for this activity

- All plant is inspected prior to sending out to site - Daily maintenance & service checks. - Daily pre-start checks are conducted on all plant and trucks by the operators and daily checklist filled out. - Regular servicing at 250hr intervals or as per manufactures specifications. Service History available on request to the office.

List of plant for this Activity

Engineering details/certificates/regulatory approvals/permits/electrical isolation (if applicable):

WORK ACTIVITY BRIEFING

## Implementation, Monitoring and Review of SWMS

The Supervisor on site has the responsibility to:

- Brief each team member on this SWMS before commencing work.

- Ensure control measures have been implemented prior to the start of works.

- Observe work being carried out.

- If controls documented in SWMS are not adequate, stop the work, review the SWMS, adjust as required and re-brief the team.

- Ensure team knows that work is to immediately stop if the SWMS is not being followed.

- Retain this SWMS on site for the duration of the high-risk construction work.

- All persons involved in these particular tasks on site must sign this document as read and understood.

SWMS attendance sheet — I have been given the opportunity to provide input into the development and review of this SWMS

| Name (please print clearly) | Signature | Company | Date |
| --- | --- | --- | --- |

Daily pre-start meetings will be held to ensure all workers are informed of control measures and additional safety hazards & controls to be noted on toolbox form.